Dear User,

this file includes the latest tender specification texts relevant to HEWI products. These can be easily copied on to the clipboard and inserted into other documents.

Via the links in the first column, you can directly access the web page of a given item in our online product catalogue, where you will find further details such as information on colours and finishes, photos, dimensioned views and installation/assembly instructions. You need to be connected to the Internet to be able to use the links.

Opening the browser and displaying the specific page can take several seconds, depending on the transmission rate of your online access and how busy the Internet is. This why we would ask you to be patient and not close the application prematurely.

Tender specification texts on group "USA Products" (71266) (produced on 27th August 2021)

| Sit |  |  |
| --- | --- | --- |
| Sit => Folding shower seats |  |  |
| 801.51BUS32L L-shaped shower compartment seat with lift-up feature (USA) Matt edition 801.51BUS32L in product catalogue |  | Shower folding seat, L-shape (USA) - design made from rails and seat elements - functions as a secure seat in the shower area - can be folded up - with corrosion-protected steel core throughout - wall plate made from synthetic material with integrated steel core - protruding seat element left - the protruding seat element is 13 3/8 inch (340 mm) wide and 15 1/16 inch (382 mm) deep - the L-part is 13 9/16 inch (345 mm) wide and 22 3/4 inch (578 mm) deep - seat elements 2 3/16 inch (55 mm) wide - maximum loading capacity 450 lbs (204 kg) - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - concealed fastening - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.51BUS32L |
| 801.51BUS32R L-shaped shower compartment seat with lift-up feature (USA) Matt edition 801.51BUS32R in product catalogue |  | Shower folding seat, L-shape (USA) - design made from rails and seat elements - functions as a secure seat in the shower area - can be folded up - with corrosion-protected steel core throughout - wall plate made from synthetic material with integrated steel core - protruding seat element right - the narrow seating element is 13 3/8 inch (340 mm) wide and 15 1/16 inch (382 mm) deep - the protruding seat element is 13 9/16 inch (345 mm) wide and 22 3/4 inch (578 mm) deep - seat elements 2 3/16 inch (55 mm) wide - maximum loading capacity 450 lbs (204 kg) - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - concealed fastening - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.51BUS32R |
| 801.51BUS33L L-shaped shower compartment seat with lift-up feature (USA) Matt edition 801.51BUS33L in product catalogue |  | Shower folding seat, L-shape (USA) - design made from rails and seat elements - functions as a secure seat in the shower area - can be folded up - with corrosion-protected steel core throughout - wall plate made from synthetic material with integrated steel core - protruding seat element left - the narrow seating element is 18 1/8 inch (460 mm) wide and 15 1/16 inch (382 mm) deep - the protruding seat element is 13 9/16 inch (345 mm) wide and 22 3/4 inch (578 mm) deep - seat elements 2 3/16 inch (55 mm) wide - maximum loading capacity 450 lbs (204 kg) - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - concealed fastening - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.51BUS33L |
| 801.51BUS33R L-shaped shower compartment seat with lift-up feature (USA) Matt edition 801.51BUS33R in product catalogue |  | Shower folding seat, L-shape (USA) - design made from rails and seat elements - functions as a secure seat in the shower area - can be folded up - with corrosion-protected steel core throughout - wall plate made from synthetic material with integrated steel core - protruding seat element right - the narrow seating element is 18 1/8 inch (460 mm) wide and 15 1/16 inch (382 mm) deep - the protruding seat element is 13 9/16 inch (345 mm) wide and 22 3/4 inch (578 mm) deep - seat elements 2 3/16 inch (55 mm) wide - maximum loading capacity 450 lbs (204 kg) - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - concealed fastening - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.51BUS33R |
| 801.51BUS440 Rectangular shower compartment seat with lift-up feature (USA) Matt edition 801.51BUS440 in product catalogue |  | Shower folding seat, rectangular (USA) - design made from rails and seat elements - functions as a secure seat in the shower area - can be folded up - with corrosion-protected steel core throughout - wall plate made from synthetic material with integrated steel core - 20 1/16 inch (510 mm) wide and 15 1/16 inch (382 mm) deep - seat elements 2 3/16 inch (55 mm) wide - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - concealed fastening - maximum loading capacity 450 lbs (204 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.51BUS440 |
| 801.51BUS450 Rectangular shower compartment seat with lift-up feature (USA) Matt edition 801.51BUS450 in product catalogue |  | Shower folding seat, rectangular (USA) - design made from rails and seat elements - functions as a secure seat in the shower area - can be folded up - with corrosion-protected steel core throughout - wall plate made from synthetic material with integrated steel core - 27 5/16 inch (694 mm) wide and 15 1/16 inch (382 mm) deep - seat elements 2 3/16 inch (55 mm) wide - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - concealed fastening - maximum loading capacity 450 lbs (204 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.51BUS450 |
| 801.51BUS460 Rectangular shower compartment seat with lift-up feature (USA) Matt edition 801.51BUS460 in product catalogue |  | Shower folding seat, rectangular (USA) - design made from rails and seat elements - functions as a secure seat in the shower area - can be folded up - with corrosion-protected steel core throughout - wall plate made from synthetic material with integrated steel core - 32 3/16 inch (817 mm) wide and 15 1/16 inch (382 mm) deep - seat elements 2 3/16 inch (55 mm) wide - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - concealed fastening - maximum loading capacity 450 lbs (204 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.51BUS460 |
| 801.51BUS520 Bathtub seat with lift-up feature (USA) Matt edition 801.51BUS520 in product catalogue |  | Bathtub folding seat (USA) - design made from rails and seat elements - functions as a secure seat in the bathtub - can be folded up - with corrosion-protected steel core throughout - wall plate made from synthetic material with integrated steel core - 15 1/4 inch (387 mm) wide and 29 1/8 inch (740 mm) deep - seat elements 2 3/16 inch (55 mm) wide - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - concealed fastening - maximum loading capacity 450 lbs (204 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.51BUS520 |
| 801.51US320L L-shaped shower compartment seat with lift-up feature (USA) 27" x 22 3/4" (685 x 578 mm), projecting seat element left 801.51US320L in product catalogue |  | Shower folding seat, L-shape (USA) - design made from rails and seat elements - functions as a secure seat in the shower area - can be folded up - with corrosion-protected steel core throughout - wall plate made from synthetic material with integrated steel core - protruding seat element left - the protruding seat element is 13 3/8 inch (340 mm) wide and 15 1/16 inch (382 mm) deep - the L-part is 13 9/16 inch (345 mm) wide and 22 3/4 inch (578 mm) deep - seat elements 2 3/16 inch (55 mm) wide - maximum loading capacity 450 lbs (204 kg) - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - concealed fastening - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.51US320L |
| 801.51US320R L-shaped shower compartment seat with lift-up feature (USA) 27" x 22 3/4" (685 x 578 mm), projecting seat element right 801.51US320R in product catalogue |  | Shower folding seat, L-shape (USA) - design made from rails and seat elements - functions as a secure seat in the shower area - can be folded up - with corrosion-protected steel core throughout - wall plate made from synthetic material with integrated steel core - protruding seat element right - the narrow seating element is 13 3/8 inch (340 mm) wide and 15 1/16 inch (382 mm) deep - the protruding seat element is 13 9/16 inch (345 mm) wide and 22 3/4 inch (578 mm) deep - seat elements 2 3/16 inch (55 mm) wide - maximum loading capacity 450 lbs (204 kg) - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - concealed fastening - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.51US320R |
| 801.51US321S L-shaped shower compartment seat with lift-up feature (USA) 27" x 22 13/16" (685 x 580 mm), with floor support 801.51US321S in product catalogue |  | Shower folding seat, L-shape (USA) - design made from rails and seat elements - with corrosion-protected steel core throughout - functions as a secure seat in the shower area - wall plate made from synthetic material with integrated steel core - with made-to-measure floor support for absorbing vertical application of force - floor support made to measure for seat heights from 17 to 19 inch (432 to 482 mm) - floor support a broad bottomed buffer to protect soft floor coverings - can be folded up - protruding seat element left or right - the protruding seat element is 13 3/8 inch (340 mm) wide and 15 1/16 inch (382 mm) deep - the L-shaped element is 13 9/16 inch (345 mm) wide and 22 3/4 inch (578 mm) deep - seat elements 2 3/16 inch (55 mm) wide - maximum loading capacity 450 lbs (204 kg) - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - concealed fastening - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.51US321S |
| 801.51US330L L-shaped shower compartment seat with lift-up feature (USA) 31 11/16" x 22 3/4" (805 x 578 mm), projecting seat element left 801.51US330L in product catalogue |  | Shower folding seat, L-shape (USA) - design made from rails and seat elements - functions as a secure seat in the shower area - can be folded up - with corrosion-protected steel core throughout - wall plate made from synthetic material with integrated steel core - protruding seat element left - the narrow seating element is 18 1/8 inch (460 mm) wide and 15 1/16 inch (382 mm) deep - the protruding seat element is 13 9/16 inch (345 mm) wide and 22 3/4 inch (578 mm) deep - seat elements 2 3/16 inch (55 mm) wide - maximum loading capacity 450 lbs (204 kg) - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - concealed fastening - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.51US330L |
| 801.51US330R L-shaped shower compartment seat with lift-up feature (USA) 31 11/16" x 22 3/4" (805 x 578 mm), projecting seat element right 801.51US330R in product catalogue |  | Shower folding seat, L-shape (USA) - design made from rails and seat elements - functions as a secure seat in the shower area - can be folded up - with corrosion-protected steel core throughout - wall plate made from synthetic material with integrated steel core - protruding seat element right - the narrow seating element is 18 1/8 inch (460 mm) wide and 15 1/16 inch (382 mm) deep - the protruding seat element is 13 9/16 inch (345 mm) wide and 22 3/4 inch (578 mm) deep - seat elements 2 3/16 inch (55 mm) wide - maximum loading capacity 450 lbs (204 kg) - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - concealed fastening - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.51US330R |
| 801.51US331S L-shaped shower compartment seat with lift-up feature (USA) 27" x 22 13/16" (685 x 580 mm), with floor support 801.51US331S in product catalogue |  | Shower folding seat, L-shape (USA) - design made from rails and seat elements - with corrosion-protected steel core throughout - functions as a secure seat in the shower area - wall plate made from synthetic material with integrated steel core - with made-to-measure floor support for absorbing vertical application of force - floor support made to measure for seat heights from 17 to 19 inch (432 to 482 mm) - floor support a broad bottomed buffer to protect soft floor coverings - can be folded up - protruding seat element left or right - the narrow seat element is 18 1/8 inch (460 mm) wide and 15 1/16 inch (382 mm) deep - the protruding seat element is 13 9/16 inch (345 mm) wide and 22 3/4 inch (578 mm) deep - seat elements 2 3/16 inch (55 mm) wide - maximum loading capacity 450 lbs (204 kg) - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - concealed fastening - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.51US331S |
| 801.51US440 Rectangular shower compartment seat with lift-up feature (USA) 20 1/16" x 15 1/16" (510 x 382 mm) 801.51US440 in product catalogue |  | Shower folding seat, rectangular (USA) - design made from rails and seat elements - functions as a secure seat in the shower area - can be folded up - with corrosion-protected steel core throughout - wall plate made from synthetic material with integrated steel core - 20 1/16 inch (510 mm) wide and 15 1/16 inch (382 mm) deep - seat elements 2 3/16 inch (55 mm) wide - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - concealed fastening - maximum loading capacity 450 lbs (204 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.51US440 |
| 801.51US450 Rectangular shower compartment seat with lift-up feature (USA) 27 5/16" x 15 1/16" (694 x 382 mm) 801.51US450 in product catalogue |  | Shower folding seat, rectangular (USA) - design made from rails and seat elements - functions as a secure seat in the shower area - can be folded up - with corrosion-protected steel core throughout - wall plate made from synthetic material with integrated steel core - 27 5/16 inch (694 mm) wide and 15 1/16 inch (382 mm) deep - seat elements 2 3/16 inch (55 mm) wide - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - concealed fastening - maximum loading capacity 450 lbs (204 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.51US450 |
| 801.51US460 Rectangular shower compartment seat with lift-up feature (USA) 32 3/16" x 15 1/16" (817 x 382 mm) 801.51US460 in product catalogue |  | Shower folding seat, rectangular (USA) - design made from rails and seat elements - functions as a secure seat in the shower area - can be folded up - with corrosion-protected steel core throughout - wall plate made from synthetic material with integrated steel core - 32 3/16 inch (817 mm) wide and 15 1/16 inch (382 mm) deep - seat elements 2 3/16 inch (55 mm) wide - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - concealed fastening - maximum loading capacity 450 lbs (204 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.51US460 |
| 801.51US520 Bathtub seat with lift-up feature (USA) 15 1/4" x 29 1/8" (387 x 740 mm) 801.51US520 in product catalogue |  | Bathtub folding seat (USA) - design made from rails and seat elements - functions as a secure seat in the bathtub - can be folded up - with corrosion-protected steel core throughout - wall plate made from synthetic material with integrated steel core - 15 1/4 inch (387 mm) wide and 29 1/8 inch (740 mm) deep - seat elements 2 3/16 inch (55 mm) wide - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - concealed fastening - maximum loading capacity 450 lbs (204 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.51US520 |
| 900.51.20640 L-shaped shower compartment seat with lift-up feature (USA) 900.51.20640 in product catalogue |  | Shower folding seat, L-shape (USA) - made of high-quality polished stainless steel with one-piece HPL seat, screwed from the underside - functions as a secure seat in the shower area - can be folded up - wall bracket made of stainless steel - projecting seat element left - projecting seat element: 13 inch (330 mm) wide and 22 13/16 inch (580 mm) deep - narrow seat element: 13 inch (330 mm) wide and 15 3/4 inch (400 mm) deep - low-maintenance brakes on shower seat allow seat surface to remain in an upright position after folding up - surfaces enable easy cleaning and hygiene - additional wall support with high-quality polished stainless steel bracket with high-quality polished stainless steel wall bracket - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - maximum loading capacity 450 lbs (204 kg) - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 - shipping only possible on pallet Article no.: HEWI 900.51.20640 |
| 900.51.206XA L-shaped shower compartment seat with lift-up feature (USA) satin 900.51.206XA in product catalogue |  | Shower folding seat, L-shape (USA) - made of high-quality stainless steel with one-piece HPL seat, screwed from the underside - functions as a secure seat in the shower area - can be folded up - wall bracket made of stainless steel - projecting seat element left - projecting seat element: 13 inch (330 mm) wide and 22 13/16 inch (580 mm) deep - narrow seat element: 13 inch (330 mm) wide and 15 3/4 inch (400 mm) deep - low-maintenance brakes on shower seat allow seat surface to remain in an upright position after folding up - surfaces enable easy cleaning and hygiene - additional wall support with high-quality stainless steel bracket with stainless steel wall bracket - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - maximum loading capacity 450 lbs (204 kg) - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 - shipping only possible on pallet Article no.: HEWI 900.51.206XA |
| 900.51.20740 L-shaped shower compartment seat with lift-up feature (USA) 900.51.20740 in product catalogue |  | Shower folding seat, L-shape (USA) - made of high-quality polished stainless steel with one-piece HPL seat, screwed from the underside - functions as a secure seat in the shower area - can be folded up - wall bracket made of stainless steel - projecting seat element right - projecting seat element: 13 inch (330 mm) wide and 22 13/16 inch (580 mm) deep - narrow seat element: 13 inch (330 mm) wide and 15 3/4 inch (400 mm) deep - low-maintenance brakes on shower seat allow seat surface to remain in an upright position after folding up - surfaces enable easy cleaning and hygiene - additional wall support with high-quality polished stainless steel bracket with high-quality polished stainless steel wall bracket - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - maximum loading capacity 450 lbs (204 kg) - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 - shipping only possible on pallet Article no.: HEWI 900.51.20740 |
| 900.51.207XA L-shaped shower compartment seat with lift-up feature (USA) satin 900.51.207XA in product catalogue |  | Shower folding seat, L-shape (USA) - made of high-quality stainless steel with one-piece HPL seat, screwed from the underside - functions as a secure seat in the shower area - can be folded up - wall bracket made of stainless steel - projecting seat element right - projecting seat element: 13 inch (330 mm) wide and 22 13/16 inch (580 mm) deep - narrow seat element: 13 inch (330 mm) wide and 15 3/4 inch (400 mm) deep - low-maintenance brakes on shower seat allow seat surface to remain in an upright position after folding up - surfaces enable easy cleaning and hygiene - additional wall support with high-quality stainless steel bracket with stainless steel wall bracket - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - maximum loading capacity 450 lbs (204 kg) - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 - shipping only possible on pallet Article no.: HEWI 900.51.207XA |
| 900.51.20840 L-shaped shower compartment seat with lift-up feature (USA) 900.51.20840 in product catalogue |  | Shower folding seat, L-shape (USA) - made of high-quality polished stainless steel with one-piece HPL seat, screwed from the underside - functions as a secure seat in the shower area - can be folded up - wall bracket made of stainless steel - projecting seat element left - narrow seat element: 18 7/8 inch (480 mm) wide and 15 3/4 inch (400 mm) deep - projecting seat element: 13 inch (330 mm) wide and 22 13/16 inch (580 mm) deep - low-maintenance brakes on shower seat allow seat surface to remain in an upright position after folding up - surfaces enable easy cleaning and hygiene - additional wall support with high-quality polished stainless steel bracket with high-quality polished stainless steel wall bracket - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board- maximum loading capacity 450 lbs (204 kg) - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 - shipping only possible on pallet Article no.: HEWI 900.51.20840 |
| 900.51.208XA L-shaped shower compartment seat with lift-up feature (USA) satin 900.51.208XA in product catalogue |  | Shower folding seat, L-shape (USA) - made of high-quality stainless steel with one-piece HPL seat, screwed from the underside - functions as a secure seat in the shower area - can be folded up - wall bracket made of stainless steel - projecting seat element left - narrow seat element: 18 7/8 inch (480 mm) wide and 15 3/4 inch (400 mm) deep - projecting seat element: 13 inch (330 mm) wide and 22 13/16 inch (580 mm) deep - low-maintenance brakes on shower seat allow seat surface to remain in an upright position after folding up - surfaces enable easy cleaning and hygiene - additional wall support with high-quality stainless steel bracket with stainless steel wall bracket - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board- maximum loading capacity 450 lbs (204 kg) - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 - shipping only possible on pallet Article no.: HEWI 900.51.208XA |
| 900.51.20940 L-shaped shower compartment seat with lift-up feature (USA) 900.51.20940 in product catalogue |  | Shower folding seat, L-shape (USA) - made of high-quality polished stainless steel with one-piece HPL seat, screwed from the underside - functions as a secure seat in the shower area - can be folded up - wall bracket made of stainless steel - projecting seat element right - narrow seat element: 18 7/8 inch (480 mm) wide and 15 3/4 inch (400 mm) deep - projecting seat element: 13 inch (330 mm) wide and 22 13/16 inch (580 mm) deep - low-maintenance brakes on shower seat allow seat surface to remain in an upright position after folding up - surfaces enable easy cleaning and hygiene - additional wall support with high-quality polished stainless steel bracket with high-quality polished stainless steel wall bracket - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - maximum loading capacity 450 lbs (204 kg) - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 - shipping only possible on pallet Article no.: HEWI 900.51.20940 |
| 900.51.209XA L-shaped shower compartment seat with lift-up feature (USA) satin 900.51.209XA in product catalogue |  | Shower folding seat, L-shape (USA) - made of high-quality stainless steel with one-piece HPL seat, screwed from the underside - functions as a secure seat in the shower area - can be folded up - wall bracket made of stainless steel - projecting seat element right - narrow seat element: 18 7/8 inch (480 mm) wide and 15 3/4 inch (400 mm) deep - projecting seat element: 13 inch (330 mm) wide and 22 13/16 inch (580 mm) deep - low-maintenance brakes on shower seat allow seat surface to remain in an upright position after folding up - surfaces enable easy cleaning and hygiene - additional wall support with high-quality stainless steel bracket with stainless steel wall bracket - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board- maximum loading capacity 450 lbs (204 kg) - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 - shipping only possible on pallet Article no.: HEWI 900.51.209XA |
| 900.51.21040 Rectangular shower compartment seat with lift-up feature (USA) 900.51.21040 in product catalogue |  | Shower folding seat, rectangular (USA) - made of high-quality polished stainless steel with one-piece HPL seat, screwed from the underside - functions as a secure seat in the shower area - can be folded up - wall bracket made of stainless steel - low-maintenance brakes on shower seat allow seat surface to remain in an upright position after folding up - surfaces enable easy cleaning and hygiene - additional wall support with high-quality polished stainless steel bracket with high-quality polished stainless steel wall bracket - 20 1/16 inch (510 mm) wide and 15 3/4 inch (400 mm) deep - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - maximum loading capacity 450 lbs (204 kg) - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 - shipping only possible on pallet Article no.: HEWI 900.51.21040 |
| 900.51.210XA Rectangular shower compartment seat with lift-up feature (USA) satin 900.51.210XA in product catalogue |  | Shower folding seat, rectangular (USA) - made of high-quality stainless steel with one-piece HPL seat, screwed invisibly, screwed from the underside - functions as a secure seat in the shower area - can be folded up - wall bracket made of stainless steel - low-maintenance brakes on shower seat allow seat surface to remain in an upright position after folding up - surfaces enable easy cleaning and hygiene - additional wall support with high-quality stainless steel bracket with stainless steel wall bracket - 20 1/16 inch (510 mm) wide and 15 3/4 inch (400 mm) deep - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - maximum loading capacity 450 lbs (204 kg) - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 - shipping only possible on pallet Article no.: HEWI 900.51.210XA |
| 900.51.21140 Rectangular shower compartment seat with lift-up feature (USA) 900.51.21140 in product catalogue |  | Shower folding seat, rectangular (USA) - made of high-quality polished stainless steel with one-piece HPL seat, screwed from the underside - functions as a secure seat in the shower area - can be folded up - wall bracket made of stainless steel - low-maintenance brakes on shower seat allow seat surface to remain in an upright position after folding up - surfaces enable easy cleaning and hygiene - additional wall support with high-quality polished stainless steel bracket with high-quality polished stainless steel wall bracket - 26 inch (660 mm) wide and 15 3/4 inch (400 mm) deep - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - maximum loading capacity 450 lbs (204 kg) - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 - shipping only possible on pallet Article no.: HEWI 900.51.21140 |
| 900.51.211XA Rectangular shower compartment seat with lift-up feature (USA) satin 900.51.211XA in product catalogue |  | Shower folding seat, rectangular (USA) - made of high-quality stainless steel with one-piece HPL seat, screwed from the underside - functions as a secure seat in the shower area - can be folded up - wall bracket made of stainless steel - low-maintenance brakes on shower seat allow seat surface to remain in an upright position after folding up - surfaces enable easy cleaning and hygiene - additional wall support with high-quality stainless steel bracket with stainless steel wall bracket - 26 inch (660 mm) wide and 15 3/4 inch (400 mm) deep - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - maximum loading capacity 450 lbs (204 kg) - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 - shipping only possible on pallet Article no.: HEWI 900.51.211XA |
| 900.51.21240 Rectangular shower compartment seat with lift-up feature (USA) 900.51.21240 in product catalogue |  | Shower folding seat, rectangular (USA) - made of high-quality polished stainless steel with one-piece HPL seat, screwed from the underside - functions as a secure seat in the shower area - can be folded up - wall bracket made of stainless steel - low-maintenance brakes on shower seat allow seat surface to remain in an upright position after folding up - surfaces enable easy cleaning and hygiene - additional wall support with high-quality polished stainless steel bracket with high-quality polished stainless steel wall bracket - 31 7/8 inch (810 mm) wide and 15 3/4 inch (400 mm) deep - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - maximum loading capacity 450 lbs (204 kg) - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 - shipping only possible on pallet Article no.: HEWI 900.51.21240 |
| 900.51.212XA Rectangular shower compartment seat with lift-up feature (USA) satin 900.51.212XA in product catalogue |  | Shower folding seat, rectangular (USA) - made of high-quality stainless steel with one-piece HPL seat, screwed from the underside - functions as a secure seat in the shower area - can be folded up - wall bracket made of stainless steel - low-maintenance brakes on shower seat allow seat surface to remain in an upright position after folding up - surfaces enable easy cleaning and hygiene - additional wall support with high-quality stainless steel bracket with stainless steel wall bracket - 31 7/8 inch (810 mm) wide and 15 3/4 inch (400 mm) deep - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - maximum loading capacity 450 lbs (204 kg) - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 - shipping only possible on pallet Article no.: HEWI 900.51.212XA |
| 900.51.21340 Bathtub seat with lift-up feature (USA) 900.51.21340 in product catalogue |  | Bathtub seat with lift-up feature (USA) - made of high-quality polished stainless steel with one-piece HPL seat, screwed from the underside - functions as a secure seat in the bathtub - can be folded up - wall bracket made of stainless steel - low-maintenance brakes on shower seat allow seat surface to remain in an upright position after folding up - surfaces enable easy cleaning and hygiene - 14 15/16 inch (380 mm) wide and 29 inch (737 mm) deep - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - maximum loading capacity 450 lbs (204 kg) - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 - shipping only possible on pallet Article no.: HEWI 900.51.21340 |
| 900.51.213XA Bathtub seat with lift-up feature (USA) satin 900.51.213XA in product catalogue |  | Bathtub seat with lift-up feature (USA) - made of high-quality stainless steel with one-piece HPL seat, screwed from the underside - functions as a secure seat in the bathtub - can be folded up - wall bracket made of stainless steel - low-maintenance brakes on shower seat allow seat surface to remain in an upright position after folding up - surfaces enable easy cleaning and hygiene - 14 15/16 inch (380 mm) wide and 29 inch (737 mm) deep - includes fixing material for mounting to cavity walls with wood backlining of two 2 inch x 12 inch laminate or hardwood boards connected with a 1/2 inch x 12 inch plywood board - maximum loading capacity 450 lbs (204 kg) - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 - shipping only possible on pallet Article no.: HEWI 900.51.213XA |
| Sit => Removable hanging seats |  |  |
| 801.51BUS110 Removable hanging seat (USA) Matt edition 801.51BUS110 in product catalogue |  |  |
| 801.51US100 Removable hanging seat (USA) 13 9/16" x 18 3/4" x 18 3/8" (345 x 477 x 466 mm) 801.51US100 in product catalogue |  | Removable hanging seat (USA) from Range 801 - design made from rails, seat and backrest elements - with corrosion-protected steel core throughout - functions as a removable seat in the shower area - can be hung on the shower grab rail and grab bar (USA), rail diameter 1 5/16 inch (33 mm) - 13 9/16 inch (345 mm) wide, 18 3/4 inch (477 mm) high, 18 3/8 inch (466 mm) deep - seat area 13 9/16 inch (345 mm) wide, 13 1/4 inch (336 mm) deep - seat elements 2 3/16 inch (55 mm) wide - maximum loading capacity 330 lbs (150 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) Article no.: HEWI 801.51US100 |
| 801.51US110 Removable hanging seat (USA) 17 5/8" x 18 11/16" x 19 1/8" (448 x 475 x 486 mm) 801.51US110 in product catalogue |  | Removable hanging seat (USA) from Range 801 - design made from rails, seat and backrest elements - with corrosion-protected steel core throughout - functions as a removable seat in the shower area - can be hung on the shower grab rail and grab bar (USA), rail diameter 1 5/16 inch (33 mm) - 17 5/8 inch (448 mm) wide, 18 11/16 inch (475 mm) high, 19 1/8 inch (486 mm) deep - seat area 17 5/8 inch (448 mm) wide, 13 15/16 inch (354 mm) deep - seat elements 2 3/16 inch (55 mm) wide - maximum loading capacity 330 lbs (150 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) Article no.: HEWI 801.51US110 |
| Sit => Shower benches |  |  |
| 801.51.300 Stool 801.51.300 in product catalogue |  | Range 801 Shower stool - made of rails and seat slats - with continuous, corrosion resistant steel core - enables user to sit safely in the bathroom - maximum weight capacity 150 kg - for mobile use - 345 mm wide, 505 mm high, 336 mm deep - seat slats 55 mm wide - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 801.51.300 |
| 801.51.8400 Shower bench with weight capacity up to 350 kg 801.51.8400 in product catalogue |  | Range 801 Shower bench - suitable for use in the shower - made of rail system, diameter 33 mm, with continuous, corrosion resistant steel core - with additional, side support rails - used for safe sitting in the bathroom and shower area - maximum weight capacity 350 kg - for mobile use - 688 mm wide, 448 mm deep, 500 mm seat height, 612 mm total height - with 55 mm wide polyamide seat slats - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 801.51.8400 |
| 801.51.8410 Shower bench with weight capacity up to 350 kg 801.51.8410 in product catalogue |  | Range 801 Shower bench - suitable for use in the shower - made of rail system, diameter 33 mm, with continuous, corrosion resistant steel core - with additional, side support rails - used for safe sitting in the bathroom and shower area - maximum weight capacity 350 kg - for mobile use - 960 mm wide, 448 mm deep, 500 mm seat height, 612 mm total height - with 55 mm wide polyamide seat slats - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 801.51.8410 |
| 801.51B300 Stool Matt edition 801.51B300 in product catalogue |  | Range 801 Shower stool - made of rails and seat slats - with continuous, corrosion resistant steel core - enables user to sit safely in the bathroom - maximum weight capacity 150 kg - for mobile use - 345 mm wide, 505 mm high, 336 mm deep - seat slats 55 mm wide - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 801.51B300 |
| Support |  |  |
| Support => Swing-up grab bars |  |  |
| 801.50.100 Hinged support rail, rotatable 600 mm projection 801.50.100 in product catalogue |  | Range 801 Hinged support rail (rotatable) - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core and wall plate made of polyamide with integrated steel core - can be folded upwards and, folded downwards with braking mechanism, in raised position can be turned to left or right towards the wall - for wall mounting with wall-specific HEWI fixing material (must be ordered separately) - concealed fixing - 600 mm projection, 259 mm high and 78 mm deep, 33 mm rail diameter - made of high-quality polyamide according to HEWI colour chart - toilet roll holder 801.50.010 and 801.50.011, WC flushing mechanism (radio-controlled) 801.50.060 and floor supports 950.50.021XA and 950.50.023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 801.50.100 |
| 801.50.107 Mobile hinged support rail, rotatable 600 mm 801.50.107 in product catalogue |  | Range 801 mobile hinged support rail (rotatable) - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core - can be folded upwards and, folded downwards with braking mechanism, in raised position can be turned to left or right towards the wall - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 950.50.016, must be ordered separately - toolless and quick installation by simple attachment of the mobile hinged support rail - secured by fixing screw to prevent unintentional removal - 600 mm projection, 252 mm high and 78 mm deep, 33 mm rail diameter - made of high-quality polyamide according to HEWI colour chart - toilet roll holder 801.50.010 and 801.50.011, WC flushing mechanism (radio-controlled) 801.50.060 and floor supports 950.50.021XA and 950.50.023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 801.50.107 |
| 801.50.110 Hinged support rail, rotatable 700 mm projection 801.50.110 in product catalogue |  | Range 801 Hinged support rail (rotatable) - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core and wall plate made of polyamide with integrated steel core - can be folded upwards and, folded downwards with braking mechanism, in raised position can be turned to left or right towards the wall - for wall mounting with wall-specific HEWI fixing material (must be ordered separately) - concealed fixing - 700 mm projection, 259 mm high and 78 mm deep, 33 mm rail diameter - made of high-quality polyamide according to HEWI colour chart - toilet roll holder 801.50.010 and 801.50.011, WC flushing mechanism (radio-controlled) 801.50.060 and floor supports 950.50.021XA and 950.50.023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 801.50.110 |
| 801.50.117 Mobile hinged support rail, rotatable 700 mm 801.50.117 in product catalogue |  | Range 801 mobile hinged support rail (rotatable) - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core - can be folded upwards and, folded downwards with braking mechanism, in raised position can be turned to left or right towards the wall - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 950.50.016, must be ordered separately - toolless and quick installation by simple attachment of the mobile hinged support rail - secured by fixing screw to prevent unintentional removal - 700 mm projection, 252 mm high and 78 mm deep, 33 mm rail diameter - made of high-quality polyamide according to HEWI colour chart - toilet roll holder 801.50.010 and 801.50.011, WC flushing mechanism (radio-controlled) 801.50.060 and floor supports 950.50.021XA and 950.50.023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 801.50.117 |
| 801.50.120 Hinged support rail, rotatable 850 mm projection 801.50.120 in product catalogue |  | Range 801 Hinged support rail (rotatable) - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core and wall plate made of polyamide with integrated steel core - can be folded upwards and, folded downwards with braking mechanism, in raised position can be turned to left or right towards the wall - for wall mounting with wall-specific HEWI fixing material (must be ordered separately) - concealed fixing - 850 mm projection, 259 mm high and 78 mm deep, 33 mm rail diameter - made of high-quality polyamide according to HEWI colour chart - toilet roll holder 801.50.010 and 801.50.011, WC flushing mechanism (radio-controlled) 801.50.060 and floor supports 950.50.021XA and 950.50.023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 801.50.120 |
| 801.50.127 Mobile hinged support rail, rotatable 850 mm 801.50.127 in product catalogue |  | Range 801 mobile hinged support rail (rotatable) - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core - can be folded upwards and, folded downwards with braking mechanism, in raised position can be turned to left or right towards the wall - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 950.50.016, must be ordered separately - toolless and quick installation by simple attachment of the mobile hinged support rail - secured by fixing screw to prevent unintentional removal - 850 mm projection, 252 mm high and 78 mm deep, 33 mm rail diameter - made of high-quality polyamide according to HEWI colour chart - toilet roll holder 801.50.010 and 801.50.011, WC flushing mechanism (radio-controlled) 801.50.060 and floor supports 950.50.021XA and 950.50.023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 801.50.127 |
| 801.50.130 Hinged support rail 900 mm projection 801.50.130 in product catalogue |  | Range 801 Hinged support rail (rotatable) - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core and wall plate made of polyamide with integrated steel core - can be folded upwards and, folded downwards with braking mechanism, in raised position can be turned to left or right towards the wall - for wall mounting with wall-specific HEWI fixing material (must be ordered separately) - concealed fixing - 900 mm projection, 259 mm high and 78 mm wide, 33 mm rail diameter - made of high-quality polyamide according to HEWI colour chart - toilet roll holder 801.50.010 and 801.50.011, WC flushing mechanism (radio-controlled) 801.50.060 and floor supports 950.50.021XA and 950.50.023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 801.50.130 |
| 801.50.137 Mobile hinged support rail, rotatable 900 mm 801.50.137 in product catalogue |  | Range 801 mobile hinged support rail (rotatable) - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core - can be folded upwards and, folded downwards with braking mechanism, in raised position can be turned to left or right towards the wall - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 950.50.016, must be ordered separately - toolless and quick installation by simple attachment of the mobile hinged support rail - secured by fixing screw to prevent unintentional removal - 900 mm projection, 252 mm high and 78 mm wide, 33 mm rail diameter - made of high-quality polyamide by HEWI colour table - toilet roll holder 801.50.010 and 801.50.011, WC flushing mechanism (radio-controlled) 801.50.060 and floor supports 950.50.021XA and 950.50.023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 801.50.137 |
| 801.50.140 Hinged support rail, rotatable 750 mm projection 801.50.140 in product catalogue |  | Range 801 Hinged support rail (rotatable) - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core and wall plate made of polyamide with integrated steel core - can be folded upwards and, folded downwards with braking mechanism, in raised position can be turned to left or right towards the wall - for wall mounting with wall-specific HEWI fixing material (must be ordered separately) - concealed fixing - 750 mm projection, 259 mm high and 78 mm wide, 33 mm rail diameter - made of high-quality polyamide by HEWI colour table - toilet roll holder 801.50.010 and 801.50.011, WC flushing mechanism (radio-controlled) 801.50.060 and floor supports 950.50.021XA and 950.50.023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) in respect of the projection of the hinged support rail on the toilet Article no.: HEWI 801.50.140 |
| 801.50.147 Mobile hinged support rail, rotatable 750 mm 801.50.147 in product catalogue |  | Range 801 mobile hinged support rail (rotatable) - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core - can be folded upwards and, folded downwards with braking mechanism, in raised position can be turned to left or right towards the wall - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 950.50.016, must be ordered separately - toolless and quick installation by simple attachment of the mobile hinged support rail - secured by fixing screw to prevent unintentional removal - 750 mm projection, 252 mm high and 78 mm wide, 33 mm rail diameter - made of high-quality polyamide by HEWI colour table - toilet roll holder 801.50.010 and 801.50.011, WC flushing mechanism (radio-controlled) 801.50.060 and floor supports 950.50.021XA and 950.50.023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) in respect of the projection of the hinged support rail on the toilet Article no.: HEWI 801.50.147 |
| 801.50.200 Hinged support rail 600 mm projection 801.50.200 in product catalogue |  | Range 801 Hinged support rail - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core and wall plate made of polyamide with integrated steel core - can be folded upwards and, folded downwards with braking mechanism - for wall mounting with wall-specific HEWI fixing material (must be ordered separately) - concealed fixing - 600 mm projection, 259 mm high and 78 mm deep, 33 mm rail diameter - made of high-quality polyamide according to HEWI colour chart - toilet roll holder 801.50.010 and 801.50.011, WC flushing mechanism (radio-controlled) 801.50.060 and floor supports 950.50.021XA and 950.50.023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the DIN 18040 requirements Article no.: HEWI 801.50.200 |
| 801.50.207 Mobile hinged support rail 600 mm 801.50.207 in product catalogue |  | Range 801 mobile hinged support rail - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core - can be folded upwards and, folded downwards with braking mechanism - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 950.50.016, must be ordered separately - toolless and quick installation by simple attachment of the mobile hinged support rail - secured by fixing screw to prevent unintentional removal - 600 mm projection, 252 mm high and 78 mm deep, 33 mm rail diameter - made of high-quality polyamide according to HEWI colour chart - toilet roll holder 801.50.010 and 801.50.011, WC flushing mechanism (radio-controlled) 801.50.060 and floor supports 950.50.021XA and 950.50.023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the DIN 18040 requirements Article no.: HEWI 801.50.207 |
| 801.50.210 Hinged support rail 700 mm projection 801.50.210 in product catalogue |  | Range 801 Hinged support rail - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core and wall plate made of polyamide with integrated steel core - can be folded upwards and, folded downwards with braking mechanism - for wall mounting with wall-specific HEWI fixing material (must be ordered separately) - concealed fixing - 700 mm projection, 259 mm high and 78 mm deep, 33 mm rail diameter - made of high-quality polyamide according to HEWI colour chart - toilet roll holder 801.50.010 and 801.50.011, WC flushing mechanism (radio-controlled) 801.50.060 and floor supports 950.50.021XA and 950.50.023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 801.50.210 |
| 801.50.217 Mobile hinged support rail 700 mm 801.50.217 in product catalogue |  | Range 801 mobile hinged support rail - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core - can be folded upwards and, folded downwards with braking mechanism - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 950.50.016, must be ordered separately - toolless and quick installation by simple attachment of the mobile hinged support rail - secured by fixing screw to prevent unintentional removal - 700 mm projection, 252 mm high and 78 mm deep, 33 mm rail diameter - made of high-quality polyamide according to HEWI colour chart - toilet roll holder 801.50.010 and 801.50.011, WC flushing mechanism (radio-controlled) 801.50.060 and floor supports 950.50.021XA and 950.50.023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 801.50.217 |
| 801.50.220 Hinged support rail 850 mm projection 801.50.220 in product catalogue |  | Range 801 Hinged support rail - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core and wall plate made of polyamide with integrated steel core - can be folded upwards and, folded downwards with braking mechanism - for wall mounting with wall-specific HEWI fixing material (must be ordered separately) - concealed fixing - 850 mm projection, 259 mm high and 78 mm deep, 33 mm rail diameter - made of high-quality polyamide according to HEWI colour chart - toilet roll holder 801.50.010 and 801.50.011, WC flushing mechanism (radio-controlled) 801.50.060 and floor supports 950.50.021XA and 950.50.023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 801.50.220 |
| 801.50.227 Mobile hinged support rail 850 mm 801.50.227 in product catalogue |  | Range 801 mobile hinged support rail - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core - can be folded upwards and, folded downwards with braking mechanism - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 950.50.016, must be ordered separately - toolless and quick installation by simple attachment of the mobile hinged support rail - secured by fixing screw to prevent unintentional removal - 850 mm projection, 252 mm high and 78 mm deep, 33 mm rail diameter - made of high-quality polyamide according to HEWI colour chart - toilet roll holder 801.50.010 and 801.50.011, WC flushing mechanism (radio-controlled) 801.50.060 and floor supports 950.50.021XA and 950.50.023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 801.50.227 |
| 801.50.230 Hinged support rail 900 mm projection 801.50.230 in product catalogue |  | Range 801 Hinged support rail - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core and wall plate made of polyamide with integrated steel core - can be folded upwards and, folded downwards with braking mechanism - for wall mounting with wall-specific HEWI fixing material (must be ordered separately) - concealed fixing - 900 mm projection, 259 mm high and 78 mm wide, 33 mm rail diameter - made of high-quality polyamide according to HEWI colour chart - toilet roll holder 801.50.010 and 801.50.011, WC flushing mechanism (radio-controlled) 801.50.060 and floor supports 950.50.021XA and 950.50.023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 801.50.230 |
| 801.50.237 Mobile hinged support rail 900 mm 801.50.237 in product catalogue |  | Range 801 mobile hinged support rail - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core - can be folded upwards and, folded downwards with braking mechanism - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 950.50.016, must be ordered separately - toolless and quick installation by simple attachment of the mobile hinged support rail - secured by fixing screw to prevent unintentional removal - 900 mm projection, 252 mm high and 78 mm wide, 33 mm rail diameter - made of high-quality polyamide by HEWI colour table - toilet roll holder 801.50.010 and 801.50.011, WC flushing mechanism (radio-controlled) 801.50.060 and floor supports 950.50.021XA and 950.50.023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 801.50.237 |
| 801.50.240 Hinged support rail 750 mm projection 801.50.240 in product catalogue |  | Range 801 Hinged support rail - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core and wall plate made of polyamide with integrated steel core - can be folded upwards and, folded downwards with braking mechanism - for wall mounting with wall-specific HEWI fixing material (must be ordered separately) - concealed fixing - 750 mm projection, 259 mm high and 78 mm wide, 33 mm rail diameter - made of high-quality polyamide by HEWI colour table - toilet roll holder 801.50.010 and 801.50.011, WC flushing mechanism (radio-controlled) 801.50.060 and floor supports 950.50.021XA and 950.50.023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) in respect of the projection of the hinged support rail on the toilet Article no.: HEWI 801.50.240 |
| 801.50.247 Mobile hinged support rail 750 mm 801.50.247 in product catalogue |  | Range 801 mobile hinged support rail - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core - can be folded upwards and, folded downwards with braking mechanism - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 950.50.016, must be ordered separately - toolless and quick installation by simple attachment of the mobile hinged support rail - secured by fixing screw to prevent unintentional removal - 750 mm projection, 252 mm high and 78 mm wide, 33 mm rail diameter - made of high-quality polyamide by HEWI colour table - toilet roll holder 801.50.010 and 801.50.011, WC flushing mechanism (radio-controlled) 801.50.060 and floor supports 950.50.021XA and 950.50.023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) in respect of the projection of the hinged support rail on the toilet Article no.: HEWI 801.50.247 |
| 801.50B100 Hinged support rail, rotatable 600 mm Matt edition 801.50B100 in product catalogue |  | Range 801 Hinged support rail (rotatable) - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core and wall plate made of polyamide with integrated steel core - can be folded upwards and, folded downwards with braking mechanism, in raised position can be turned to left or right towards the wall - for wall mounting with wall-specific HEWI fixing material (must be ordered separately) - concealed fixing - 600 mm projection, 259 mm high and 78 mm deep, 33 mm rail diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) - toilet roll holder 801.50B010 and 801.50B011, WC flushing mechanism (radio-controlled) 801.50B060 and floor supports 950.50B021XA and 950.50B023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 801.50B100 |
| 801.50B107 Mobile hinged support rail, rotatable Matt edition 801.50B107 in product catalogue |  | Range 801 mobile hinged support rail (rotatable) - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core - can be folded upwards and, folded downwards with braking mechanism, in raised position can be turned to left or right towards the wall - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 950.50.016, must be ordered separately - toolless and quick installation by simple attachment of the mobile hinged support rail - secured by fixing screw to prevent unintentional removal - 600 mm projection, 252 mm high and 78 mm deep, 33 mm rail diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) - toilet roll holder 801.50B010 and 801.50B011, WC flushing mechanism (radio-controlled) 801.50B060 and floor supports 950.50B021XA and 950.50B023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 801.50B107 |
| 801.50B110 Hinged support rail, rotatable 700 mm Matt edition 801.50B110 in product catalogue |  | Range 801 Hinged support rail (rotatable) - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core and wall plate made of polyamide with integrated steel core - can be folded upwards and, folded downwards with braking mechanism, in raised position can be turned to left or right towards the wall - for wall mounting with wall-specific HEWI fixing material (must be ordered separately) - concealed fixing - 700 mm projection, 259 mm high and 78 mm deep, 33 mm rail diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) - toilet roll holder 801.50B010 and 801.50B011, WC flushing mechanism (radio-controlled) 801.50B060 and floor supports 950.50B021XA and 950.50B023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 801.50B110 |
| 801.50B117 Mobile hinged support rail, rotatable Matt edition 801.50B117 in product catalogue |  | Range 801 mobile hinged support rail (rotatable) - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core - can be folded upwards and, folded downwards with braking mechanism, in raised position can be turned to left or right towards the wall - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 950.50.016, must be ordered separately - toolless and quick installation by simple attachment of the mobile hinged support rail - secured by fixing screw to prevent unintentional removal - 700 mm projection, 252 mm high and 78 mm deep, 33 mm rail diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) - toilet roll holder 801.50B010 and 801.50B011, WC flushing mechanism (radio-controlled) 801.50B060 and floor supports 950.50B021XA and 950.50B023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 801.50B117 |
| 801.50B120 Hinged support rail, rotatable 850 mm Matt edition 801.50B120 in product catalogue |  | Range 801 Hinged support rail (rotatable) - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core and wall plate made of polyamide with integrated steel core - can be folded upwards and, folded downwards with braking mechanism, in raised position can be turned to left or right towards the wall - for wall mounting with wall-specific HEWI fixing material (must be ordered separately) - concealed fixing - 850 mm projection, 259 mm high and 78 mm deep, 33 mm rail diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) - toilet roll holder 801.50B010 and 801.50B011, WC flushing mechanism (radio-controlled) 801.50B060 and floor supports 950.50B021XA and 950.50B023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 801.50B120 |
| 801.50B127 Mobile hinged support rail, rotatable Matt edition 801.50B127 in product catalogue |  | Range 801 mobile hinged support rail (rotatable) - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core - can be folded upwards and, folded downwards with braking mechanism, in raised position can be turned to left or right towards the wall - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 950.50.016, must be ordered separately - toolless and quick installation by simple attachment of the mobile hinged support rail - secured by fixing screw to prevent unintentional removal - 850 mm projection, 252 mm high and 78 mm deep, 33 mm rail diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) - toilet roll holder 801.50B010 and 801.50B011, WC flushing mechanism (radio-controlled) 801.50B060 and floor supports 950.50B021XA and 950.50B023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 801.50B127 |
| 801.50B130 Hinged support rail, rotatable 900 mm Matt edition 801.50B130 in product catalogue |  | Range 801 Hinged support rail (rotatable) - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core and wall plate made of polyamide with integrated steel core - can be folded upwards and, folded downwards with braking mechanism, in raised position can be turned to left or right towards the wall - for wall mounting with wall-specific HEWI fixing material (must be ordered separately) - concealed fixing - 900 mm projection, 259 mm high and 78 mm wide, 33 mm rail diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) - toilet roll holder 801.50B010 and 801.50B011, WC flushing mechanism (radio-controlled) 801.50B060 and floor supports 950.50B021XA and 950.50B023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 801.50B130 |
| 801.50B137 Mobile hinged support rail, rotatable Matt edition 801.50B137 in product catalogue |  | Range 801 mobile hinged support rail (rotatable) - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core - can be folded upwards and, folded downwards with braking mechanism, in raised position can be turned to left or right towards the wall - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 950.50.016, must be ordered separately - toolless and quick installation by simple attachment of the mobile hinged support rail - secured by fixing screw to prevent unintentional removal - 900 mm projection, 252 mm high and 78 mm wide, 33 mm rail diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) - toilet roll holder 801.50B010 and 801.50B011, WC flushing mechanism (radio-controlled) 801.50B060 and floor supports 950.50B021XA and 950.50B023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 801.50B137 |
| 801.50B140 Hinged support rail, rotatable 750 mm Matt edition 801.50B140 in product catalogue |  | Range 801 Hinged support rail (rotatable) - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core and wall plate made of polyamide with integrated steel core - can be folded upwards and, folded downwards with braking mechanism, in raised position can be turned to left or right towards the wall - for wall mounting with wall-specific HEWI fixing material (must be ordered separately) - concealed fixing - 750 mm projection, 259 mm high and 78 mm wide, 33 mm rail diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) - toilet roll holder 801.50B010 and 801.50B011, WC flushing mechanism (radio-controlled) 801.50B060 and floor supports 950.50B021XA and 950.50B023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) in respect of the projection of the hinged support rail on the toilet Article no.: HEWI 801.50B140 |
| 801.50B147 Mobile hinged support rail, rotatable Matt edition 801.50B147 in product catalogue |  | Range 801 mobile hinged support rail (rotatable) - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core - can be folded upwards and, folded downwards with braking mechanism, in raised position can be turned to left or right towards the wall - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 950.50.016, must be ordered separately - toolless and quick installation by simple attachment of the mobile hinged support rail - secured by fixing screw to prevent unintentional removal - 750 mm projection, 252 mm high and 78 mm wide, 33 mm rail diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) - toilet roll holder 801.50B010 and 801.50B011, WC flushing mechanism (radio-controlled) 801.50B060 and floor supports 950.50B021XA and 950.50B023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) in respect of the projection of the hinged support rail on the toilet Article no.: HEWI 801.50B147 |
| 801.50B200 Hinged support rail 600 mm Matt edition 801.50B200 in product catalogue |  | Range 801 Hinged support rail - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core and wall plate made of polyamide with integrated steel core - can be folded upwards and, folded downwards with braking mechanism - for wall mounting with wall-specific HEWI fixing material (must be ordered separately) - concealed fixing - 600 mm projection, 259 mm high and 78 mm deep, 33 mm rail diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) - toilet roll holder 801.50B010 and 801.50B011, WC flushing mechanism (radio-controlled) 801.50B060 and floor supports 950.50B021XA and 950.50B023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the DIN 18040 requirements Article no.: HEWI 801.50B200 |
| 801.50B207 Mobile hinged support rail Matt edition 801.50B207 in product catalogue |  | Range 801 mobile hinged support rail - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core - can be folded upwards and, folded downwards with braking mechanism - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 950.50.016, must be ordered separately - toolless and quick installation by simple attachment of the mobile hinged support rail - secured by fixing screw to prevent unintentional removal - 600 mm projection, 252 mm high and 78 mm deep, 33 mm rail diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) - toilet roll holder 801.50B010 and 801.50B011, WC flushing mechanism (radio-controlled) 801.50B060 and floor supports 950.50B021XA and 950.50B023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the DIN 18040 requirements Article no.: HEWI 801.50B207 |
| 801.50B210 Hinged support rail 700 mm Matt edition 801.50B210 in product catalogue |  | Range 801 Hinged support rail - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core and wall plate made of polyamide with integrated steel core - can be folded upwards and, folded downwards with braking mechanism - for wall mounting with wall-specific HEWI fixing material (must be ordered separately) - concealed fixing - 700 mm projection, 259 mm high and 78 mm deep, 33 mm rail diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) - toilet roll holder 801.50B010 and 801.50B011, WC flushing mechanism (radio-controlled) 801.50B060 and floor supports 950.50B021XA and 950.50B023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 801.50B210 |
| 801.50B217 Mobile hinged support rail Matt edition 801.50B217 in product catalogue |  | Range 801 mobile hinged support rail - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core - can be folded upwards and, folded downwards with braking mechanism - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 950.50.016, must be ordered separately - toolless and quick installation by simple attachment of the mobile hinged support rail - secured by fixing screw to prevent unintentional removal - 700 mm projection, 252 mm high and 78 mm deep, 33 mm rail diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) - toilet roll holder 801.50B010 and 801.50B011, WC flushing mechanism (radio-controlled) 801.50B060 and floor supports 950.50B021XA and 950.50B023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 801.50B217 |
| 801.50B220 Hinged support rail 850 mm Matt edition 801.50B220 in product catalogue |  | Range 801 Hinged support rail - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core and wall plate made of polyamide with integrated steel core - can be folded upwards and, folded downwards with braking mechanism - for wall mounting with wall-specific HEWI fixing material (must be ordered separately) - concealed fixing - 850 mm projection, 259 mm high and 78 mm deep, 33 mm rail diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) - toilet roll holder 801.50B010 and 801.50B011, WC flushing mechanism (radio-controlled) 801.50B060 and floor supports 950.50B021XA and 950.50B023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 801.50B220 |
| 801.50B227 Mobile hinged support rail Matt edition 801.50B227 in product catalogue |  | Range 801 mobile hinged support rail - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core - can be folded upwards and, folded downwards with braking mechanism - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 950.50.016, must be ordered separately - toolless and quick installation by simple attachment of the mobile hinged support rail - secured by fixing screw to prevent unintentional removal - 850 mm projection, 252 mm high and 78 mm deep, 33 mm rail diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) - toilet roll holder 801.50B010 and 801.50B011, WC flushing mechanism (radio-controlled) 801.50B060 and floor supports 950.50B021XA and 950.50B023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 801.50B227 |
| 801.50B230 Hinged support rail 900 mm Matt edition 801.50B230 in product catalogue |  | Range 801 Hinged support rail - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core and wall plate made of polyamide with integrated steel core - can be folded upwards and, folded downwards with braking mechanism - for wall mounting with wall-specific HEWI fixing material (must be ordered separately) - concealed fixing - 900 mm projection, 259 mm high and 78 mm wide, 33 mm rail diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) - toilet roll holder 801.50B010 and 801.50B011, WC flushing mechanism (radio-controlled) 801.50B060 and floor supports 950.50B021XA and 950.50B023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 801.50B230 |
| 801.50B237 Mobile hinged support rail Matt edition 801.50B237 in product catalogue |  | Range 801 mobile hinged support rail - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core - can be folded upwards and, folded downwards with braking mechanism - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 950.50.016, must be ordered separately - toolless and quick installation by simple attachment of the mobile hinged support rail - secured by fixing screw to prevent unintentional removal - 900 mm projection, 252 mm high and 78 mm wide, 33 mm rail diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) - toilet roll holder 801.50B010 and 801.50B011, WC flushing mechanism (radio-controlled) 801.50B060 and floor supports 950.50B021XA and 950.50B023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 801.50B237 |
| 801.50B240 Hinged support rail 750 mm Matt edition 801.50B240 in product catalogue |  | Range 801 Hinged support rail - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core and wall plate made of polyamide with integrated steel core - can be folded upwards and, folded downwards with braking mechanism - for wall mounting with wall-specific HEWI fixing material (must be ordered separately) - concealed fixing - 750 mm projection, 259 mm high and 78 mm wide, 33 mm rail diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) - toilet roll holder 801.50B010 and 801.50B011, WC flushing mechanism (radio-controlled) 801.50B060 and floor supports 950.50B021XA and 950.50B023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) in respect of the projection of the hinged support rail on the toilet Article no.: HEWI 801.50B240 |
| 801.50B247 Mobile hinged support rail Matt edition 801.50B247 in product catalogue |  | Range 801 mobile hinged support rail - two parallel rails running one above the other joined by a connecting bend - for holding on to and providing support - weight capacity 100 kg - with continuous, corrosion resistant steel core - can be folded upwards and, folded downwards with braking mechanism - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 950.50.016, must be ordered separately - toolless and quick installation by simple attachment of the mobile hinged support rail - secured by fixing screw to prevent unintentional removal - 750 mm projection, 252 mm high and 78 mm wide, 33 mm rail diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) - toilet roll holder 801.50B010 and 801.50B011, WC flushing mechanism (radio-controlled) 801.50B060 and floor supports 950.50B021XA and 950.50B023XA for retrofitting - CE marking according to Directive 93/42 / EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) in respect of the projection of the hinged support rail on the toilet Article no.: HEWI 801.50B247 |
| 900.50.00140 Mounting plate chrome 900.50.00140 in product catalogue |  | Mounting plate for mobile hinged support rails, System 900 - rectangular mounting plate with fixing screw - used to hold and fix the mobile hinged support rails 900.50.4...40 - 70 mm wide, 210 mm high, 18 mm deep - made of stainless steel, chrome - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply Article no.: HEWI 900.50.00140 |
| 900.50.00160 Mounting plate powder-coated 900.50.00160 in product catalogue |  | Mounting plate for mobile hinged support rails, System 900 - rectangular mounting plate with fixing screw - used to hold and fix the mobile hinged support rails 900.50.4...60 - 70 mm wide, 210 mm high, 18 mm deep - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply Article no.: HEWI 900.50.00160 |
| 900.50.001XA Mounting plate satin 900.50.001XA in product catalogue |  | Mounting plate for mobile hinged support rails, System 900 - rectangular mounting plate with fixing screw - used to hold and fix the mobile hinged support rails 900.50.4...XA - 70 mm wide, 210 mm high, 18 mm deep - made of stainless steel, satin - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply Article no.: HEWI 900.50.001XA |
| 900.50.00240 Wall plate cover for mounting plate chrome 900.50.00240 in product catalogue |  | Cover for mounting plate of the mobile hinged support rails, System 900 - rectangular cover - used to cover installed mounting plates by simply hooking it in - 80 mm wide, 226 mm high, 20 mm deep - cover frame made of high-quality polyamide in HEWI colour 98 (signal white) and 92 (anthracite grey) - face plate made of high-quality stainless steel, chrome Article no.: HEWI 900.50.00240 |
| 900.50.00260 Wall plate cover for mounting plate powder-coated 900.50.00260 in product catalogue |  | Cover for mounting plate of the mobile hinged support rails, System 900 - rectangular cover - used to cover installed mounting plates by simply hooking it in - 80 mm wide, 226 mm high, 20 mm deep - face plate made of high-quality stainless steel, powder-coated in the HEWI colour CV (matt black) with cover frame in matt black polyamide or powder-coated in the HEWI colour AS (matt white) with cover frame in matt white polyamide Article no.: HEWI 900.50.00260 |
| 900.50.002XA Wall plate cover for mounting plate satin 900.50.002XA in product catalogue |  | Cover for mounting plate of the mobile hinged support rails, System 900 - rectangular cover - used to cover installed mounting plates by simply hooking it in - 80 mm wide, 226 mm high, 20 mm deep - cover frame made of high-quality polyamide in HEWI colour 98 (signal white) and 92 (anthracite grey) - face plate made of stainless steel, satin Article no.: HEWI 900.50.002XA |
| 900.50.00340 Mounting plate with cover chrome 900.50.00340 in product catalogue |  | Mounting plate with cover of the mobile hinged support rails, System 900 - rectangular mounting plate with matching, rectangular cover - used to hold the mobile hinged support rails 900.50.4...40 - 80 mm wide, 226 mm high, 20 mm deep - mounting plate made of stainless steel, chrome - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - cover frame made of high-quality polyamide in HEWI colours 98 (signal white) and 92 (anthracite grey) - face plate of the cover made of high-quality stainless steel, chrome Article no.: HEWI 900.50.00340 |
| 900.50.00360 Mounting plate with cover powder-coated 900.50.00360 in product catalogue |  | Mounting plate with cover of the mobile hinged support rails, System 900 - rectangular mounting plate with matching, rectangular cover - used to hold the mobile hinged support rails 900.50.4...60 - 80 mm wide, 226 mm high, 20 mm deep - mounting plate made of stainless steel, powder-coated in the HEWI colour CV (matt black) and ER (matt dark grey pearl mica) with cover frame in matt black polyamide or powder-coated in the HEWI colours AS (matt white) and AX (matt light grey pearl mica) with cover frame in matt white polyamide - face plate of the cover made of high-quality stainless steel, colour matching to frame, powder-coated in the HEWI colours AS (matt white) and CV (matt black) - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply Article no.: HEWI 900.50.00360 |
| 900.50.003XA Mounting plate with cover satin 900.50.003XA in product catalogue |  | Mounting plate with cover of the mobile hinged support rails, System 900 - rectangular mounting plate with matching, rectangular cover - used to hold the mobile hinged support rails 900.50.4...XA - 80 mm wide, 226 mm high, 20 mm deep - mounting plate made of stainless steel, satin - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - cover frame made of high-quality polyamide in HEWI colours 98 (signal white) and 92 (anthracite grey) - face plate of the cover made of stainless steel, satin Article no.: HEWI 900.50.003XA |
| 900.50.10040 Hinged support rail Mono 600 mm projection chrome 900.50.10040 in product catalogue |  | Mono hinged support rail, design (A), System 900 - with one horizontal, round gripping level - can be folded upwards and folded downwards with braked movement - 600 mm projection, 210 mm high and 101 mm wide, 33.7 mm rail diameter - weight capacity up to 100 kg - made of stainless steel, chrome - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040 requirements Article no.: HEWI 900.50.10040 |
| 900.50.10060 Hinged support rail Mono 600 mm powder-coated 900.50.10060 in product catalogue |  | Mono hinged support rail, design (A), System 900 - with one horizontal, round gripping level - can be folded upwards and folded downwards with braked movement - 600 mm projection, 210 mm high and 101 mm wide, 33.7 mm rail diameter - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040 requirements Article no.: HEWI 900.50.10060 |
| 900.50.100XA Hinged support rail Mono 600 mm projection satin 900.50.100XA in product catalogue |  | Mono hinged support rail, design (A), System 900 - with one horizontal, round gripping level - can be folded upwards and folded downwards with braked movement - 600 mm projection, 210 mm high and 101 mm wide, 33.7 mm rail diameter - weight capacity up to 100 kg - made of stainless steel, satin - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040 requirements Article no.: HEWI 900.50.100XA |
| 900.50.10140 Hinged support rail Duo 600 mm projection chrome 900.50.10140 in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 600 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040 requirements Article no.: HEWI 900.50.10140 |
| 900.50.10160 Hinged support rail Duo 600 mm powder-coated 900.50.10160 in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 600 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040 requirements Article no.: HEWI 900.50.10160 |
| 900.50.101XA Hinged support rail Duo 600 mm projection satin 900.50.101XA in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 600 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040 requirements Article no.: HEWI 900.50.101XA |
| 900.50.10240 Hinged support rail Duo 700 mm projection chrome 900.50.10240 in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 700 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.10240 |
| 900.50.10260 Hinged support rail Duo 700 mm powder-coated 900.50.10260 in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 700 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.10260 |
| 900.50.102XA Hinged support rail Duo 700 mm projection satin 900.50.102XA in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 700 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.102XA |
| 900.50.10340 Hinged support rail Duo 750 mm projection chrome 900.50.10340 in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 750 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) regarding the projection of the hinged support rail at the toilet Article no.: HEWI 900.50.10340 |
| 900.50.10360 Hinged support rail Duo 750 mm powder-coated 900.50.10360 in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 750 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) regarding the projection of the hinged support rail at the toilet Article no.: HEWI 900.50.10360 |
| 900.50.103XA Hinged support rail Duo 750 mm projection satin 900.50.103XA in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 750 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) regarding the projection of the hinged support rail at the toilet Article no.: HEWI 900.50.103XA |
| 900.50.10440 Hinged support rail Duo 850 mm projection chrome 900.50.10440 in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 850 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.10440 |
| 900.50.10460 Hinged support rail Duo 850 mm powder-coated 900.50.10460 in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 850 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.10460 |
| 900.50.104XA Hinged support rail Duo 850 mm projection satin 900.50.104XA in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 850 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.104XA |
| 900.50.10540 Hinged support rail Duo 900 mm projection chrome 900.50.10540 in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 900 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 900.50.10540 |
| 900.50.10560 Hinged support rail Duo 900 mm powder-coated 900.50.10560 in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 900 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 900.50.10560 |
| 900.50.105XA Hinged support rail Duo 900 mm projection satin 900.50.105XA in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 900 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 900.50.105XA |
| 900.50.10640 Hinged support rail Duo 700 mm projection chrome 900.50.10640 in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - projection 700 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.10640 |
| 900.50.10660 Hinged support rail Duo 700 mm powder-coated 900.50.10660 in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - projection 700 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.10660 |
| 900.50.106XA Hinged support rail Duo 700 mm projection satin 900.50.106XA in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - projection 700 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.106XA |
| 900.50.10740 Hinged support rail Duo 750 mm projection chrome 900.50.10740 in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - projection 750 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) regarding the projection of the hinged support rail at the toilet Article no.: HEWI 900.50.10740 |
| 900.50.10760 Hinged support rail Duo 750 mm powder-coated 900.50.10760 in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - projection 750 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) regarding the projection of the hinged support rail at the toilet Article no.: HEWI 900.50.10760 |
| 900.50.107XA Hinged support rail Duo 750 mm projection satin 900.50.107XA in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - projection 750 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) regarding the projection of the hinged support rail at the toilet Article no.: HEWI 900.50.107XA |
| 900.50.10840 Hinged support rail Duo 850 mm projection chrome 900.50.10840 in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - projection 850 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.10840 |
| 900.50.10860 Hinged support rail Duo 850 mm powder-coated 900.50.10860 in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - projection 850 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.10860 |
| 900.50.108XA Hinged support rail Duo 850 mm projection satin 900.50.108XA in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - projection 850 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.108XA |
| 900.50.10940 Hinged support rail Duo 900 mm projection chrome 900.50.10940 in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - projection 900 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 900.50.10940 |
| 900.50.10960 Hinged support rail Duo 900 mm powder-coated 900.50.10960 in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - projection 900 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 900.50.10960 |
| 900.50.109XA Hinged support rail Duo 900 mm projection satin 900.50.109XA in product catalogue |  | Duo hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - projection 900 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 900.50.109XA |
| 900.50.15840 Hinged support rail Duo 600 mm chrome 900.50.15840 in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 600 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - wall plate made of polyamide with integrated steel core and cover made of chrome stainless steel - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040 requirements Article no.: HEWI 900.50.15840 |
| 900.50.15860 Hinged support rail Duo 600 mm powder-coated 900.50.15860 in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 600 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - wall plate made of polyamide with integrated steel core and cover made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040 requirements Article no.: HEWI 900.50.15860 |
| 900.50.158XA Hinged support rail Duo 600 mm satin 900.50.158XA in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 600 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - wall plate made of polyamide with integrated steel core and satin stainless steel cover - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040 requirements Article no.: HEWI 900.50.158XA |
| 900.50.15940 Hinged support rail Duo 700 mm projection chrome 900.50.15940 in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 700 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - wall plate made of polyamide with integrated steel core and cover made of chrome stainless steel - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.15940 |
| 900.50.15960 Hinged support rail Duo 700 mm powder-coated 900.50.15960 in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 700 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - wall plate made of polyamide with integrated steel core and cover made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.15960 |
| 900.50.159XA Hinged support rail Duo 700 mm projection satin 900.50.159XA in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 700 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - wall plate made of polyamide with integrated steel core and satin stainless steel cover - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.159XA |
| 900.50.16040 Hinged support rail Duo 750 mm projection chrome 900.50.16040 in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 750 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - wall plate made of polyamide with integrated steel core and cover made of chrome stainless steel - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) regarding the projection of the hinged support rail at the toilet Article no.: HEWI 900.50.16040 |
| 900.50.16060 Hinged support rail Duo 750 mm powder-coated 900.50.16060 in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 750 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - wall plate made of polyamide with integrated steel core and cover made of chrome stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) regarding the projection of the hinged support rail at the toilet Article no.: HEWI 900.50.16060 |
| 900.50.160XA Hinged support rail Duo 750 mm projection satin 900.50.160XA in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 750 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - wall plate made of polyamide with integrated steel core and satin stainless steel cover - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) regarding the projection of the hinged support rail at the toilet Article no.: HEWI 900.50.160XA |
| 900.50.16140 Hinged support rail Duo 850 mm projection chrome 900.50.16140 in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 850 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - wall plate made of polyamide with integrated steel core and cover made of chrome stainless steel - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.16140 |
| 900.50.16160 Hinged support rail Duo 850 mm powder-coated 900.50.16160 in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 850 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - wall plate made of polyamide with integrated steel core and cover made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.16160 |
| 900.50.161XA Hinged support rail Duo 850 mm projection satin 900.50.161XA in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 850 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - wall plate made of polyamide with integrated steel core and satin stainless steel cover - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.161XA |
| 900.50.16240 Hinged support rail Duo 900 mm projection chrome 900.50.16240 in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 900 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - wall plate made of polyamide with integrated steel core and cover made of chrome stainless steel - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 900.50.16240 |
| 900.50.16260 Hinged support rail Duo 900 mm powder-coated 900.50.16260 in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 900 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - wall plate made of polyamide with integrated steel core and cover made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 900.50.16260 |
| 900.50.162XA Hinged support rail Duo 900 mm projection satin 900.50.162XA in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - projection 900 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - wall plate made of polyamide with integrated steel core and satin stainless steel cover - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 900.50.162XA |
| 900.50.16340 Hinged support rail Duo 700 mm projection chrome 900.50.16340 in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - projection 700 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - wall plate made of polyamide with integrated steel core and cover made of chrome stainless steel - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.16340 |
| 900.50.16360 Hinged support rail Duo 700 mm powder-coated 900.50.16360 in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - projection 700 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - wall plate made of polyamide with integrated steel core and cover made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.16360 |
| 900.50.163XA Hinged support rail Duo 700 mm projection satin 900.50.163XA in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - projection 700 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - wall plate made of polyamide with integrated steel core and satin stainless steel cover - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.163XA |
| 900.50.16440 Hinged support rail Duo 750 mm projection chrome 900.50.16440 in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - projection 750 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - wall plate made of polyamide with integrated steel core and cover made of chrome stainless steel - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) regarding the projection of the hinged support rail at the toilet Article no.: HEWI 900.50.16440 |
| 900.50.16460 Hinged support rail Duo 750 mm powder-coated 900.50.16460 in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - projection 750 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - wall plate made of polyamide with integrated steel core and cover made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) regarding the projection of the hinged support rail at the toilet Article no.: HEWI 900.50.16460 |
| 900.50.164XA Hinged support rail Duo 750 mm projection satin 900.50.164XA in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - projection 750 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - wall plate made of polyamide with integrated steel core and satin stainless steel cover - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) regarding the projection of the hinged support rail at the toilet Article no.: HEWI 900.50.164XA |
| 900.50.16540 Hinged support rail Duo 850 mm projection chrome 900.50.16540 in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - projection 850 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - wall plate made of polyamide with integrated steel core and cover made of chrome stainless steel - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.16540 |
| 900.50.16560 Hinged support rail Duo 850 mm powder-coated 900.50.16560 in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - projection 850 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - wall plate made of polyamide with integrated steel core and cover made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.16560 |
| 900.50.165XA Hinged support rail Duo 850 mm projection satin 900.50.165XA in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - projection 850 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - wall plate made of polyamide with integrated steel core and satin stainless steel cover - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.165XA |
| 900.50.16640 Hinged support rail Duo 900 mm projection chrome 900.50.16640 in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - projection 900 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - wall plate made of polyamide with integrated steel core and cover made of chrome stainless steel - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 900.50.16640 |
| 900.50.16660 Hinged support rail Duo 900 mm powder-coated 900.50.16660 in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - projection 900 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - wall plate made of polyamide with integrated steel core and cover made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 900.50.16660 |
| 900.50.166XA Hinged support rail Duo 900 mm projection satin 900.50.166XA in product catalogue |  | Hinged support rail Duo, design (B), System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - projection 900 mm, 256 mm high and 75 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - wall plate made of polyamide with integrated steel core and satin stainless steel cover - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 900.50.166XA |
| 900.50.21540 Hinged support rail Mono 600 mm projection chrome 900.50.21540 in product catalogue |  | Hinged support rail Mono, design (B), System 900 - with one horizontal, round gripping level - can be folded upwards and folded downwards with braked movement - 600 mm projection, 256 mm high and 75 mm wide, 33.7 mm rail diameter - weight capacity up to 75 kg - made of stainless steel, chrome - wall plate made of polyamide with integrated steel core and cover made of chrome stainless steel - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - CE marking according to Directive 93/42/EEC Article no.: HEWI 900.50.21540 |
| 900.50.21560 Hinged support rail Mono 600 mm powder-coated 900.50.21560 in product catalogue |  | Hinged support rail Mono, design (B), System 900 - with one horizontal, round gripping level - can be folded upwards and folded downwards with braked movement - 600 mm projection, 256 mm high and 75 mm wide, 33.7 mm rail diameter - weight capacity up to 75 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - wall plate made of polyamide with integrated steel core and cover made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - CE marking according to Directive 93/42/EEC Article no.: HEWI 900.50.21560 |
| 900.50.215XA Hinged support rail Mono 600 mm projection satin 900.50.215XA in product catalogue |  | Hinged support rail Mono, design (B), System 900 - with one horizontal, round gripping level - can be folded upwards and folded downwards with braked movement - 600 mm projection, 256 mm high and 75 mm wide, 33.7 mm rail diameter - weight capacity up to 75 kg - made of stainless steel, satin - wall plate made of polyamide with integrated steel core and satin stainless steel cover - for mounting with non-corrosive and tested HEWI fixing materials for different wall systems (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of supply - CE marking according to Directive 93/42/EEC Article no.: HEWI 900.50.215XA |
| 900.50.40040 Mobile hinged support rail Mono 600 mm projection chrome 900.50.40040 in product catalogue |  | Mobile mobile hinged grab bar Mono, design (A), System 900 - with one horizontal, round gripping level - can be folded up and curbed to fold down - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.00140, must be ordered separately - toolless and quick installation by simple attachment of the swing-up grab bar - secured by fixing screw to prevent unintentional removal - projection 600 mm, 210 mm high and 101 mm wide, bar diameter 33.7 mm - weight capacity up to 100 kg - made of stainless steel, chrome - CE marking as per directive 93/42/ EWG - fulfils the DIN 18040 requirements Article no.: HEWI 900.50.40040 |
| 900.50.40060 Mobile hinged support rail Mono 600 mm projection powder-coated 900.50.40060 in product catalogue |  | Mobile mobile hinged grab bar Mono, design (A), System 900 - with one horizontal, round gripping level - can be folded up and curbed to fold down - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.00160, must be ordered separately - toolless and quick installation by simple attachment of the swing-up grab bar - secured by fixing screw to prevent unintentional removal - projection 600 mm, 210 mm high and 101 mm wide, bar diameter 33.7 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - CE marking as per directive 93/42/ EWG - fulfils the DIN 18040 requirements Article no.: HEWI 900.50.40060 |
| 900.50.400XA Mobile hinged support rail Mono 600 mm projection satin 900.50.400XA in product catalogue |  | Mobile mobile hinged grab bar Mono, design (A), System 900 - with one horizontal, round gripping level - can be folded up and curbed to fold down - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.001XA, must be ordered separately - toolless and quick installation by simple attachment of the swing-up grab bar - secured by fixing screw to prevent unintentional removal - projection 600 mm, 210 mm high and 101 mm wide, bar diameter 33.7 mm - weight capacity up to 100 kg - made of stainless steel, satin - CE marking as per directive 93/42/ EWG - fulfils the DIN 18040 requirements Article no.: HEWI 900.50.400XA |
| 900.50.40140 Mobile hinged support rail Duo 600 mm projection chrome 900.50.40140 in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.00140, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 600 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040 requirements Article no.: HEWI 900.50.40140 |
| 900.50.40160 Mobile hinged support rail Duo 600 mm projection powder-coated 900.50.40160 in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.00160, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 600 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040 requirements Article no.: HEWI 900.50.40160 |
| 900.50.401XA Mobile hinged support rail Duo 600 mm projection satin 900.50.401XA in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.001XA, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 600 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040 requirements Article no.: HEWI 900.50.401XA |
| 900.50.40240 Mobile hinged support rail Duo 700 mm projection chrome 900.50.40240 in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.00140, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 700 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.40240 |
| 900.50.40260 Mobile hinged support rail Duo 700 mm projection powder-coated 900.50.40260 in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.00160, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 700 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.40260 |
| 900.50.402XA Mobile hinged support rail Duo 700 mm projection satin 900.50.402XA in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.001XA, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 700 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.402XA |
| 900.50.40340 Mobile hinged support rail Duo 750 mm projection chrome 900.50.40340 in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.00140, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 750 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) regarding the projection of the hinged support rail at the toilet Article no.: HEWI 900.50.40340 |
| 900.50.40360 Mobile hinged support rail Duo 750 mm projection powder-coated 900.50.40360 in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.00160, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 750 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) regarding the projection of the hinged support rail at the toilet Article no.: HEWI 900.50.40360 |
| 900.50.403XA Mobile hinged support rail Duo 750 mm projection satin 900.50.403XA in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.001XA, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 750 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) regarding the projection of the hinged support rail at the toilet Article no.: HEWI 900.50.403XA |
| 900.50.40440 Mobile hinged support rail Duo 850 mm projection chrome 900.50.40440 in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.00140, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 850 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.40440 |
| 900.50.40460 Mobile hinged support rail Duo 850 mm projection powder-coated 900.50.40460 in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.00160, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 850 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.40460 |
| 900.50.404XA Mobile hinged support rail Duo 850 mm projection satin 900.50.404XA in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.001XA, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 850 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.404XA |
| 900.50.40540 Mobile hinged support rail Duo 900 mm projection chrome 900.50.40540 in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.00140, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 900 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 900.50.40540 |
| 900.50.40560 Mobile hinged support rail Duo 900 mm projection powder-coated 900.50.40560 in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.00160, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 900 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 900.50.40560 |
| 900.50.405XA Mobile hinged support rail Duo 900 mm projection satin 900.50.405XA in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.001XA, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 900 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 900.50.405XA |
| 900.50.40640 Mobile hinged support rail Duo 700 mm projection chrome 900.50.40640 in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.00140, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 700 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.40640 |
| 900.50.40660 Mobile hinged support rail Duo 700 mm projection powder-coated 900.50.40660 in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.00160, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 700 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.40660 |
| 900.50.406XA Mobile hinged support rail Duo 700 mm projection satin 900.50.406XA in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.001XA, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 700 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.406XA |
| 900.50.40740 Mobile hinged support rail Duo 750 mm projection chrome 900.50.40740 in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.00140, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 750 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) regarding the projection of the hinged support rail at the toilet Article no.: HEWI 900.50.40740 |
| 900.50.40760 Mobile hinged support rail Duo 750 mm projection powder-coated 900.50.40760 in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.00160, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 750 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) regarding the projection of the hinged support rail at the toilet Article no.: HEWI 900.50.40760 |
| 900.50.407XA Mobile hinged support rail Duo 750 mm projection satin 900.50.407XA in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.001XA, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 750 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the requirements applicable in Belgium (CWATUPE, WVG Vlaanderen) regarding the projection of the hinged support rail at the toilet Article no.: HEWI 900.50.407XA |
| 900.50.40840 Mobile hinged support rail Duo 850 mm projection chrome 900.50.40840 in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.00140, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 850 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.40840 |
| 900.50.40860 Mobile hinged support rail Duo 850 mm projection powder-coated 900.50.40860 in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.00160, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 850 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.40860 |
| 900.50.408XA Mobile hinged support rail Duo 850 mm projection satin 900.50.408XA in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.001XA, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 850 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 900.50.408XA |
| 900.50.40940 Mobile hinged support rail Duo 900 mm projection chrome 900.50.40940 in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.00140, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 900 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, chrome - floor supports 900.50.02040 and 900.50.02140 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 900.50.40940 |
| 900.50.40960 Mobile hinged support rail Duo 900 mm projection powder-coated 900.50.40960 in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.00160, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 900 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - floor supports 900.50.02060 and 900.50.02160 for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 900.50.40960 |
| 900.50.409XA Mobile hinged support rail Duo 900 mm projection satin 900.50.409XA in product catalogue |  | Duo mobile hinged support rail, System 900 - two parallel, round gripping levels arranged on top of each other - with toilet roll holder - can be folded upwards and folded downwards with braked movement - for mobile use - for easy and flexible retrofitting on existing, preassembled mounting plate 900.50.001XA, must be ordered separately - toolless and quick installation by simple attachment of the hinged support rail - secured by fixing screw to prevent unintentional removal - projection 900 mm, 210 mm high and 101 mm wide, top rail diameter 33.7 mm, bottom rail diameter 25 mm - weight capacity up to 100 kg - made of stainless steel, satin - floor supports 900.50.020XA and 900.50.021XA for retrofitting - CE marking according to Directive 93/42/EEC - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 900.50.409XA |
| 950.50.016 Mounting plate for mobile hinged support rails 950.50.016 in product catalogue |  | Mounting plate for mobile hinged support rails, Series 805 Classic and Series 801 - rectangular mounting plate with fixing screw - serves to hold and fix the mobile hinged support rails 805.50.1..., 805.50.2..., 801.50.1..., 801.50D1..., 801.50B1..., 801.50.2..., 801.50D2... and 801.50B2... - 74 mm wide, 249 mm high, 18 mm deep - made of stainless steel, satin finish - for mounting with corrosion-free and tested HEWI fixing material for different wall structures (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of delivery Article no.: HEWI 950.50.016 |
| 950.50.01790 Wall plate cover for mounting plate 950.50.01790 in product catalogue |  | Cover for mounting plate of mobile hinged support rails, Series 801 - rectangular cover - serves to cover mounted mounting plates by means of simple hanging - 82 mm wide, 265 mm high, 19 mm deep - cover made of high-quality polyamide in the HEWI colours 98 (signal white) or 99 (pure white) Article no.: HEWI 950.50.01790 |
| 950.50.10190 Mounting plate with cover 950.50.10190 in product catalogue |  | Mounting plate with cover for the mobile hinged support rails, Series 801 - rectangular mounting plate with matching, rectangular cover - serves to hold and fix the mobile hinged support rails 801.50.1..., 801.50D1..., 801.50B1..., 801.50.2..., 801.50D2... and 801.50B2... - 82 mm wide, 265 mm high, 19 mm deep - mounting plate made of stainless steel, satin finish - for mounting with corrosion-free and tested HEWI fixing material for various wall structures (to be ordered separately) - sealing tape for sealing the fixing points is included in the scope of delivery - cover made of high-quality polyamide in the HEWI colours 98 (signal white) or 99 (pure white) Article no.: HEWI 950.50.10190 |
| BM16.2US Fixing material (USA) BM16.2US in product catalogue |  | Fixing type - for hinged support rails Range 801(USA) for fixing on cavity walls with wood lining Consisting of: - 2 stainless steel screws - 2 screwin nuts (rampa sleeves) Article no.: HEWI BM16.2US |
| Support => Back rest rails |  |  |
| 900.51.90040 Backrest chrome 900.51.90040 in product catalogue |  | Backrest rail, System 900 - backrest with fixing rails with roses on both sides - serves as back support for toilet users - for wall mounting - 330 mm high and 150 mm deep, backrest 350 mm wide, rail diameter 32 mm, tube thickness 2 mm - Centre-to-centre from 582 to 682 mm, adjustable on site - backrest made of synthetic material in HEWI colours 98 (signal white) and 92 (anthracite grey) - connection made of high-quality stainless steel, surface chrome - including non-corrosive and tested HEWI fixing material - fulfils the DIN 18040 and ÖNORM B1600/1601 requirements Article no.: HEWI 900.51.90040 |
| 900.51.90060 Backrest powder-coated 900.51.90060 in product catalogue |  | Backrest rail, System 900 - backrest with fixing rails with roses on both sides - serves as back support for toilet users - for wall mounting - 330 mm high and 150 mm deep, backrest 350 mm wide, rail diameter 32 mm, tube thickness 2 mm - Centre-to-centre from 582 to 682 mm, adjustable on site - connection made of high-quality stainless steel, powder-coated in the HEWI colour CV (matt black) and ER (matt dark grey pearl mica) with a black backrest or powder-coated in the HEWI colours AS (matt white) and AX (matt dark grey pearl mica) with a white backrest - including non-corrosive and tested HEWI fixing material - fulfils the DIN 18040 and ÖNORM B1600/1601 requirements Article no.: HEWI 900.51.90060 |
| 900.51.900XA Backrest satin 900.51.900XA in product catalogue |  | Backrest rail, System 900 - backrest with fixing rails with roses on both sides - serves as back support for toilet users - for wall mounting - 330 mm high and 150 mm deep, backrest 350 mm wide, rail diameter 32 mm, tube thickness 2 mm - Centre-to-centre from 582 to 682 mm, adjustable on site - backrest made of synthetic material in HEWI colours 98 (signal white) and 92 (anthracite grey) - connection made of high-quality stainless steel, satin - including non-corrosive and tested HEWI fixing material - fulfils the DIN 18040 and ÖNORM B1600/1601 requirements Article no.: HEWI 900.51.900XA |
| 900.51.90140 Backrest chrome 900.51.90140 in product catalogue |  | Backrest rail, System 900 - backrest with fixing rails on both sides - serves as back support for toilet users - mounting onto hinged support rails, design (A) from the System 900 - 330 mm high and 150 mm deep, backrest 350 mm wide, rail diameter 32 mm, tube thickness 2 mm - centre-to-centre from 588 to 688 mm (clear dimension between the rails of the hinged support rails 650-750 mm) adjustable on site - backrest made of synthetic material in HEWI colours 98 (signal white) and 92 (anthracite grey) - connection made of high-quality stainless steel, surface chrome - including non-corrosive and tested HEWI fixing material - fulfils the DIN 18040 and ÖNORM B1600/1601 requirements Article no.: HEWI 900.51.90140 |
| 900.51.90160 Backrest powder-coated 900.51.90160 in product catalogue |  | Backrest rail, System 900 - backrest with fixing rails on both sides - serves as back support for toilet users - mounting onto hinged support rails, design (A) from the System 900 - 330 mm high and 150 mm deep, backrest 350 mm wide, rail diameter 32 mm, tube thickness 2 mm - centre-to-centre from 588 to 688 mm (clear dimension between the rails of the hinged support rails 650-750 mm) adjustable on site - connection made of high-quality stainless steel, powder-coated in the HEWI colour CV (matt black) and ER (matt dark grey pearl mica) with a black backrest or powder-coated in the HEWI colours AS (matt white) and AX (matt dark grey pearl mica) with a white backrest - including non-corrosive and tested HEWI fixing material - fulfils the DIN 18040 and ÖNORM B1600/1601 requirements Article no.: HEWI 900.51.90160 |
| 900.51.901XA Backrest satin 900.51.901XA in product catalogue |  | Backrest rail, System 900 - backrest with fixing rails on both sides - serves as back support for toilet users - mounting onto hinged support rails, design (A) from the System 900 - 330 mm high and 150 mm deep, backrest 350 mm wide, rail diameter 32 mm, tube thickness 2 mm - centre-to-centre from 588 to 688 mm (clear dimension between the rails of the hinged support rails 650-750 mm) adjustable on site - backrest made of synthetic material in HEWI colours 98 (signal white) and 92 (anthracite grey) - connection made of high-quality stainless steel, satin - including non-corrosive and tested HEWI fixing material - fulfils the DIN 18040 and ÖNORM B1600/1601 requirements Article no.: HEWI 900.51.901XA |
| 900.51.90240 Backrest chrome 900.51.90240 in product catalogue |  | Backrest rail, System 900 - backrest with fixing rails on both sides - serves as back support for toilet users - one-sided installation on the wall with roses (right) - one-sided fitting onto hinged support rails, design (A) from the System (left) - 330 mm high and 150 mm deep, backrest 350 mm wide, rail diameter 32 mm, tube thickness 2 mm - centre-to-centre adjustable (clear dimension between the spar of the hinged support rail and the centre-to-centre of the wall fixing 616-716 mm) - backrest made of synthetic material in HEWI colours 98 (signal white) and 92 (anthracite grey) - connection made of high-quality stainless steel, surface chrome - including non-corrosive and tested HEWI fixing material - fulfils the DIN 18040 and ÖNORM B1600/1601 requirements Article no.: HEWI 900.51.90240 |
| 900.51.90260 Backrest powder-coated 900.51.90260 in product catalogue |  | Backrest rail, System 900 - backrest with fixing rails on both sides - serves as back support for toilet users - one-sided installation on the wall with roses (right) - one-sided fitting onto hinged support rails, design (A) from the System (left) - 330 mm high and 150 mm deep, backrest 350 mm wide, rail diameter 32 mm, tube thickness 2 mm - centre-to-centre adjustable (clear dimension between the spar of the hinged support rail and the centre-to-centre of the wall fixing 616-716 mm) - connection made of high-quality stainless steel, powder-coated in the HEWI colour CV (matt black) and ER (matt dark grey pearl mica) with a black backrest or powder-coated in the HEWI colours AS (matt white) and AX (matt dark grey pearl mica) with a white backrest - including non-corrosive and tested HEWI fixing material - fulfils the DIN 18040 and ÖNORM B1600/1601 requirements Article no.: HEWI 900.51.90260 |
| 900.51.902XA Backrest satin 900.51.902XA in product catalogue |  | Backrest rail, System 900 - backrest with fixing rails on both sides - serves as back support for toilet users - one-sided installation on the wall with roses (right) - one-sided fitting onto hinged support rails, design (A) from the System (left) - 330 mm high and 150 mm deep, backrest 350 mm wide, rail diameter 32 mm, tube thickness 2 mm - centre-to-centre adjustable (clear dimension between the spar of the hinged support rail and the centre-to-centre of the wall fixing 616-716 mm) - backrest made of synthetic material in HEWI colours 98 (signal white) and 92 (anthracite grey) - connection made of high-quality stainless steel, satin - including non-corrosive and tested HEWI fixing material - fulfils the DIN 18040 and ÖNORM B1600/1601 requirements Article no.: HEWI 900.51.902XA |
| 900.51.90340 Backrest chrome 900.51.90340 in product catalogue |  | Backrest rail, System 900 - backrest with fixing rails on both sides - serves as back support for toilet users - one-sided installation on the wall with roses (left) - one-sided fitting onto hinged support rails, design (A) from the System (right) - 330 mm high and 150 mm deep, backrest 350 mm wide, rail diameter 32 mm, tube thickness 2 mm - centre-to-centre adjustable (clear dimension between the spar of the hinged support rail and the centre-to-centre of the wall fixing 616-716 mm) - backrest made of synthetic material in HEWI colours 98 (signal white) and 92 (anthracite grey) - connection made of high-quality stainless steel, surface chrome - including non-corrosive and tested HEWI fixing material - fulfils the DIN 18040 and ÖNORM B1600/1601 requirements Article no.: HEWI 900.51.90340 |
| 900.51.90360 Backrest powder-coated 900.51.90360 in product catalogue |  | Backrest rail, System 900 - backrest with fixing rails on both sides - serves as back support for toilet users - one-sided installation on the wall with roses (left) - one-sided fitting onto hinged support rails, design (A) from the System (right) - 330 mm high and 150 mm deep, backrest 350 mm wide, rail diameter 32 mm, tube thickness 2 mm - centre-to-centre adjustable (clear dimension between the spar of the hinged support rail and the centre-to-centre of the wall fixing 616-716 mm) - connection made of high-quality stainless steel, powder-coated in the HEWI colour CV (matt black) and ER (matt dark grey pearl mica) with a black backrest or powder-coated in the HEWI colours AS (matt white) and AX (matt dark grey pearl mica) with a white backrest - including non-corrosive and tested HEWI fixing material - fulfils the DIN 18040 and ÖNORM B1600/1601 requirements Article no.: HEWI 900.51.90360 |
| 900.51.903XA Backrest satin 900.51.903XA in product catalogue |  | Backrest rail, System 900 - backrest with fixing rails on both sides - serves as back support for toilet users - one-sided installation on the wall with roses (left) - one-sided fitting onto hinged support rails, design (A) from the System (right) - 330 mm high and 150 mm deep, backrest 350 mm wide, rail diameter 32 mm, tube thickness 2 mm - centre-to-centre adjustable (clear dimension between the spar of the hinged support rail and the centre-to-centre of the wall fixing 616-716 mm) - backrest made of synthetic material in HEWI colours 98 (signal white) and 92 (anthracite grey) - connection made of high-quality stainless steel, satin - including non-corrosive and tested HEWI fixing material - fulfils the DIN 18040 and ÖNORM B1600/1601 requirements Article no.: HEWI 900.51.903XA |
| 900.51.90440 Backrest chrome 900.51.90440 in product catalogue |  | Backrest rail, System 900 - backrest with fixing rails with roses on both sides - serves as back support for toilet users - for wall mounting next to the hinged support rails, design (B) from System 900 - for wall mounting - 330 mm high and 150 mm deep, backrest 350 mm wide, rail diameter 32 mm, tube thickness 2 mm - centre-to-centre from 584 to 684 mm (clear dimension between the rails of the hinged support rails 650-750 mm) adjustable on site - backrest made of synthetic material in HEWI colours 98 (signal white) and 92 (anthracite grey) - connection made of high-quality stainless steel, surface chrome - including non-corrosive and tested HEWI fixing material - fulfils the DIN 18040 and ÖNORM B1600/1601 requirements Article no.: HEWI 900.51.90440 |
| 900.51.90460 Backrest powder-coated 900.51.90460 in product catalogue |  | Backrest rail, System 900 - backrest with fixing rails with roses on both sides - serves as back support for toilet users - for wall mounting next to the hinged support rails, design (B) from System 900 - for wall mounting - 330 mm high and 150 mm deep, backrest 350 mm wide, rail diameter 32 mm, tube thickness 2 mm - centre-to-centre from 584 to 684 mm (clear dimension between the rails of the hinged support rails 650-750 mm) adjustable on site - connection made of high-quality stainless steel, powder-coated in the HEWI colour CV (matt black) and ER (matt dark grey pearl mica) with a black backrest or powder-coated in the HEWI colours AS (matt white) and AX (matt dark grey pearl mica) with a white backrest - including non-corrosive and tested HEWI fixing material - fulfils the DIN 18040 and ÖNORM B1600/1601 requirements Article no.: HEWI 900.51.90460 |
| 900.51.904XA Backrest satin 900.51.904XA in product catalogue |  | Backrest rail, System 900 - backrest with fixing rails with roses on both sides - serves as back support for toilet users - for wall mounting next to the hinged support rails, design (B) from System 900 - for wall mounting - 330 mm high and 150 mm deep, backrest 350 mm wide, rail diameter 32 mm, tube thickness 2 mm - centre-to-centre from 584 to 684 mm (clear dimension between the rails of the hinged support rails 650-750 mm) adjustable on site - backrest made of synthetic material in HEWI colours 98 (signal white) and 92 (anthracite grey) - connection made of high-quality stainless steel, satin - including non-corrosive and tested HEWI fixing material - fulfils the DIN 18040 and ÖNORM B1600/1601 requirements Article no.: HEWI 900.51.904XA |
| Support => Upgrade kits |  |  |
| 801.50US010 Upgrade kit toilet roll holder (USA) 801.50US010 in product catalogue |  | Range 801 Upgrade kit toilet roll holder (USA) - right angled add-on toilet roll holder - for easy mounting and for equip on HEWI rail systems ø 1 5/16" (33 mm) and on swing-up grab bars Range 801 - with integrated roll brake for commercial toilet paper rolls - 1 3/8 inch (35 mm) wide, 5 3/16 inch (131 mm) high, 6 1/4 inch (158 mm) deep - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) Article no.: HEWI 801.50US010 |
| 900.50.02040 Floor support for retrofitting on hinged support rails Duo 900.50.... chrome 900.50.02040 in product catalogue |  | Floor support - vertical floor support for retrofitting on hinged support rails and stationary support rails Duo 900.50... - mounting height 760 mm (to fulfil the DIN 18040 height requirements) - support made of high-quality stainless steel, chrome - vertical length 645 mm - height adjustment within +/- 15 mm range possible - CE marking according to Directive 93/42 / EEC Article no.: HEWI 900.50.02040 |
| 900.50.02060 Floor support powder-coated 900.50.02060 in product catalogue |  | Floor support - vertical floor support for retrofitting on hinged support rails and stationary support rails Duo 900.50... - mounting height 760 mm (to fulfil the DIN 18040 height requirements) - support made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - vertical length 645 mm - height adjustment within +/- 15 mm range possible - CE marking according to Directive 93/42 / EEC Article no.: HEWI 900.50.02060 |
| 900.50.020XA Floor support for retrofitting on hinged support rails Duo 900.50.... satin 900.50.020XA in product catalogue |  | Floor support - vertical floor support for retrofitting on hinged support rails Duo 900.50... - mounting height 760 mm (to fulfil the DIN 18040 height requirements) - support made of high-quality stainless steel with satin - vertical length 645 mm - height adjustment within +/- 15 mm range possible - CE marking according to Directive 93/42 / EEC Article no.: HEWI 900.50.020XA |
| Grip |  |  |
| Grip => Grab bars |  |  |
| 801.36BUS100 Grab bar (ADA) 12" (305 mm) 801.36BUS100 in product catalogue |  | Grab bar (USA) from Range 801 - rail curved at right-angles at the ends with steel fixing roses - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 12 inch (305 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36BUS100 |
| 801.36BUS110 Grab bar (ADA) 16" (406 mm) 801.36BUS110 in product catalogue |  | Grab bar (USA) from Range 801 - rail curved at right-angles at the ends with steel fixing roses - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 16 inch (406 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36BUS110 |
| 801.36BUS120 Grab bar (ADA) 18" (457 mm) 801.36BUS120 in product catalogue |  | Grab bar (USA) from Range 801 - rail curved at right-angles at the ends with steel fixing roses - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 18 inch (457 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36BUS120 |
| 801.36BUS130 Grab bar (ADA) 24" (610 mm) 801.36BUS130 in product catalogue |  | Grab bar (USA) from Range 801 - rail curved at right-angles at the ends with steel fixing roses - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 24 inch (610 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - suitable for removable hanging seats 801.51US100 and 801.51US110 - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36BUS130 |
| 801.36BUS140 Grab bar (ADA) 30" (762 mm) 801.36BUS140 in product catalogue |  | Grab bar (USA) from Range 801 - rail curved at right-angles at the ends with steel fixing roses - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 30 inch (762 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36BUS140 |
| 801.36BUS150 Grab bar (ADA) 36" (914 mm) 801.36BUS150 in product catalogue |  | Grab bar (USA) from Range 801 - rail curved at right-angles at the ends with steel fixing roses - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 36 inch (914 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 250 lbs (114 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36BUS150 |
| 801.36BUS160 Grab bar (ADA) 42" (1067 mm) 801.36BUS160 in product catalogue |  | Grab bar (USA) from Range 801 - rail curved at right-angles at the ends with steel fixing roses - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 42 inch (1067 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 250 lbs (114 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36BUS160 |
| 801.36BUS200 Grab bar (ADA) Matt edition 801.36BUS200 in product catalogue |  | Grab bar with centrally positioned third support (USA) from Range 801 - rail curved at right-angles at the ends with steel fixing roses and centrally positioned third support - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 36 inch (914 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - suitable for removable hanging seat 801.51US100 - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36BUS200 |
| 801.36BUS210 Grab bar (ADA) Matt edition 801.36BUS210 in product catalogue |  | Grab bar with centrally positioned third support (USA) from Range 801 - rail curved at right-angles at the ends with steel fixing roses and centrally positioned third support - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 42 inch (1067 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36BUS210 |
| 801.36BUS220 Grab bar (ADA) Matt edition 801.36BUS220 in product catalogue |  | Grab bar with centrally positioned third support (USA) from Range 801 - rail curved at right-angles at the ends with steel fixing roses and centrally positioned third support - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 48 inch (1219 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36BUS220 |
| 801.36BUS300 Grab bar (ADA) Matt edition 801.36BUS300 in product catalogue |  | Grab bar for toilet cabins (USA) from Range 801 - rail with corrosion-protected steel core throughout and steel fixing roses - one end has a right-angle curve, the other end is straight - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 48 inch (1219 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36BUS300 |
| 801.36BUS310 Grab bar (ADA) Matt edition 801.36BUS310 in product catalogue |  | Grab bar for toilet cabins (USA) from Range 801 - rail with corrosion-protected steel core throughout and steel fixing roses - one end has a right-angle curve, the other end is straight - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 52 inch (1321 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36BUS310 |
| 801.36BUS320 Grab bar (ADA) Matt edition 801.36BUS320 in product catalogue |  | Grab bar for toilet cabins (USA) from Range 801 - rail with corrosion-protected steel core throughout and steel fixing roses - one end has a right-angle curve, the other end is straight - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 54 inch (1372 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36BUS320 |
| 801.36US100 Grab bar (ADA) 12" (305 mm) 801.36US100 in product catalogue |  | Grab bar (USA) from Range 801 - rail curved at right-angles at the ends with steel fixing roses - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 12 inch (305 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36US100 |
| 801.36US110 Grab bar (ADA) 16" (406 mm) 801.36US110 in product catalogue |  | Grab bar (USA) from Range 801 - rail curved at right-angles at the ends with steel fixing roses - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 16 inch (406 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36US110 |
| 801.36US120 Grab bar (ADA) 18" (457 mm) 801.36US120 in product catalogue |  | Grab bar (USA) from Range 801 - rail curved at right-angles at the ends with steel fixing roses - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 18 inch (457 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36US120 |
| 801.36US1250 Grab bar (ADA) 18" (457 mm) 801.36US1250 in product catalogue |  | Range 801 grab bar (USA) - Made of high-quality chrome-look coated polyamide with warm touch - Bar curved up at right-angles at the ends with steel fixing roses - For hold and support - With continuous, corrosion protected steel core - Wall mounting with wall-specific fixing material and HEWI roses - Fixing spacing 18 inch (457 mm), 2 3/4 inch (70 mm) deep - Bar diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - Maximum load capacity 330 lbs (150 kg) - Fulfils the requirements according to 2010 ADA Standards and ICC/ANSI A117.1-2017 - Toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36US1250 |
| 801.36US130 Grab bar (ADA) 24" (610 mm) 801.36US130 in product catalogue |  | Grab bar (USA) from Range 801 - rail curved at right-angles at the ends with steel fixing roses - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 24 inch (610 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - suitable for removable hanging seats 801.51US100 and 801.51US110 - maximum loading capacity 330 lbs (150 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36US130 |
| 801.36US140 Grab bar (ADA) 30" (762 mm) 801.36US140 in product catalogue |  | Grab bar (USA) from Range 801 - rail curved at right-angles at the ends with steel fixing roses - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 30 inch (762 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - suitable for removable hanging seats 801.51US100 and 801.51US110 - maximum loading capacity 330 lbs (150 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36US140 |
| 801.36US150 Grab bar (ADA) 36" (914 mm) 801.36US150 in product catalogue |  | Grab bar (USA) from Range 801 - rail curved at right-angles at the ends with steel fixing roses - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 36 inch (914 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 250 lbs (114 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36US150 |
| 801.36US1550 Grab bar (ADA) 36" (914 mm) 801.36US1550 in product catalogue |  | Range 801 grab bar (USA) - Made of high-quality chrome-look coated polyamide with warm touch - Bar curved up at right-angles at the ends with steel fixing roses - For hold and support - With continuous, corrosion protected steel core - Wall mounting with wall-specific fixing material and HEWI roses - Fixing spacing 36 inch (914 mm), 2 3/4 inch (70 mm) deep - Bar diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - Maximum load capacity 250 lbs (114 kg) - Fulfils the requirements according to 2010 ADA Standards and ICC/ANSI A117.1-2017 - Toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36US1550 |
| 801.36US160 Grab bar (ADA) 42" (1067 mm) 801.36US160 in product catalogue |  | Grab bar (USA) from Range 801 - rail curved at right-angles at the ends with steel fixing roses - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 42 inch (1067 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 250 lbs (114 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36US160 |
| 801.36US1650 Grab bar (ADA) 42" (1067 mm) 801.36US1650 in product catalogue |  | Range 801 grab bar (USA) - Made of high-quality chrome-look coated polyamide with warm touch - Bar curved up at right-angles at the ends with steel fixing roses - For hold and support - With continuous, corrosion protected steel core - Wall mounting with wall-specific fixing material and HEWI roses - Fixing spacing 42 inch (1067 mm), 2 3/4 inch (70 mm) deep - Bar diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - Maximum load capacity 250 lbs (114 kg) - Fulfils the requirements according to 2010 ADA Standards and ICC/ANSI A117.1-2017 - Toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36US1650 |
|  |  | Support rail (USA) from Range 801 - rail with right-angled bends at the ends and steel fixing roses - for holding on to and providing support - with continuous, corrosion resistant steel core - for wall mounting with special HEWI fixing materials and roses - centre-to-centre made-to-measure from 8 11/16 inch (220 mm) to 63 inch (1600 mm), 2 3/4 inch (70 mm) deep - from 34 3/4 inch (851 mm) with additional, centrally positioned support - bar diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made of high-quality polyamide in HEWI colours 99 (pure white), 95 (stone grey), 92 (anthracite grey), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36US1S |
| 801.36US200 Grab bar (ADA) 36" (914 mm) 801.36US200 in product catalogue |  | Grab bar with centrally positioned third support (USA) from Range 801 - rail curved at right-angles at the ends with steel fixing roses and centrally positioned third support - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 36 inch (914 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - suitable for removable hanging seat 801.51US100 - maximum loading capacity 330 lbs (150 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36US200 |
| 801.36US210 Grab bar (ADA) 42" (1067 mm) 801.36US210 in product catalogue |  | Grab bar with centrally positioned third support (USA) from Range 801 - rail curved at right-angles at the ends with steel fixing roses and centrally positioned third support - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 42 inch (1067 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - suitable for removable hanging seats 801.51US100 and 801.51US110 - maximum loading capacity 330 lbs (150 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36US210 |
| 801.36US220 Grab bar (ADA) 48" (1219 mm) 801.36US220 in product catalogue |  | Grab bar with centrally positioned third support (USA) from Range 801 - rail curved at right-angles at the ends with steel fixing roses and centrally positioned third support - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 48 inch (1219 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - suitable for removable hanging seats 801.51US100 and 801.51US110 - maximum loading capacity 330 lbs (150 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36US220 |
| 801.36US300 Grab bar (ADA) 48" (1219 mm) 801.36US300 in product catalogue |  | Grab bar for toilet cabins (USA) from Range 801 - rail with corrosion-protected steel core throughout and steel fixing roses - one end has a right-angle curve, the other end is straight - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 48 inch (1219 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36US300 |
| 801.36US310 Grab bar (ADA) 52" (1321 mm) 801.36US310 in product catalogue |  | Grab bar for toilet cabins (USA) from Range 801 - rail with corrosion-protected steel core throughout and steel fixing roses - one end has a right-angle curve, the other end is straight - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 52 inch (1321 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36US310 |
| 801.36US320 Grab bar (ADA) 54" (1372 mm) 801.36US320 in product catalogue |  | Grab bar for toilet cabins (USA) from Range 801 - rail with corrosion-protected steel core throughout and steel fixing roses - one end has a right-angle curve, the other end is straight - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 54 inch (1372 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36US320 |
|  |  | Support rail with centrally positioned third support (USA) from Range 801 - rail with right-angled bends at the ends with steel fixing roses and centrally positioned third support - for holding on to and providing support - with continuous, corrosion resistant steel core - for wall mounting with special HEWI fixing materials and roses - centre-to-centre made-to-measure from 48 1/16 inch (1220 mm) to 63 inch (1600 mm), 2 3/4 inch (70 mm) deep - bar diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - suitable for removable hanging seat 801.51US100 - maximum loading capacity 330 lbs (150 kg) - made of high-quality polyamide in HEWI colours 99 (pure white), 95 (stone grey), 92 (anthracite grey), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 - toilet paper holder upgrade kit 801.50US010 can be retrofitted Article no.: HEWI 801.36US3S |
| 900.36.06040 Support rail 12" (305 mm) chrome 900.36.06040 in product catalogue |  | Grab bar, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 12 inch (305 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of stainless steel, chrome - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.06040 |
| 900.36.06060 Support rail 12" (305 mm) powder-coated 900.36.06060 in product catalogue |  | Grab bar, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 12 inch (305 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.06060 |
| 900.36.060XA Support rail 12" (305 mm) satin 900.36.060XA in product catalogue |  | Grab bar, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 12 inch (305 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of stainless steel, satin - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.060XA |
| 900.36.06140 Support rail 16" (406 mm) chrome 900.36.06140 in product catalogue |  | Grab bar, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 16 inch (406 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of stainless steel, chrome - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.06140 |
| 900.36.06160 Support rail 16" (406 mm) powder-coated 900.36.06160 in product catalogue |  | Grab bar, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 16 inch (406 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.06160 |
| 900.36.061XA Support rail 16" (406 mm) satin 900.36.061XA in product catalogue |  | Grab bar, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 16 inch (406 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of stainless steel, satin - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.061XA |
| 900.36.06240 Support rail 18" (457 mm) chrome 900.36.06240 in product catalogue |  | Grab bar, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 18 inch (457 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of stainless steel, chrome - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.06240 |
| 900.36.06260 Support rail 18" (457 mm) powder-coated 900.36.06260 in product catalogue |  | Grab bar, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 18 inch (457 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.06260 |
| 900.36.062XA Support rail 18" (457 mm) satin 900.36.062XA in product catalogue |  | Grab bar, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 18 inch (457 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of stainless steel, satin - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.062XA |
| 900.36.06340 Support rail 24" (610 mm) chrome 900.36.06340 in product catalogue |  | Grab bar, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 24 inch (610 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of stainless steel, chrome - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.06340 |
| 900.36.06360 Support rail 24" (610 mm) powder-coated 900.36.06360 in product catalogue |  | Grab bar, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 24 inch (610 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.06360 |
| 900.36.063XA Support rail 24" (610 mm) satin 900.36.063XA in product catalogue |  | Grab bar, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 24 inch (610 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of stainless steel, satin - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.063XA |
| 900.36.06440 Support rail 30" (762 mm) chrome 900.36.06440 in product catalogue |  | Grab bar, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 30 inch (765 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of stainless steel, chrome - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.06440 |
| 900.36.06460 Support rail 30" (762 mm) powder-coated 900.36.06460 in product catalogue |  | Grab bar, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 30 inch (765 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.06460 |
| 900.36.064XA Support rail 30" (762 mm) satin 900.36.064XA in product catalogue |  | Grab bar, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 30 inch (765 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of stainless steel, satin - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.064XA |
| 900.36.06540 Support rail 36" (914 mm) chrome 900.36.06540 in product catalogue |  | Grab bar, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 36 inch (914 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 250 lbs (114 kg) - made of stainless steel, chrome - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.06540 |
| 900.36.06560 Support rail 36" (914 mm) powder-coated 900.36.06560 in product catalogue |  | Grab bar, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 36 inch (914 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 250 lbs (114 kg) - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.06560 |
| 900.36.065XA Support rail 36" (914 mm) satin 900.36.065XA in product catalogue |  | Grab bar, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 36 inch (914 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 250 lbs (114 kg) - made of stainless steel, satin - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.065XA |
| 900.36.06640 Support rail 42" (1067 mm) chrome 900.36.06640 in product catalogue |  | Grab bar, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 42 inch (1067 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 250 lbs (114 kg) - made of stainless steel, chrome - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.06640 |
| 900.36.06660 Support rail 42" (1067 mm) powder-coated 900.36.06660 in product catalogue |  | Grab bar, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 42 inch (1067 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 250 lbs (114 kg) - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.06660 |
| 900.36.066XA Support rail 42" (1067 mm) satin 900.36.066XA in product catalogue |  | Grab bar, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 42 inch (1067 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 250 lbs (114 kg) - made of stainless steel, satin - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.066XA |
| 900.36.06740 Support rail 48" (1219 mm) chrome 900.36.06740 in product catalogue |  | Grab bar with additional support positioned in the middle, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 48 inch (1219 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of stainless steel, chrome - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.06740 |
| 900.36.06760 Support rail 48" (1219 mm) powder-coated 900.36.06760 in product catalogue |  | Grab bar with additional support positioned in the middle, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 48 inch (1219 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.06760 |
| 900.36.067XA Support rail 48" (1219 mm) satin 900.36.067XA in product catalogue |  | Grab bar with additional support positioned in the middle, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 48 inch (1219 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of stainless steel, satin - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.067XA |
| 900.36.06840 Support rail 52" (1321 mm) chrome 900.36.06840 in product catalogue |  | Grab bar with additional support positioned in the middle, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 52 inch (1321 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of stainless steel, chrome - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.06840 |
| 900.36.06860 Support rail 52" (1321 mm) powder-coated 900.36.06860 in product catalogue |  | Grab bar with additional support positioned in the middle, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 52 inch (1321 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.06860 |
| 900.36.068XA Support rail 52" (1321 mm) satin 900.36.068XA in product catalogue |  | Grab bar with additional support positioned in the middle, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 52 inch (1321 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of stainless steel, satin - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.068XA |
| 900.36.06940 Support rail 54" (1372 mm) chrome 900.36.06940 in product catalogue |  | Grab bar with additional support positioned in the middle, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 54 inch (1372 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of stainless steel, chrome - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.06940 |
| 900.36.06960 Support rail 54" (1372 mm) powder-coated 900.36.06960 in product catalogue |  | Grab bar with additional support positioned in the middle, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 54 inch (1372 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.06960 |
| 900.36.069XA Support rail 54" (1372 mm) satin 900.36.069XA in product catalogue |  | Grab bar with additional support positioned in the middle, System 900 - straight rail with supports straight to the wall and roses - fixing spacing 54 inch (1372 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of stainless steel, satin - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.36.069XA |
|  |  | Grab bar, System 900 - straight rail with supports straight to the wall and roses - fixing spacing made-to-measure from 8 11/16 inch (220 mm) to 63 inch (1100 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - from 34 3/4 inch (851 mm) with additional, centrally positioned support - maximum loading capacity 330 lbs (150 kg) - made of high-quality stainless steel, satin - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 Article no.: HEWI 900.36.0S2XA |
| 950.36US1190 Support rail (USA) 18" (457 mm) 950.36US1190 in product catalogue |  | Grab bar (USA) from System 800 K - straight rail with supports straight to the wall and roses - for holding onto and as a support - with corrosion-protected steel core throughout - length 18 inch (457 mm), rail diameter of rail 1 5/16 inch (33 mm), 2 13/16 inch (71 mm) deep - wall connection with discreet supports perpendicular to wall and flat roses - easy installation due to individually mountable fixing plates made of high-quality stainless steel for hanging the support rail - rose diameter 3 1/8 inch (80 mm), cap 1/2 inch (13 mm) - includes non-corroding HEWI fixing material - maximum loading capacity 250 lbs (113 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 950.36US1190 |
| 950.36US1390 Support rail (USA) 24" (610 mm) 950.36US1390 in product catalogue |  | Grab bar (USA) from System 800 K - straight rail with supports straight to the wall and roses - for holding onto and as a support - with corrosion-protected steel core throughout - length 24 inch (610 mm), rail diameter of rail 1 5/16 inch (33 mm), 2 13/16 inch (71 mm) deep - wall connection with discreet supports perpendicular to wall and flat roses - easy installation due to individually mountable fixing plates made of high-quality stainless steel for hanging the support rail - rose diameter 3 1/8 inch (80 mm), cap 1/2 inch (13 mm) - includes non-corroding HEWI fixing material - maximum loading capacity 250 lbs (113 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 950.36US1390 |
| 950.36US1490 Support rail (USA) 36" (914 mm) 950.36US1490 in product catalogue |  | Grab bar (USA) from System 800 K - straight rail with supports straight to the wall and roses - for holding onto and as a support - with corrosion-protected steel core throughout - length 36 inch (914 mm), rail diameter of rail 1 5/16 inch (33 mm), 2 13/16 inch (71 mm) deep - wall connection with discreet supports perpendicular to wall and flat roses - easy installation due to individually mountable fixing plates made of high-quality stainless steel for hanging the support rail - rose diameter 3 1/8 inch (80 mm), cap 1/2 inch (13 mm) - includes non-corroding HEWI fixing material - maximum loading capacity 250 lbs (113 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 950.36US1490 |
| 950.36US1590 Support rail (USA) 42" (1067 mm) 950.36US1590 in product catalogue |  | Grab bar (USA) from System 800 K - straight rail with supports straight to the wall and roses - for holding onto and as a support - with corrosion-protected steel core throughout - length 42 inch (1067 mm), rail diameter of rail 1 5/16 inch (33 mm), 2 13/16 inch (71 mm) deep - wall connection with discreet supports perpendicular to wall and flat roses - easy installation due to individually mountable fixing plates made of high-quality stainless steel for hanging the support rail - rose diameter 3 1/8 inch (80 mm), cap 1/2 inch (13 mm) - includes non-corroding HEWI fixing material - maximum loading capacity 250 lbs (113 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 950.36US1590 |
| Grip => L-shaped grab bars |  |  |
| 801.22.200 L-shaped support rail 325 x 295 x 200 mm with steel core and fixing roses made of steel 801.22.200 in product catalogue |  | Range 801 L-shaped support rail - can be mounted in left hand or right hand position - rails joined at a flat angle and fitted with fixing roses made of steel - for holding on to and providing support - with continuous, corrosion resistant steel core - for wall mounting with special HEWI fixing materials and roses - centre-to-centre dimensions 295 mm and 325 mm, 135° angle, 200 mm high, 88 mm deep - bar diameter 33 mm, rose diameter 70 mm, - increased stability through 3 mounting supports - made of high-quality polyamide according to HEWI colour chart - toilet roll holder upgrade kit 801.50.010, can be retrofitted Article no.: HEWI 801.22.200 |
| 801.22B200 L-shaped support rail 325 x 295 x 200 mm Matt edition 801.22B200 in product catalogue |  | Range 801 L-shaped support rail - can be mounted in left hand or right hand position - rails joined at a flat angle and fitted with fixing roses made of steel - for holding on to and providing support - with continuous, corrosion resistant steel core - for wall mounting with special HEWI fixing materials and roses - centre-to-centre dimensions 295 mm and 325 mm, 135° angle, 200 mm high, 90 mm deep - bar diameter 33 mm, rose diameter 70 mm, - increased stability through 3 mounting supports - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) - toilet roll holder upgrade kit 801.50B010, can be retrofitted Article no.: HEWI 801.22B200 |
| 801.22BUS100 L-shaped grab bar (ADA) Matt edition 801.22BUS100 in product catalogue |  | L-shaped support rail (USA) from Range 801 - can be mounted on the right and left side - positioned horizontally and vertically, rails connected at a right angle with steel fixing roses - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - horizontal length 12 inch (305 mm), vertical length 24 inch (610 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - toilet paper holder upgrade kit 801.50US010 can be retrofitted - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.22BUS100 |
| 801.22BUS110 L-shaped grab bar (ADA) Matt edition 801.22BUS110 in product catalogue |  | L-shaped support rail (USA) from Range 801 - can be mounted on the right and left side - positioned horizontally and vertically, rails connected at a right angle with steel fixing roses - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - horizontal length 18 inch (457 mm), vertical length 36 inch (915 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - toilet paper holder upgrade kit 801.50US010 can be retrofitted - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.22BUS110 |
| 801.22US100 L-shaped grab bar (ADA) 24" x 12" (610 mm x 305 mm) 801.22US100 in product catalogue |  | L-shaped support rail (USA) from Range 801 - can be mounted on the right and left side - positioned horizontally and vertically, rails connected at a right angle with steel fixing roses - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - horizontal length 12 inch (305 mm), vertical length 24 inch (610 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - toilet paper holder upgrade kit 801.50US010 can be retrofitted - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.22US100 |
| 801.22US110 L-shaped grab bar (ADA) 36" x 18" (915 mm x 457 mm) 801.22US110 in product catalogue |  | L-shaped support rail (USA) from Range 801 - can be mounted on the right and left side - positioned horizontally and vertically, rails connected at a right angle with steel fixing roses - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - horizontal length 18 inch (457 mm), vertical length 36 inch (915 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - toilet paper holder upgrade kit 801.50US010 can be retrofitted - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.22US110 |
| 900.22.10040 L-shaped support rail 255 x 255 mm chrome 900.22.10040 in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - can be mounted in left-hand or right-hand position - horizontal length 255 mm, vertical length 255 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, chrome - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.22.10040 |
| 900.22.10060 L-shaped support rail 255 x 255 mm powder-coated 900.22.10060 in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - can be mounted in left-hand or right-hand position - horizontal length 255 mm, vertical length 255 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.22.10060 |
| 900.22.100XA L-shaped support rail 255 x 255 mm satin 900.22.100XA in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - can be mounted in left-hand or right-hand position - horizontal length 255 mm, vertical length 255 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, satin - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.22.100XA |
| 900.22.10140 L-shaped support rail 600 x 300 mm version right chrome 900.22.10140 in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - right-hand version - horizontal length 300 mm, vertical length 600 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, chrome - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.22.10140 |
| 900.22.10160 L-shaped support rail 600 x 300 mm version right powder-coated 900.22.10160 in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - right-hand version - horizontal length 300 mm, vertical length 600 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.22.10160 |
| 900.22.101XA L-shaped support rail 600 x 300 mm version right satin 900.22.101XA in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - right-hand version - horizontal length 300 mm, vertical length 600 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, satin - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.22.101XA |
| 900.22.10240 L-shaped support rail 750 x 500 mm version right chrome 900.22.10240 in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - right-hand version - horizontal length 500 mm, vertical length 750 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, chrome - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 900.22.10240 |
| 900.22.10260 L-shaped support rail 750 x 500 mm version right powder-coated 900.22.10260 in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - right-hand version - horizontal length 500 mm, vertical length 750 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 900.22.10260 |
| 900.22.102XA L-shaped support rail 750 x 500 mm version right satin 900.22.102XA in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - right-hand version - horizontal length 500 mm, vertical length 750 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, satin - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 900.22.102XA |
| 900.22.10340 L-shaped support rail 900 x 400 mm version right chrome 900.22.10340 in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - right-hand version - horizontal length 400 mm, vertical length 900 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, chrome - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.22.10340 |
| 900.22.10360 L-shaped support rail 900 x 400 mm version right powder-coated 900.22.10360 in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - right-hand version - horizontal length 400 mm, vertical length 900 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.22.10360 |
| 900.22.103XA L-shaped support rail 900 x 400 mm version right satin 900.22.103XA in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - right-hand version - horizontal length 400 mm, vertical length 900 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, satin - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.22.103XA |
| 900.22.10440 L-shaped support rail 1000 x 500 mm version right chrome 900.22.10440 in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - right-hand version - horizontal length 500 mm, vertical length 1000 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, chrome - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the SIA 500 requirements Article no.: HEWI 900.22.10440 |
| 900.22.10460 L-shaped support rail 1000 x 500 mm version right powder-coated 900.22.10460 in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - right-hand version - horizontal length 500 mm, vertical length 1000 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the SIA 500 requirements Article no.: HEWI 900.22.10460 |
| 900.22.104XA L-shaped support rail 1000 x 500 mm version right satin 900.22.104XA in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - right-hand version - horizontal length 500 mm, vertical length 1000 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, satin - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the SIA 500 requirements Article no.: HEWI 900.22.104XA |
| 900.22.10540 L-shaped support rail 600 x 300 mm version left chrome 900.22.10540 in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - left-hand version - horizontal length 300 mm, vertical length 600 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, chrome - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.22.10540 |
| 900.22.10560 L-shaped support rail 600 x 300 mm version left powder-coated 900.22.10560 in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - left-hand version - horizontal length 300 mm, vertical length 600 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.22.10560 |
| 900.22.105XA L-shaped support rail 600 x 300 mm version left satin 900.22.105XA in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - left-hand version - horizontal length 300 mm, vertical length 600 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, satin - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.22.105XA |
| 900.22.10640 L-shaped support rail 750 x 500 mm version left chrome 900.22.10640 in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - left-hand version - horizontal length 500 mm, vertical length 750 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, chrome - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 900.22.10640 |
| 900.22.10660 L-shaped support rail 750 x 500 mm version left powder-coated 900.22.10660 in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - left-hand version - horizontal length 500 mm, vertical length 750 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 900.22.10660 |
| 900.22.106XA L-shaped support rail 750 x 500 mm version left satin 900.22.106XA in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - left-hand version - horizontal length 500 mm, vertical length 750 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, satin - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the ÖNORM B1600/1601 requirements Article no.: HEWI 900.22.106XA |
| 900.22.10740 L-shaped support rail 900 x 400 mm version left chrome 900.22.10740 in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - left-hand version - horizontal length 400 mm, vertical length 900 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, chrome - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.22.10740 |
| 900.22.10760 L-shaped support rail 900 x 400 mm version left powder-coated 900.22.10760 in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - left-hand version - horizontal length 400 mm, vertical length 900 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.22.10760 |
| 900.22.107XA L-shaped support rail 900 x 400 mm version left satin 900.22.107XA in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - left-hand version - horizontal length 400 mm, vertical length 900 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, satin - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.22.107XA |
| 900.22.10840 L-shaped support rail 1000 x 500 mm version left chrome 900.22.10840 in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - left-hand version - horizontal length 500 mm, vertical length 1000 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, chrome - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the SIA 500 requirements Article no.: HEWI 900.22.10840 |
| 900.22.10860 L-shaped support rail 1000 x 500 mm version left powder-coated 900.22.10860 in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - left-hand version - horizontal length 500 mm, vertical length 1000 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the SIA 500 requirements Article no.: HEWI 900.22.10860 |
| 900.22.108XA L-shaped support rail 1000 x 500 mm version left satin 900.22.108XA in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - left-hand version - horizontal length 500 mm, vertical length 1000 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, satin - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the SIA 500 requirements Article no.: HEWI 900.22.108XA |
| 900.22.1S0XA L-shaped support rail, special length version right satin 900.22.1S0XA in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - right-hand version - horizontal length made-to-measure from 300 mm to 850 mm, vertical length made-to-measure from 600 mm to 1100 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, satin - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.22.1S0XA |
| 900.22.1S2XA L-shaped support rail, special length version left satin 900.22.1S2XA in product catalogue |  | L-shaped support rail, System 900 - vertical and horizontal rails, joined at right-angles with mounting roses - left-hand version - horizontal length made-to-measure from 300 mm to 850 mm, vertical length made-to-measure from 600 mm to 1100 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, satin - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.22.1S2XA |
| 900.22.30040 L-shaped support rail version right chrome 900.22.30040 in product catalogue |  | L-shaped support rail, System 900 - rails joined at a flat angle with mounting roses - right-hand version - fixing spacings 620 mm and 283 mm, 135° angle, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, chrome - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.22.30040 |
| 900.22.30060 L-shaped support rail version right powder-coated 900.22.30060 in product catalogue |  | L-shaped support rail, System 900 - rails joined at a flat angle with mounting roses - right-hand version - fixing spacings 620 mm and 283 mm, 135° angle, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.22.30060 |
| 900.22.300XA L-shaped support rail version right satin 900.22.300XA in product catalogue |  | L-shaped support rail, System 900 - rails joined at a flat angle with mounting roses - right-hand version - fixing spacings 620 mm and 283 mm, 135° angle, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, satin - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.22.300XA |
| 900.22.30140 L-shaped support rail version left chrome 900.22.30140 in product catalogue |  | L-shaped support rail, System 900 - rails joined at a flat angle with mounting roses - left-hand version - fixing spacings 620 mm and 283 mm, 135° angle, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, chrome - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.22.30140 |
| 900.22.30160 L-shaped support rail version left powder-coated 900.22.30160 in product catalogue |  | L-shaped support rail, System 900 - rails joined at a flat angle with mounting roses - left-hand version - fixing spacings 620 mm and 283 mm, 135° angle, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.22.30160 |
| 900.22.301XA L-shaped support rail version left satin 900.22.301XA in product catalogue |  | L-shaped support rail, System 900 - rails joined at a flat angle with mounting roses - left-hand version - fixing spacings 620 mm and 283 mm, 135° angle, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, satin - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.22.301XA |
| Grip => Shower head rail / L-shaped support rail |  |  |
| 801.33BUS100 Grab bar with shower head holder (ADA) 24" (610 mm) 801.33BUS100 in product catalogue |  | Rail with shower head holder (USA) from Range 801 - vertical rail curved at right-angles at the ends with fixing roses and shower head holder - suitable for handheld shower heads from various manufacturers - shower head holder can be tilted continuously and its height can be adjusted by pulling or pushing a broad lever - functional area of the shower head holder in high-quality, glossy polyamide - cone-shaped insert on the shower head holder makes it easier to mount the handheld shower head - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 24 inch (610 mm) - 2 3/4 inch (70 mm) deep, rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.33BUS100 |
| 801.33BUS200 L-shaped grab bar with shower head holder (ADA) 36" x 18" (915 x 457 mm) 801.33BUS200 in product catalogue |  | L-shaped support rail with shower head holder (USA) from Range 801 - can be mounted on the right and left side - positioned horizontally and vertically, rails connected at a right angle with steel fixing roses and shower head holder - for holding onto and as a support in the shower and bathtub - suitable for handheld shower heads from various manufacturers - shower head holder can be tilted continuously and its height can be adjusted by pulling or pushing a broad lever - cone-shaped insert on the shower head holder makes it easier to mount the handheld shower head - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - horizontal length 18 inch (457 mm), vertical length 36 inch (915 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.33BUS200 |
| 801.33US100 Grab bar with shower head holder (ADA) 24" (610 mm) 801.33US100 in product catalogue |  | Rail with shower head holder (USA) from Range 801 - vertical rail curved at right-angles at the ends with fixing roses and shower head holder - suitable for handheld shower heads from various manufacturers - shower head holder can be tilted continuously and its height can be adjusted by pulling or pushing a broad lever - cone-shaped insert on the shower head holder makes it easier to mount the handheld shower head - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 24 inch (610 mm) - 2 3/4 inch (70 mm) deep, rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.33US100 |
| 801.33US200 L-shaped grab bar with shower head holder (ADA) 36" x 18" (915 x 457 mm) 801.33US200 in product catalogue |  | L-shaped support rail with shower head holder (USA) from Range 801 - can be mounted on the right and left side - positioned horizontally and vertically, rails connected at a right angle with steel fixing roses and shower head holder - for holding onto and as a support in the shower and bathtub - suitable for handheld shower heads from various manufacturers - shower head holder can be tilted continuously and its height can be adjusted by pulling or pushing a broad lever - cone-shaped insert on the shower head holder makes it easier to mount the handheld shower head - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - horizontal length 18 inch (457 mm), vertical length 36 inch (915 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.33US200 |
| 801.33USE02 Shower head holder (USA) 801.33USE02 in product catalogue |  | Shower head holder (USA) from Range 801 - for rails with shower head holder, L-shaped support rail with shower head holder and shower & bathtub hand rails from Range 801 (USA) - shower head holder with cone-shaped intake (left) for shower hose and handheld shower head - suitable for handheld shower heads from various manufacturers - shower head holder can be tilted and its height can be adjusted by pulling or pushing a lever - cone-shaped insert makes it easier to mount the handheld shower head - 4 1/4 inch (108 mm) high, 3 11/16 inch (93 mm) wide - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - with high contrast (white or black) marked shower head holder Article no.: HEWI 801.33USE02 |
| 900.33.06040 Rail with shower head holder 24" (610 mm) chrome 900.33.06040 in product catalogue |  | Shower head rail, System 900 - vertical rail with supports perpendicular to the wall, mounting and shower head holder - c to c 24 inch (610 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - load capacity up to 330 lbs (150 kg) - made of stainless steel, chrome - shower head holder made of high-quality matt polyamide in the HEWI colors 98 (signal white) and 92 (anthracite gray) - suitable for handheld shower heads from various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - cone-shaped insert on the shower head holder makes it easier to mount the handheld shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.33.06040 |
| 900.33.06060 Rail with shower head holder 24" (610 mm) powder-coated 900.33.06060 in product catalogue |  | Shower head rail, System 900 - vertical rail with supports perpendicular to the wall, mounting and shower head holder - fixing spacing 24 inch (610 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - made of high-quality stainless steel, powder-coated in the HEWI colour CV (matt black) and ER (matt dark grey pearl mica) with a showerhead holder in matt black polyamide or powder-coated in the HEWI colours AS (matt white) and AX (matt light grey pearl mica) with a showerhead holder in matt white polyamide - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017. Article no.: HEWI 900.33.06060 |
| 900.33.060XA Rail with shower head holder 24" (610 mm) satin 900.33.060XA in product catalogue |  | Shower head rail, System 900 - vertical rail with supports perpendicular to the wall, mounting and shower head holder - c to c 24 inch (610 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - load capacity up to 330 lbs (150 kg) - made of stainless steel, matt sanded - shower head holder made of high-quality matt polyamide in the HEWI colors 98 (signal white) and 92 (anthracite gray) - suitable for handheld shower heads from various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - cone-shaped insert on the shower head holder makes it easier to mount the handheld shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.33.060XA |
|  |  | Shower head rail, System 900 - vertical rail with supports perpendicular to the wall, mounting and shower head holder - fixing spacing made-to-measure from 23 5/8 inch (600 mm) to 49 3/16 inch (1250 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made of high-quality stainless steel, satin - shower head holder made of high-quality matt polyamide in HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 Article no.: HEWI 900.33.0S2XA |
| Grip => Shower grab bars |  |  |
| 801.35BUS100 Shower grab bar (ADA) Matt edition 801.35BUS100 in product catalogue |  | Shower grab rail (USA) from Range 801 - arranged horizontally, rods connected at a right angle with steel fixing roses - horizontal lengths 37 1/2 inch x 19 1/2 inch (952 x 495 mm) - 2 3/4 inch (70 mm) deep, rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - can be mounted on the right and left side - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.35BUS100 |
| 801.35BUS110 Shower grab bar (ADA) Matt edition 801.35BUS110 in product catalogue |  | Shower grab rail (USA) from Range 801 - arranged horizontally, rods connected at a right angle with steel fixing roses - horizontal lengths 25 1/2 inch x 25 1/2 inch (647 x 647 mm) - 2 3/4 inch (70 mm) deep, rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - can be mounted on the right and left side - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.35BUS110 |
| 801.35BUS120 Shower grab bar (ADA) Matt edition 801.35BUS120 in product catalogue |  | Shower grab rail (USA) from Range 801 - arranged horizontally, rods connected at a right angle with steel fixing roses - horizontal lengths 37 1/2 inch x 37 1/2 inch (952 x 952 mm) - 2 3/4 inch (70 mm) deep, rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - can be mounted on the right and left side - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.35BUS120 |
| 801.35BUS200 Shower grab bar (USA) Matt edition 801.35BUS200 in product catalogue |  | Shower grab rail (USA) from Range 801 - positioned horizontally and vertically, rails connected at a right angle with steel fixing roses - with corrosion-protected steel core throughout - vertical length 30 inch (762 mm), horizontal lengths 31 1/2 inch (800 mm) - 2 3/4 inch (70 mm) deep, rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - wall mounting with wall-specific fixing material and roses from HEWI - can be mounted on the right and left side - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.35BUS200 |
| 801.35BUS220 Shower grab bar (USA) Matt edition 801.35BUS220 in product catalogue |  | Shower grab rail with shower head holder rail (USA) from Range 801 - positioned horizontally and vertically, rails connected at a right angle with steel fixing roses and shower head holder - with corrosion-protected steel core throughout - vertical length 49 3/16 inch (1250 mm), horizontal lengths 31 1/2 inch (800 mm) - 2 3/4 inch (70 mm) deep, rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - suitable for handheld shower heads from various manufacturers - shower head holder can be tilted continuously and its height can be adjusted by pulling or pushing a broad lever - functional area of the shower head holder in high-quality, glossy polyamide - cone-shaped insert on the shower head holder makes it easier to mount the handheld shower head - wall mounting with wall-specific fixing material and roses from HEWI - can be mounted on the right and left side - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.35BUS220 |
| 801.35BUS310 Rail with sideways adjustable vertical support bar and shower head holder (USA) Matt edition 801.35BUS310 in product catalogue |  | Shower grab rail with adjustable shower head holder rail (USA) from Range 801 - positioned horizontally and vertically, rails connected at a right angle with steel fixing roses and shower head holder - with sideways (for mounting) adjustable vertical rail with shower head holder - with corrosion-protected steel core throughout - vertical length 49 3/16 inch (1250 mm), horizontal lengths 37 1/2 inch (952 mm) and 25 1/2 inch (647 mm) - 2 3/4 inch (70 mm) deep, rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - suitable for handheld shower heads from various manufacturers - shower head holder can be tilted continuously and its height can be adjusted by pulling or pushing a broad lever - functional area of the shower head holder in high-quality, glossy polyamide - cone-shaped insert on the shower head holder makes it easier to mount the handheld shower head - wall mounting with wall-specific fixing material and roses from HEWI - can be mounted on the right and left side - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.35BUS310 |
| 801.35US100 Shower grab bar (ADA) 37 1/2" x 19 1/2" (952 x 495 mm) 801.35US100 in product catalogue |  | Shower grab rail (USA) from Range 801 - arranged horizontally, rods connected at a right angle with steel fixing roses - horizontal lengths 37 1/2 inch x 19 1/2 inch (952 x 495 mm) - 2 3/4 inch (70 mm) deep, rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - can be mounted on the right and left side - suitable for removable hanging seats 801.51US100 and 801.51US110 - maximum loading capacity 330 lbs (150 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.35US100 |
| 801.35US110 Shower grab bar (ADA) 25 1/2" x 25 1/2" (647 x 647 mm) 801.35US110 in product catalogue |  | Shower grab rail (USA) from Range 801 - arranged horizontally, rods connected at a right angle with steel fixing roses - horizontal lengths 25 1/2 inch x 25 1/2 inch (647 x 647 mm) - 2 3/4 inch (70 mm) deep, rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - can be mounted on the right and left side - suitable for removable hanging seats 801.51US100 and 801.51US110 - maximum loading capacity 330 lbs (150 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.35US110 |
| 801.35US120 Shower grab bar (ADA) 37 1/2" x 37 1/2" (952 x 952 mm) 801.35US120 in product catalogue |  | Shower grab rail (USA) from Range 801 - arranged horizontally, rods connected at a right angle with steel fixing roses - horizontal lengths 37 1/2 inch x 37 1/2 inch (952 x 952 mm) - 2 3/4 inch (70 mm) deep, rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - can be mounted on the right and left side - suitable for removable hanging seats 801.51US100 and 801.51US110 - maximum loading capacity 330 lbs (150 kg) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.35US120 |
|  |  | Shower grab rail (USA) from Range 801 - horizontally arranged rails connected at right-angles, with fixing roses made of steel - horizontal lengths made-to-measure from 18 11/16 inch (475 mm) to 70 7/8 inch (1800 mm) - 2 3/4 inch (70 mm deep), rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - additional support required for lengths above 47 1/4 inch (1200 mm) as spacings between supports may not exceed 33 7/16 inch (850 mm) - with continuous, corrosion resistant steel core - for wall mounting with special HEWI fixing materials and roses - can be mounted in left hand or right hand position - use for removable hanging seats 801.51US100 (W1/W2 = min. 25 3/4 inch (650 mm)) and 801.51US110 (W1/W2 = min. 29 1/2 inch (750 mm)) - removable hanging seats only conditionally usable, if W1 or W2 = 40 15/16 inch (1040 mm) to 70 7/8 inch (1800 mm) - maximum loading capacity 330 lbs (150 kg) - made of high-quality polyamide in HEWI colours 99 (pure white), 95 (stone grey), 92 (anthracite grey), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 Article no.: HEWI 801.35US1S |
| 801.35US200 Shower grab bar (USA) 30" x 31 1/2" x 31 1/2" (762 x 800 x 800 mm) 801.35US200 in product catalogue |  | Shower grab rail (USA) from Range 801 - positioned horizontally and vertically, rails connected at a right angle with steel fixing roses - with corrosion-protected steel core throughout - vertical length 30 inch (762 mm), horizontal lengths 31 1/2 inch (800 mm) - 2 3/4 inch (70 mm) deep, rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - wall mounting with wall-specific fixing material and roses from HEWI - can be mounted on the right and left side - maximum loading capacity 330 lbs (150 kg) - suitable for removable hanging seats 801.51US100 and 801.51US110 - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.35US200 |
| 801.35US220 Shower grab bar (USA) 49 3/16" x 31 1/2" x 31 1/2" (1250 x 800 x 800 mm) 801.35US220 in product catalogue |  | Shower grab rail with shower head holder rail (USA) from Range 801 - positioned horizontally and vertically, rails connected at a right angle with steel fixing roses and shower head holder - with corrosion-protected steel core throughout - vertical length 49 3/16 inch (1250 mm), horizontal lengths 31 1/2 inch (800 mm) - 2 3/4 inch (70 mm) deep, rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - suitable for handheld shower heads from various manufacturers - shower head holder can be tilted continuously and its height can be adjusted by pulling or pushing a broad lever - cone-shaped insert on the shower head holder makes it easier to mount the handheld shower head - wall mounting with wall-specific fixing material and roses from HEWI - can be mounted on the right and left side - maximum loading capacity 330 lbs (150 kg) - suitable for removable hanging seats 801.51US100 and 801.51US110 - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.35US220 |
|  |  | Shower grab rail (USA) from Range 801 - vertical and horizontal bars joined at right-angles with fixing roses made of steel - vertical length made-to-measure from 23 5/8 inch (600 mm) to 49 3/16 inch (1250 mm), horizontal length made-to-measure from 18 11/16 inch (475 mm) to 70 7/8 inch (1800 mm) - 2 3/4 inch (70 mm) deep, rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - additional support required for lengths above 47 1/4 inch (1200 mm) as spacings between supports may not exceed 33 7/16 inch (850 mm) - with continuous, corrosion resistant steel core - for wall mounting with special HEWI fixing materials and roses - can be mounted in left hand or right hand position - maximum loading capacity 330 lbs (150 kg) - use for removable hanging seats 801.51US100 (W1/W2 = min. 25 3/4 inch (650 mm)) and 801.51US110 (W1/W2 = min. 29 1/2 inch (750 mm)) - removable hanging seats only conditionally usable, if W1 or W2 = 40 15/16 inch (1040 mm) to 70 7/8 inch (1800 mm) - made of high-quality polyamide in HEWI colours 99 (pure white), 95 (stone grey), 92 (anthracite grey), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 Article no.: HEWI 801.35US2S |
| 801.35US310 Rail with sideways adjustable vertical support bar and shower head holder (USA) 49 3/16" x 37 1/2" x 25 1/2" (1250 x 952 x 647 mm) 801.35US310 in product catalogue |  | Shower grab rail with adjustable shower head holder rail (USA) from Range 801 - positioned horizontally and vertically, rails connected at a right angle with steel fixing roses and shower head holder - with sideways (for mounting) adjustable vertical rail with shower head holder - with corrosion-protected steel core throughout - vertical length 49 3/16 inch (1250 mm), horizontal lengths (corner mounting) 37 1/2 inch (952 mm) and 25 1/2 inch (647 mm) - 2 3/4 inch (70 mm) deep, rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - suitable for handheld shower heads from various manufacturers - shower head holder can be tilted continuously and its height can be adjusted by pulling or pushing a broad lever - cone-shaped insert on the shower head holder makes it easier to mount the handheld shower head - wall mounting with wall-specific fixing material and roses from HEWI - can be mounted on the right and left side - maximum loading capacity 330 lbs (150 kg) - suitable for removable hanging seats 801.51US100 and 801.51US110 - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.35US310 |
|  |  | Rail with vertical support bar and sliding shower head holder (USA) from Range 801 - vertical and horizontal rails joined to form an L shape and fitted with fixing roses made of steel and a shower head holder - with vertical rail with shower head holder that can be moved to the side (for installation) - vertical length made-to-measure from 23 5/8 inch (600 mm) to 49 3/16 inch (1250 mm), horizontal lengths (corner mounting) made-to-measure from 18 11/16 inch (475 mm) to 70 7/8 inch (1800 mm) - 2 3/4 inch (70 mm) deep, rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - additional support required for lengths above 47 1/4 inch (1200 mm) as spacings between supports may not exceed 33 7/16 inch (850 mm) - suitable for shower heads of various manufacturers - shower head holder can be continuously tilted and, after pulling or pushing a large lever, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the handheld shower head - with continuous, corrosion resistant steel core - for wall mounting with special HEWI fixing materials and roses - can be mounted in left hand or right hand position - use for removable hanging seats 801.51US100 (W1/W2 = min. 25 3/4 inch (650 mm)) and 801.51US110 (W1/W2 = min. 29 1/2 inch (750 mm)) - removable hanging seats only conditionally usable, if W1 or W2 = 40 15/16 inch (1040 mm) to 70 7/8 inch (1800 mm) - made of high-quality polyamide in HEWI colours 99 (pure white), 95 (stone grey), 92 (anthracite grey), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 Article no.: HEWI 801.35US3S |
| 900.35.16040 Shower grab rail chrome 900.35.16040 in product catalogue |  | Shower handrail, System 900 - horizontally arranged rails connected at right-angles, with mounting roses - right-hand version - horizontal lengths 25 1/2 inch (647 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, chrome - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.16040 |
| 900.35.16060 Shower grab rail powder-coated 900.35.16060 in product catalogue |  | Shower handrail, System 900 - horizontally arranged rails connected at right-angles, with mounting roses - right-hand version - horizontal lengths 25 1/2 inch x 647 2/3 inch (4 x 70 mm) - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.16060 |
| 900.35.160XA Shower grab rail satin 900.35.160XA in product catalogue |  | Shower handrail, System 900 - horizontally arranged rails connected at right-angles, with mounting roses - right-hand version - horizontal lengths 25 1/2 inch (647 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, satin - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.160XA |
| 900.35.16140 Shower grab rail chrome 900.35.16140 in product catalogue |  | Shower handrail, System 900 - horizontally arranged rails connected at right-angles, with mounting roses - right-hand version - horizontal lengths 37 5/16 inch x 19 1/2 inch (947 x 495 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, chrome - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.16140 |
| 900.35.16160 Shower grab rail powder-coated 900.35.16160 in product catalogue |  | Shower handrail, System 900 - horizontally arranged rails connected at right-angles, with mounting roses - right-hand version - horizontal lengths 37 5/16 inch x 19 1/2 inch (947 x 495 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.16160 |
| 900.35.161XA Shower grab rail satin 900.35.161XA in product catalogue |  | Shower handrail, System 900 - horizontally arranged rails connected at right-angles, with mounting roses - right-hand version - horizontal lengths 37 5/16 inch x 19 1/2 inch (947 x 495 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, satin - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.161XA |
| 900.35.16240 Shower grab rail chrome 900.35.16240 in product catalogue |  | Shower handrail, System 900 - horizontally arranged rails connected at right-angles, with mounting roses - right-hand version - horizontal lengths 37 5/16 inch (947 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, chrome - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.16240 |
| 900.35.16260 Shower grab rail powder-coated 900.35.16260 in product catalogue |  | Shower handrail, System 900 - horizontally arranged rails connected at right-angles, with mounting roses - right-hand version - horizontal lengths 37 5/16 inch (947 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.16260 |
| 900.35.162XA Shower grab rail satin 900.35.162XA in product catalogue |  | Shower handrail, System 900 - horizontally arranged rails connected at right-angles, with mounting roses - right-hand version - horizontal lengths 37 5/16 inch (947 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, satin - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.162XA |
| 900.35.16340 Shower grab rail chrome 900.35.16340 in product catalogue |  | Shower handrail, System 900 - horizontally arranged rails connected at right-angles, with mounting roses - left-hand version - horizontal lengths 25 1/2 inch x 647 2/3 inch (4 x 70 mm) - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, chrome - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.16340 |
| 900.35.16360 Shower grab rail powder-coated 900.35.16360 in product catalogue |  | Shower handrail, System 900 - horizontally arranged rails connected at right-angles, with mounting roses - left-hand version - horizontal lengths 25 1/2 inch x 647 2/3 inch (4 x 70 mm) - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.16360 |
| 900.35.163XA Shower grab rail satin 900.35.163XA in product catalogue |  | Shower handrail, System 900 - horizontally arranged rails connected at right-angles, with mounting roses - left-hand version - horizontal lengths 25 1/2 inch x 647 2/3 inch (4 x 70 mm) - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, satin - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.163XA |
| 900.35.16440 Shower grab rail chrome 900.35.16440 in product catalogue |  | Shower handrail, System 900 - horizontally arranged rails connected at right-angles, with mounting roses - left-hand version - horizontal lengths 37 5/16 inch x 19 1/2 inch (947 x 495 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, chrome - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.16440 |
| 900.35.16460 Shower grab rail powder-coated 900.35.16460 in product catalogue |  | Shower handrail, System 900 - horizontally arranged rails connected at right-angles, with mounting roses - left-hand version - horizontal lengths 37 5/16 inch x 19 1/2 inch (947 x 495 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.16460 |
| 900.35.164XA Shower grab rail satin 900.35.164XA in product catalogue |  | Shower handrail, System 900 - horizontally arranged rails connected at right-angles, with mounting roses - left-hand version - horizontal lengths 37 5/16 inch x 19 1/2 inch (947 x 495 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, satin - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.164XA |
| 900.35.16540 Shower grab rail chrome 900.35.16540 in product catalogue |  | Shower handrail, System 900 - horizontally arranged rails connected at right-angles, with mounting roses - left-hand version - horizontal lengths 37 5/16 inch (947 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, chrome - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.16540 |
| 900.35.16560 Shower grab rail powder-coated 900.35.16560 in product catalogue |  | Shower handrail, System 900 - horizontally arranged rails connected at right-angles, with mounting roses - left-hand version - horizontal lengths 37 5/16 inch (947 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.16560 |
| 900.35.165XA Shower grab rail satin 900.35.165XA in product catalogue |  | Shower handrail, System 900 - horizontally arranged rails connected at right-angles, with mounting roses - left-hand version - horizontal lengths 37 5/16 inch (947 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, satin - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.165XA |
|  |  | Shower handrail, System 900 - horizontally arranged rails connected at right-angles, with mounting roses - right-hand version - horizontal lengths made-to-measure, each from 18 11/16 inch (475 mm) to 70 7/8 inch (1800 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made of high-quality stainless steel, satin - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 Article no.: HEWI 900.35.1S4XA |
|  |  | Shower handrail, System 900 - horizontally arranged rails connected at right-angles, with mounting roses - left-hand version - horizontal lengths made-to-measure, each from 18 11/16 inch (475 mm) to 70 7/8 inch (1800 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made of high-quality stainless steel, satin - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 Article no.: HEWI 900.35.1S5XA |
| 900.35.36040 Shower grab rail with sideways adjustable vertical support bar and shower head holder chrome 900.35.36040 in product catalogue |  | Shower handrail with sliding shower head rail, System 900 - vertical and horizontally arranged rails connected at right angles with mounting roses and shower head holder - with shower head rail that can be pushed to the side (for installation) - right-hand version - vertical length 49 3/16 inch (1250 mm), horizontal lengths (corner mounting) 31 1/2 inch (800 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, chrome - shower head holder made of high-quality matt polyamide in the HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.36040 |
| 900.35.36060 Shower grab rail with sideways adjustable vertical support bar and shower head holder powder-coated 900.35.36060 in product catalogue |  | Shower handrail with sliding shower head rail, System 900 - vertical and horizontally arranged rails connected at right angles with mounting roses and shower head holder - with shower head rail that can be pushed to the side (for installation) - right-hand version - vertical length 49 3/16 inch (1250 mm), horizontal lengths (corner mounting) 31 1/2 inch (800 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, powder-coated in the HEWI colour CV (matt black) and ER (matt dark grey pearl mica) with a showerhead holder in matt black polyamide or powder-coated in the HEWI colours AS (matt white) and AX (matt light grey pearl mica) with a showerhead holder in matt white polyamide - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.36060 |
| 900.35.360XA Shower grab rail with sideways adjustable vertical support bar and shower head holder satin 900.35.360XA in product catalogue |  | Shower handrail with sliding shower head rail, System 900 - vertical and horizontally arranged rails connected at right angles with mounting roses and shower head holder - with shower head rail that can be pushed to the side (for installation) - right-hand version - vertical length 49 3/16 inch (1250 mm), horizontal lengths (corner mounting) 31 1/2 inch (800 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, satin - shower head holder made of high-quality matt polyamide in the HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.360XA |
| 900.35.36140 Shower grab rail with sideways adjustable vertical support bar and shower head holder chrome 900.35.36140 in product catalogue |  | Shower handrail with sliding shower head rail, System 900 - vertical and horizontally arranged rails connected at right angles with mounting roses and shower head holder - with shower head rail that can be pushed to the side (for installation) - right-hand version - vertical length 49 3/16 inch (1250 mm), horizontal lengths (corner mounting) 37 5/16 inch (947 mm) and 25 1/2 inch (647 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, chrome - shower head holder made of high-quality matt polyamide in the HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.36140 |
| 900.35.36160 Shower grab rail with sideways adjustable vertical support bar and shower head holder powder-coated 900.35.36160 in product catalogue |  | Shower handrail with sliding shower head rail, System 900 - vertical and horizontally arranged rails connected at right angles with mounting roses and shower head holder - with shower head rail that can be pushed to the side (for installation) - right-hand version - vertical length 49 3/16 inch (1250 mm), horizontal lengths (corner mounting) 37 5/16 inch (947 mm) and 25 1/2 inch (647 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, powder-coated in the HEWI colour CV (matt black) and ER (matt dark grey pearl mica) with a showerhead holder in matt black polyamide or powder-coated in the HEWI colours AS (matt white) and AX (matt light grey pearl mica) with a showerhead holder in matt white polyamide - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.36160 |
| 900.35.361XA Shower grab rail with sideways adjustable vertical support bar and shower head holder satin 900.35.361XA in product catalogue |  | Shower handrail with sliding shower head rail, System 900 - vertical and horizontally arranged rails connected at right angles with mounting roses and shower head holder - with shower head rail that can be pushed to the side (for installation) - right-hand version - vertical length 49 3/16 inch (1250 mm), horizontal lengths (corner mounting) 37 5/16 inch (947 mm) and 25 1/2 inch (647 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, satin - shower head holder made of high-quality matt polyamide in the HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.361XA |
| 900.35.36240 Shower grab rail with sideways adjustable vertical support bar and shower head holder chrome 900.35.36240 in product catalogue |  | Shower handrail with sliding shower head rail, System 900 - vertical and horizontally arranged rails connected at right angles with mounting roses and shower head holder - with shower head rail that can be pushed to the side (for installation) - left-hand version - vertical length 49 3/16 inch (1250 mm), horizontal lengths (corner mounting) 31 1/2 inch (800 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, chrome - shower head holder made of high-quality matt polyamide in the HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.36240 |
| 900.35.36260 Shower grab rail with sideways adjustable vertical support bar and shower head holder powder-coated 900.35.36260 in product catalogue |  | Shower handrail with sliding shower head rail, System 900 - vertical and horizontally arranged rails connected at right angles with mounting roses and shower head holder - with shower head rail that can be pushed to the side (for installation) - left-hand version - vertical length 49 3/16 inch (1250 mm), horizontal lengths (corner mounting) 31 1/2 inch (800 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, powder-coated in the HEWI colour CV (matt black) and ER (matt dark grey pearl mica) with a showerhead holder in matt black polyamide or powder-coated in the HEWI colours AS (matt white) and AX (matt light grey pearl mica) with a showerhead holder in matt white polyamide - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.36260 |
| 900.35.362XA Shower grab rail with sideways adjustable vertical support bar and shower head holder satin 900.35.362XA in product catalogue |  | Shower handrail with sliding shower head rail, System 900 - vertical and horizontally arranged rails connected at right angles with mounting roses and shower head holder - with shower head rail that can be pushed to the side (for installation) - left-hand version - vertical length 49 3/16 inch (1250 mm), horizontal lengths (corner mounting) 31 1/2 inch (800 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, satin - shower head holder made of high-quality matt polyamide in the HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.362XA |
| 900.35.36340 Shower grab rail with sideways adjustable vertical support bar and shower head holder chrome 900.35.36340 in product catalogue |  | Shower handrail with sliding shower head rail, System 900 - vertical and horizontally arranged rails connected at right angles with mounting roses and shower head holder - with shower head rail that can be pushed to the side (for installation) - left-hand version - vertical length 49 3/16 inch (1250 mm), horizontal lengths (corner mounting) 37 5/16 inch (947 mm) and 25 1/2 inch (647 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, chrome - shower head holder made of high-quality matt polyamide in the HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.36340 |
| 900.35.36360 Shower grab rail with sideways adjustable vertical support bar and shower head holder powder-coated 900.35.36360 in product catalogue |  | Shower handrail with sliding shower head rail, System 900 - vertical and horizontally arranged rails connected at right angles with mounting roses and shower head holder - with shower head rail that can be pushed to the side (for installation) - left-hand version - vertical length 49 3/16 inch (1250 mm), horizontal lengths (corner mounting) 37 5/16 inch (947 mm) and 25 1/2 inch (647 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, powder-coated in the HEWI colour CV (matt black) and ER (matt dark grey pearl mica) with a showerhead holder in matt black polyamide or powder-coated in the HEWI colours AS (matt white) and AX (matt light grey pearl mica) with a showerhead holder in matt white polyamide - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.36360 |
| 900.35.363XA Shower grab rail with sideways adjustable vertical support bar and shower head holder satin 900.35.363XA in product catalogue |  | Shower handrail with sliding shower head rail, System 900 - vertical and horizontally arranged rails connected at right angles with mounting roses and shower head holder - with shower head rail that can be pushed to the side (for installation) - left-hand version - vertical length 49 3/16 inch (1250 mm), horizontal lengths (corner mounting) 37 5/16 inch (947 mm) and 25 1/2 inch (647 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - weight capacity up to 330 lbs (150 kg) - made of high-quality stainless steel, satin - shower head holder made of high-quality matt polyamide in the HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.363XA |
|  |  | Shower handrail with sliding shower head rail, System 900 - vertical and horizontally arranged rails connected at right angles with mounting roses and shower head holder - with shower head rail that can be pushed to the side (for installation) - right-hand version - vertical length made-to-measure from 23 5/8 inch (600 mm) to 49 3/16 inch (1250 mm), horizontal length made-to-measure from 18 11/16 inch (475 mm) to 70 7/8 inch (1800 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made of high-quality stainless steel, satin - shower head holder made of high-quality matt polyamide in HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 Article no.: HEWI 900.35.3S4XA |
|  |  | Shower handrail with sliding shower head rail, System 900 - vertical and horizontally arranged rails connected at right angles with mounting roses and shower head holder - with shower head rail that can be pushed to the side (for installation) - left-hand version - vertical length made-to-measure from 23 5/8 inch (600 mm) to 49 3/16 inch (1250 mm), horizontal length made-to-measure from 18 11/16 inch (475 mm) to 70 7/8 inch (1800 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made of high-quality stainless steel, satin - shower head holder made of high-quality matt polyamide in HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - reduced number of mounting roses due to innovative corner connection - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 Article no.: HEWI 900.35.3S5XA |
| 900.35.46040 Shower grab rail with sideways adjustable vertical support bar and shower head holder chrome 900.35.46040 in product catalogue |  | Shower handrail with sliding shower head rail, System 900 - vertical and horizontally arranged rails connected at right angles with mounting roses and shower head holder - with shower head rail that can be pushed to the side (for installation) - vertical length 49 3/16 inch (1250 mm), horizontal length 18" (457 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - made of high-quality stainless steel, chrome - shower head holder made of high-quality matt polyamide in the HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - maximum loading capacity 330 lbs (150 kg) - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.46040 |
| 900.35.46060 Shower grab rail with sideways adjustable vertical support bar and shower head holder powder-coated 900.35.46060 in product catalogue |  | Shower handrail with sliding shower head rail, System 900 - vertical and horizontally arranged rails connected at right angles with mounting roses and shower head holder - with shower head rail that can be pushed to the side (for installation) - vertical length 49 3/16 inch (1250 mm), horizontal length 18 inch (457 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - made of high-quality stainless steel, powder-coated in the HEWI colour CV (matt black) and ER (matt dark grey pearl mica) with a showerhead holder in matt black polyamide or powder-coated in the HEWI colours AS (matt white) and AX (matt light grey pearl mica) with a showerhead holder in matt white polyamide - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - maximum loading capacity 330 lbs (150 kg) - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.46060 |
| 900.35.460XA Shower grab rail with sideways adjustable vertical support bar and shower head holder satin 900.35.460XA in product catalogue |  | Shower handrail with sliding shower head rail, System 900 - vertical and horizontally arranged rails connected at right angles with mounting roses and shower head holder - with shower head rail that can be pushed to the side (for installation) - vertical length 49 3/16 inch (1250 mm), horizontal length 18 inch (457 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - made of high-quality stainless steel, satin - shower head holder made of high-quality matt polyamide in the HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - maximum loading capacity 330 lbs (150 kg) - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.460XA |
| 900.35.46140 Shower grab rail with sideways adjustable vertical support bar and shower head holder chrome 900.35.46140 in product catalogue |  | Shower handrail with sliding shower head rail, System 900 - vertical and horizontally arranged rails connected at right angles with mounting roses and shower head holder - with shower head rail that can be pushed to the side (for installation) - vertical length 49 3/16 inch (1250 mm), horizontal length 24 inch (610 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - made of high-quality stainless steel, chrome - shower head holder made of high-quality matt polyamide in the HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - maximum loading capacity 330 lbs (150 kg) - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.46140 |
| 900.35.46160 Shower grab rail with sideways adjustable vertical support bar and shower head holder powder-coated 900.35.46160 in product catalogue |  | Shower handrail with sliding shower head rail, System 900 - vertical and horizontally arranged rails connected at right angles with mounting roses and shower head holder - with shower head rail that can be pushed to the side (for installation) - vertical length 49 3/16 inch (1250 mm), horizontal length 24 inch (610 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - made of high-quality stainless steel, powder-coated in the HEWI colour CV (matt black) and ER (matt dark grey pearl mica) with a showerhead holder in matt black polyamide or powder-coated in the HEWI colours AS (matt white) and AX (matt light grey pearl mica) with a showerhead holder in matt white polyamide - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - maximum loading capacity 330 lbs (150 kg) - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.46160 |
| 900.35.461XA Shower grab rail with sideways adjustable vertical support bar and shower head holder satin 900.35.461XA in product catalogue |  | Shower handrail with sliding shower head rail, System 900 - vertical and horizontally arranged rails connected at right angles with mounting roses and shower head holder - with shower head rail that can be pushed to the side (for installation) - vertical length 49 3/16 inch (1250 mm), horizontal length 24 inch (610 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - made of high-quality stainless steel, satin - shower head holder made of high-quality matt polyamide in the HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - maximum loading capacity 330 lbs (150 kg) - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.461XA |
| 900.35.46240 Shower grab rail with sideways adjustable vertical support bar and shower head holder chrome 900.35.46240 in product catalogue |  | Shower handrail with sliding shower head rail, System 900 - vertical and horizontally arranged rails connected at right angles with mounting roses and shower head holder - with shower head rail that can be pushed to the side (for installation) - vertical length 49 3/16 inch (1250 mm), horizontal length 36 inch (914 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - made of high-quality stainless steel, chrome - shower head holder made of high-quality matt polyamide in the HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - maximum loading capacity 330 lbs (150 kg) - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.46240 |
| 900.35.46260 Shower grab rail with sideways adjustable vertical support bar and shower head holder powder-coated 900.35.46260 in product catalogue |  | Shower handrail with sliding shower head rail, System 900 - vertical and horizontally arranged rails connected at right angles with mounting roses and shower head holder - with shower head rail that can be pushed to the side (for installation) - vertical length 49 3/16 inch (1250 mm), horizontal length 36 inch (914 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - made of high-quality stainless steel, powder-coated in the HEWI colour CV (matt black) and ER (matt dark grey pearl mica) with a showerhead holder in matt black polyamide or powder-coated in the HEWI colours AS (matt white) and AX (matt light grey pearl mica) with a showerhead holder in matt white polyamide - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - maximum loading capacity 330 lbs (150 kg) - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.46260 |
| 900.35.462XA Shower grab rail with sideways adjustable vertical support bar and shower head holder satin 900.35.462XA in product catalogue |  | Shower handrail with sliding shower head rail, System 900 - vertical and horizontally arranged rails connected at right angles with mounting roses and shower head holder - with shower head rail that can be pushed to the side (for installation) - vertical length 49 3/16 inch (1250 mm), horizontal length 36 inch (914 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - made of high-quality stainless steel, satin - shower head holder made of high-quality matt polyamide in the HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - maximum loading capacity 330 lbs (150 kg) - fulfils the requirements of the 2010 ADA Standards and ICC/ANSI A117.1-2017 Article no.: HEWI 900.35.462XA |
|  |  | Shower handrail with sliding shower head rail, System 900 - vertical and horizontally arranged rails connected at right angles with mounting roses and shower head holder - with shower head rail that can be pushed to the side (for installation) - vertical length made-to-measure from 23 5/8 inch (600 mm) to 49 3/16 inch (1250 mm), horizontal length made-to-measure from 11 13/16 inch (300 mm) to 59 1/16 inch (1500 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 1/4 inch (32 mm), tube thickness 1/16 inch (2 mm), rose diameter 2 3/4 inch (70 mm) - made of high-quality stainless steel, satin - shower head holder made of high-quality matt polyamide in HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points - maximum loading capacity 330 lbs (150 kg) - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 Article no.: HEWI 900.35.4S2XA |
| Accessories |  |  |
| Accessories => Basin area |  |  |
| 162.03.110540 Shelf 600 mm finish - chrome 162.03.110540 in product catalogue |  | System 162 Shelf - shelf consisting of lateral supports and a shelf made of glass - holder made of metal tube diameter 18 mm, high-quality chrome-plated - shelf made of glass (4 mm) with matt sandblasted underside - 600 mm wide and 122 mm deep - for wall mounting, concealed fixing - not suitable for use in the shower - incl. non-corrosive HEWI fixing materials Article no.: HEWI 162.03.110540 |
| 162.03.1105XA Shelf 600 mm satin 162.03.1105XA in product catalogue |  | System 162 Shelf - shelf consisting of lateral supports and a shelf made of glass - holder made of high-quality stainless steel tube diameter 18 mm, satin finish - shelf made of glass (4 mm) with matt sandblasted underside - 600 mm wide and 122 mm deep - for wall mounting, concealed fixing - not suitable for use in the shower - incl. non-corrosive HEWI fixing materials Article no.: HEWI 162.03.1105XA |
| 162.03.11560 Shelf powder-coated 162.03.11560 in product catalogue |  | System 162 Shelf - shelf consisting of lateral supports and a shelf made of glass - holder made of metal tube diameter 18 mm, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - shelf made of glass (4 mm) with matt sandblasted underside - 600 mm wide and 122 mm deep - for wall mounting, concealed fixing - not suitable for use in the shower - incl. non-corrosive HEWI fixing materials Article no.: HEWI 162.03.11560 |
| 162.04.11040 Tumbler with holder chrome 162.04.11040 in product catalogue |  | System 162 Tumbler with holder - cylindrical wall holder - holder made of metal, high-quality chrome-plated - holder with protective film on the inside for a secure, defined position and to protect the insert from damage - diameter 71 mm, 124 mm high and 84 mm deep - cylindrical tumbler made of high-quality polyamide in HEWI colours 90 (jet black), 92 (anthracite grey), 98 (signal white), 99 (pure white) - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 162.04.11040 |
| 162.04.11060 Tumbler with holder powder-coated 162.04.11060 in product catalogue |  | System 162 Tumbler with holder - cylindrical wall holder - holder made of high-quality satin stainless steel, powder-coated in the HEWI colour CV (matt black) and ER (matt dark grey pearl mica) with a cylindrical tumbler in matt black polyamide or powder-coated in the HEWI colours AS (matt white) and AX (matt light grey pearl mica) with a cylindrical tumbler in matt white polyamide - holder with protective film on the inside for a secure, defined position and to protect the insert from damage - diameter 71 mm, 124 mm high and 84 mm deep - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 162.04.11060 |
| 162.04.110XA Tumbler with holder satin 162.04.110XA in product catalogue |  | System 162 Tumbler with holder - cylindrical wall holder - holder made of high-quality satin stainless steel - holder with protective film on the inside for a secure, defined position and to protect the insert from damage - diameter 70 mm, 124 mm high and 85 mm deep - cylindrical tumbler made of high-quality polyamide in HEWI colours 90 (jet black), 92 (anthracite grey), 98 (signal white), 99 (pure white) - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 162.04.110XA |
| 162.09.10040 Towel rail finish - chrome 162.09.10040 in product catalogue |  | System 162 Towel rail - towel rail with two cylindrical arms arranged above each other - for hanging up towels and other pieces of clothing - the arms can be swung to the side separately - made of metal, high-quality chrome-plated - 445 mm long and 70 mm high, diameter of the arms 15 mm - for wall mounting, concealed fixing - incl. non-corrosive HEWI fixing materials Article no.: HEWI 162.09.10040 |
| 162.09.10060 Towel rail powder-coated 162.09.10060 in product catalogue |  | System 162 Towel rail - towel rail with two cylindrical arms arranged above each other - for hanging up towels and other pieces of clothing - the arms can be swung to the side separately - made of metal, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - 445 mm long and 70 mm high, diameter of the arms 15 mm - for wall mounting, concealed fixing - incl. non-corrosive HEWI fixing materials Article no.: HEWI 162.09.10060 |
| 162.09.100XA Towel rail satin 162.09.100XA in product catalogue |  | System 162 Towel rail - towel rail with two cylindrical arms arranged above each other - for hanging up towels and other pieces of clothing - the arms can be swung to the side separately - made of stainless steel, satin finish - 445 mm long and 70 mm high, diameter of the arms 15 mm - for wall mounting, concealed fixing - incl. non-corrosive HEWI fixing materials Article no.: HEWI 162.09.100XA |
| 162.90.01040 Hook finish - chrome 162.90.01040 in product catalogue |  | System 162 Hook - cylindrical hook - made of metal, high-quality chrome-plated - 35 mm long, diameter 15 mm - for wall mounting, concealed fixing - incl. non-corrosive HEWI fixing materials Article no.: HEWI 162.90.01040 |
| 162.90.01060 Hook powder-coated 162.90.01060 in product catalogue |  | System 162 Hook - cylindrical hook - made of metal, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - 35 mm long, diameter 15 mm - for wall mounting, concealed fixing - incl. non-corrosive HEWI fixing materials Article no.: HEWI 162.90.01060 |
| 162.90.010XA Hook satin 162.90.010XA in product catalogue |  | System 162 Hook - cylindrical hook - made of stainless steel, satin finish - 35 mm long, diameter 15 mm - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 162.90.010XA |
| 162.90.03040 Double hook chrome 162.90.03040 in product catalogue |  | System 162 Double hook - double hook with two cylindrical hook elements - made of metal, high-quality chrome-plated - 37 mm deep, 58 mm wide - for wall mounting, concealed fixing - incl. non-corrosive HEWI fixing materials Article no.: HEWI 162.90.03040 |
| 162.90.03060 Double hook powder-coated 162.90.03060 in product catalogue |  | System 162 Double hook - double hook with two cylindrical hook elements - made of metal, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - 37 mm deep, 58 mm wide - for wall mounting, concealed fixing - incl. non-corrosive HEWI fixing materials Article no.: HEWI 162.90.03060 |
| 162.90.030XA Double hook satin 162.90.030XA in product catalogue |  | System 162 Double hook - double hook with two cylindrical hook elements - made of stainless steel, satin finish - 37 mm deep, 58 mm wide - for wall mounting, concealed fixing - incl. non-corrosive HEWI fixing materials Article no.: HEWI 162.90.030XA |
| 477.00.100 Holder 477.00.100 in product catalogue |  | Range 477 Holder, small - ring-shaped wall-mounted bracket - serves as holder for tumblers or soap dish inserts - 97 mm wide, 25 mm high and 100 mm deep, insert opening 74 mm in diameter - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.00.100 |
| 477.00B100 Holder Matt edition 477.00B100 in product catalogue |  | Range 477 Holder, small - ring-shaped wall-mounted bracket - serves as holder for tumblers or soap dish inserts - 97 mm wide, 25 mm high and 100 mm deep, insert opening 74 mm in diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.00B100 |
| 477.01.010 Plate glass mirror 600 mm wide, 390 mm high, 6 mm deep 477.01.010 in product catalogue |  | Range 477 Plate glass mirror, small - rectangular plate glass mirror with polished edges - can be held by four mirror clips (477.01.100) - 600 mm wide, 390 mm high and 6 mm thick Article no.: HEWI 477.01.010 |
| 477.02.100 Soap dish 477.02.100 in product catalogue |  | Range 477 Soap dish, small - Soap dish with knobs, push-button facilitated release - anti-theft locking system - 97 mm wide, 25 mm high and 100 mm deep - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.02.100 |
| 477.02.200 Soap dish 477.02.200 in product catalogue |  | Range 477 Soap dish, large - Soap dish with knobs, push-button facilitated release - anti-theft locking system - 120 mm wide, 25 mm high and 120 mm deep - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.02.200 |
| 477.02B100 Soap dish Matt edition 477.02B100 in product catalogue |  | Range 477 Soap dish, small - Soap dish with knobs, push-button facilitated release - anti-theft locking system - 97 mm wide, 25 mm high and 100 mm deep - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.02B100 |
| 477.02B200 Soap dish Matt edition 477.02B200 in product catalogue |  | Range 477 Soap dish, large - Soap dish with knobs, push-button facilitated release - anti-theft locking system - 120 mm wide, 25 mm high and 120 mm deep - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.02B200 |
| 477.03.100 Shelf 600 mm 477.03.100 in product catalogue |  | Range 477 Shelf - shelf with all-round edge - anti-theft locking system - 600 mm wide, 26 mm high and 140 mm deep - made of high-quality polyamide in HEWI colours 24 (orange), 33 (ruby red), 55 (aqua blue), 74 (apple green), 84 (umber), 86 (sand), 95 (stone grey), 97 (light grey), 98 (signal white), 99 (pure white) - including non-corrosive HEWI fixing material. Article no.: HEWI 477.03.100 |
| 477.03B100 Shelf 600 mm Matt edition 477.03B100 in product catalogue |  | Range 477 Shelf - shelf with all-round edge - anti-theft locking system - 600 mm wide, 26 mm high and 140 mm deep - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey) and 95 (stone grey) - including non-corrosive HEWI fixing material. Article no.: HEWI 477.03B100 |
| 477.04.020 Tumbler with flat bottom 477.04.020 in product catalogue |  | Range 477 Tumbler with flat bottom - round tumbler with flat bottom - free-standing and suitable as insert for the small holder (477.00.100) - 80 mm high and 74 mm diameter - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.04.020 |
| 477.04.02005 Tumbler with flat bottom 477.04.02005 in product catalogue |  | Range 477 Tumbler with flat bottom - round tumbler with flat bottom - free-standing and suitable as insert for small holder (477.00.100) - 80 mm high and 74 mm diameter - made of opaque white glass Article no.: HEWI 477.04.02005 |
| 477.04B020 Tumbler with flat bottom Matt edition 477.04B020 in product catalogue |  | Range 477 Tumbler with flat bottom - round tumbler with flat bottom - free-standing and suitable as insert for the small holder (477.00.100) - 80 mm high and 74 mm diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.04B020 |
| 477.05.20011 Paper towel basket black container 477.05.20011 in product catalogue |  | Range 477 Paper towel basket - rectangular container made of perforated sheet metal with polyamide top - serves as waste bin for used paper towels - removable top holds and covers waste bags - free-standing or wall-mounted - 305 mm wide, 515 mm high and 300 mm deep - perforated sheet metal, black - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.05.20011 |
| 477.05.20012 Paper towel basket white container 477.05.20012 in product catalogue |  | Range 477 Paper towel basket - rectangular container made of perforated sheet metal with polyamide top - serves as waste bin for used paper towels - removable top holds and covers waste bags - free-standing or wall-mounted - 305 mm wide, 515 mm high and 300 mm deep - perforated sheet metal, white - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.05.20012 |
| 477.05.300 Waste bin 477.05.300 in product catalogue |  | Range 477 Waste bin - cylindrical container with hinged lid - removable waste container - lid is opened by applying slight pressure to container with the knee or hand - integrated compartment for standard hygienic bags - with self-adhesive label with instructions for knee operation - only suitable for wall mounting - capacity approx. 6 litres - 340 mm high and 210 mm deep and 180 mm in diameter - made of high-quality polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey), 92 (anthracite grey), 90 (jet black) and 33 (ruby red) Article no.: HEWI 477.05.300 |
| 477.05B20011 Paper towel basket Matt edition 477.05B20011 in product catalogue |  | Range 477 Paper towel basket - rectangular container made of perforated sheet metal with polyamide top - serves as waste bin for used paper towels - removable top holds and covers waste bags - free-standing or wall-mounted - 305 mm wide, 515 mm high and 300 mm deep - perforated sheet metal, black - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.05B20011 |
| 477.05B20012 Paper towel basket Matt edition 477.05B20012 in product catalogue |  | Range 477 Paper towel basket - rectangular container made of perforated sheet metal with polyamide top - serves as waste bin for used paper towels - removable top holds and covers waste bags - free-standing or wall-mounted - 305 mm wide, 515 mm high and 300 mm deep - perforated sheet metal, white - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.05B20012 |
| 477.05B300 Waste bin Matt edition 477.05B300 in product catalogue |  | Range 477 Waste bin - cylindrical container with hinged lid - removable waste container - lid is opened by applying slight pressure to container with the knee or hand - integrated compartment for standard hygienic bags - with self-adhesive label with instructions for knee operation - only suitable for wall mounting - capacity approx. 6 litres - 340 mm high and 210 mm deep and 180 mm in diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.05B300 |
| 477.06.10005 Soap dispenser 477.06.10005 in product catalogue |  | Range 477 Soap dispenser - cylindrical container in ring-shaped, wall-mounted bracket - used to hold standard liquid soap - allows one-hand operation - soap dosage control by means of a large push-button at the front - container can be removed - anti-theft protection through concealed screw fixing - lockable container lid - with wall-mounted base to conceal drill holes - 70 mm wide, 208 mm high, 122 mm deep, diameter of container 75 mm, capacity 500 ml - wall-mounted base and holder made of high-quality polyamide according to HEWI colour chart - push-button made of high-quality polyamide in HEWI colour 92 (anthracite grey) - container and lid made of matt white transparent synthetic material Article no.: HEWI 477.06.10005 |
| 477.06.60005 Paper towel dispenser 477.06.60005 in product catalogue |  | Range 477 Paper towel dispenser - rectangular container with edged access opening - for standard paper towels, holds approx. 450 towels - user-friendly dispenser opening - easy to fill and lockable - for wall mounting - 310 mm wide, 459 mm high, 160 mm deep - container made of high-quality white-translucent synthetic material - dispenser opening edges with high-quality polyamide, available according to HEWI colour chart Article no.: HEWI 477.06.60005 |
| 477.06B10005 Soap dispenser Matt edition 477.06B10005 in product catalogue |  | Range 477 Soap dispenser - cylindrical container in ring-shaped, wall-mounted bracket - used to hold standard liquid soap - allows one-hand operation - soap dosage control by means of a large push-button at the front - container can be removed - anti-theft protection through concealed screw fixing - lockable container lid - with wall-mounted base to conceal drill holes - 70 mm wide, 208 mm high, 122 mm deep, diameter of container 75 mm, capacity 500 ml - wall-mounted base and holder made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) - push-button made of high-quality matt polyamide in HEWI colour 92 (anthracite grey) - container and lid made of matt white transparent synthetic material Article no.: HEWI 477.06B10005 |
| 477.06B60005 Paper towel dispenser Matt edition 477.06B60005 in product catalogue |  | Range 477 Paper towel dispenser - rectangular container with edged access opening - for standard paper towels, holds approx. 450 towels - user-friendly dispenser opening - easy to fill and lockable - for wall mounting - 310 mm wide, 459 mm high, 160 mm deep - container made of high-quality white-translucent synthetic material - dispenser opening edges with high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.06B60005 |
| 477.09.100 Towel rail 445 mm 477.09.100 in product catalogue |  | Range 477 Towel rail - towel rail with two cylindrical arms arranged above each other - for hanging up towels and other pieces of clothing - the arms can be swung to the side separately - 445 mm long and 160 mm high, diameter of the arms 28 mm - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.09.100 |
| 477.09.200 Towel rail 445 mm 477.09.200 in product catalogue |  | Range 477 Towel rail - towel rail with one cylindrical arm - for hanging up towels and other pieces of clothing - the arm can be swung to the side - 445 mm long and 105 mm high, arm diameter 28 mm - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.09.200 |
| 477.09.300 Towel ring 477.09.300 in product catalogue |  | Range 477 Towel ring - elongated ring - for hanging up towels and other pieces of clothing - the ring is pivoted and locks at approx. 30° - 320 mm wide and 150 mm high - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.09.300 |
| 477.09B100 Towel rail 445 mm Matt edition 477.09B100 in product catalogue |  | Range 477 Towel rail - towel rail with two cylindrical arms arranged above each other - for hanging up towels and other pieces of clothing - the arms can be swung to the side separately - 445 mm long and 160 mm high, diameter of the arms 28 mm - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.09B100 |
| 477.09B200 Towel rail 445 mm Matt edition 477.09B200 in product catalogue |  | Range 477 Towel rail - towel rail with one cylindrical arm - for hanging up towels and other pieces of clothing - the arm can be swung to the side - 445 mm long and 105 mm high, arm diameter 28 mm - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.09B200 |
| 477.09B300 Towel ring Matt edition 477.09B300 in product catalogue |  | Range 477 Towel ring - elongated ring - for hanging up towels and other pieces of clothing - the ring is pivoted and locks at approx. 30° - 320 mm wide and 150 mm high - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.09B300 |
| 477.90.010 Hook ø 50 mm 477.90.010 in product catalogue |  | Range 477 Single hook (small) - right-angled bend, cylindrical hook with fixing rose - for hanging up towels and other pieces of clothing and other utensils - anti-theft protection through concealed screw fixing - 45 mm deep, rose diameter 50 mm - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.90.010 |
| 477.90.015 Coat hook ø 50 mm 477.90.015 in product catalogue |  | Range 477 Hook - half-round bend cylindrical hook - for screwing on from underneath - for hanging up towels and other pieces of clothing and other utensils - anti-theft protection through concealed screw fixing - diameter rose 50 mm - 50 mm high - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.90.015 |
| 477.90.025 Double hook ø 50 mm 477.90.025 in product catalogue |  | Range 477 Double hook - double hook with fixing rose - for hanging up towels and other pieces of clothing and other utensils - diameter rose 50 mm - 43.5 mm deep - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.90.025 |
| 477.90.030 Hook ø 70 mm 477.90.030 in product catalogue |  | Range 477 Single hook (large) - right-angled bend, cylindrical hook with fixing rose - for hanging up towels and other pieces of clothing and other utensils - anti-theft protection through concealed screw fixing - 100 mm deep, rose diameter 70 mm - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.90.030 |
| 477.90.035 Coat hook with buffer door stop ø 70 mm 477.90.035 in product catalogue |  | Range 477 Wall hook - right-angled bend, cylindrical hook with fixing rose and black door stop - for hanging up towels and other pieces of clothing and other utensils - anti-theft protection through concealed screw fixing - 105 mm deep, rose diameter 70 mm - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.90.035 |
| 477.90.040 Coat hook ø 40 mm 477.90.040 in product catalogue |  | Range 477 Hook - half-round bend cylindrical hook - for screwing on from underneath - for hanging up towels and other pieces of clothing and other utensils - diameter rose 40 mm - 40 mm high - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.90.040 |
| 477.90.045 Coat hook ø 60 mm 477.90.045 in product catalogue |  | Range 477 Wall hook - right-angled bend, cylindrical hook with fixing rose - for hanging up towels and other pieces of clothing and other utensils - anti-theft protection through concealed screw fixing - 65 mm deep, rose diameter 60 mm - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.90.045 |
| 477.90.050 Triple hook ø 50 mm 477.90.050 in product catalogue |  | Range 477 Triple hook - rotatable hook for screwing on from underneath - for hanging up towels and other pieces of clothing and other utensils - diameter rose 50 mm - 70 mm high - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.90.050 |
| 477.90.051 Triple hook ø 50 mm 477.90.051 in product catalogue |  | Range 477 Triple hook - made of high-quality polyamide, with rotatable hook - support plate for carrying screws from below with three 4 mm countersunk wood screws - diameter rose 50 mm - 70 mm high - for boards up to 15 mm thickness. - supplied without screws and plugs. - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.90.051 |
| 477.90.052 Triple hook ø 50 mm 477.90.052 in product catalogue |  | Range 477 Triple hook - made of high-quality polyamide - with rotatable hook - diameter rose 50 mm - 70 mm high - support plate for carrying screws from below with three 4 mm countersunk wood screws - for boards over 15 mm thickness. - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.90.052 |
| 477.90B010 Hook Matt edition 477.90B010 in product catalogue |  | Range 477 Single hook (small) - right-angled bend, cylindrical hook with fixing rose - the hook is for hanging up pieces of clothing, towels and utensils - anti-theft protection through concealed screw fixing - 45 mm deep, rose diameter 50 mm - made of high-quality matt polyamide according to HEWI colour chart Article no.: HEWI 477.90B010 |
| 477.90B015 Coat hook Matt edition 477.90B015 in product catalogue |  | Range 477 Hook - half-round bend cylindrical hook - for screwing on from underneath - for hanging up towels and other pieces of clothing and other utensils - anti-theft protection through concealed screw fixing - diameter rose 50 mm - 50 mm high - made of high-quality matt polyamide according to HEWI colour chart Article no.: HEWI 477.90B015 |
| 477.90B025 Double hook Matt edition 477.90B025 in product catalogue |  | Range 477 Double hook - double hook with fixing rose - for hanging up towels and other pieces of clothing and other utensils - diameter rose 50 mm - 43.5 mm deep - made of high-quality matt polyamide according to HEWI colour chart Article no.: HEWI 477.90B025 |
| 477.90B030 Hook Matt edition 477.90B030 in product catalogue |  | Range 477 Single hook (large) - right-angled bend, cylindrical hook with fixing rose - for hanging up pieces of clothing and utensils - anti-theft protection through concealed screw fixing - 100 mm deep, rose diameter 70 mm - made of high-quality matt polyamide according to HEWI colour chart Article no.: HEWI 477.90B030 |
| 477.90B035 Coat hook with buffer door stop Matt edition 477.90B035 in product catalogue |  | Range 477 Wall hook - right-angled bend, cylindrical hook with fixing rose and black door stop - for hanging up towels and other pieces of clothing and other utensils - anti-theft protection through concealed screw fixing - 105 mm deep, rose diameter 70 mm - made of high-quality matt polyamide according to HEWI colour chart Article no.: HEWI 477.90B035 |
| 477.90B040 Coat hook Matt edition 477.90B040 in product catalogue |  | Range 477 Hook - half-round bend cylindrical hook - for screwing on from underneath - for hanging up towels and other pieces of clothing and other utensils - diameter rose 40 mm - 40 mm high - made of high-quality matt polyamide according to HEWI colour chart Article no.: HEWI 477.90B040 |
| 477.90B045 Coat hook Matt edition 477.90B045 in product catalogue |  | Range 477 Wall hook - right-angled bend, cylindrical hook with fixing rose - for hanging up towels and other pieces of clothing and other utensils - anti-theft protection through concealed screw fixing - 65 mm deep, rose diameter 60 mm - made of high-quality matt polyamide according to HEWI colour chart Article no.: HEWI 477.90B045 |
| 477.90B050 Triple hook Matt edition 477.90B050 in product catalogue |  | Range 477 Triple hook - rotatable hook for screwing on from underneath - for hanging up towels and other pieces of clothing and other utensils - diameter rose 50 mm - 70 mm high - made of high-quality matt polyamide according to HEWI colour chart Article no.: HEWI 477.90B050 |
| 477.90B051 Triple hook Matt edition 477.90B051 in product catalogue |  | Range 477 Triple hook - made of high-quality matt polyamide, with rotatable hook - support plate for carrying screws from below with three 4 mm countersunk wood screws - diameter rose 50 mm - 70 mm high - for boards up to 15 mm thickness. - supplied without screws and plugs. - made of high-quality matt polyamide according to HEWI colour chart Article no.: HEWI 477.90B051 |
| 477.90B052 Triple hook Matt edition 477.90B052 in product catalogue |  | Range 477 Triple hook - made of high-quality matt polyamide - with rotatable hook - diameter rose 50 mm - 70 mm high - support plate for carrying screws from below with three 4 mm countersunk wood screws - for boards over 15 mm thickness. - made of high-quality matt polyamide according to HEWI colour chart Article no.: HEWI 477.90B052 |
| 63070 HEWI case for soap dispenser, serie 800K 63070 in product catalogue | Similar picture | Container - for soap dispenser 800.06.010 and product-Sets 800.06.11041, 815.06.110, 815.06.11090, 162.06.110XA, 162.06.11040, 162.06.119XA, 162.06.11940, 900.06.00140, 900.06.00160 and 900.06.001XA - Diameter 69 mm, 113 mm high - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 63070 |
| 63501 Glass tumbler for tumbler with holder 815.04.11005, 815.04.11095, 900.04.00040, 900.04.00060 and 900.04.000XA 63501 in product catalogue |  | Tumbler - for tumbler with holder 815.04.11045, 815.04.11065, 815.04.11005, 815.04.11095, 900.04.00040, 900.04.00060 and 900.04.000XA - made of high-quality, satin-finish glass - 69 mm diameter, 110 mm high Article no.: HEWI 63501 |
| 63502 Glass container for soap dispenser with holder 815.06.11005 and 815.06.11095 63502 in product catalogue |  | Glass container - for soap dispenser with holder 815.06.11005 and 815.06.11095 - made of high-quality, satin-finish crystal glass Article no.: HEWI 63502 |
| 63907 Pump head 63907 in product catalogue |  | Pump head - for soap dispenser 815.06.110, 815.06.11090, 900.06.00140 and 900.06.00160 - including riser tube - high-quality chrome-plated Article no.: HEWI 63907 |
| 63910 Pump head 63910 in product catalogue |  | Pump head - for soap dispenser 815.06.11045, 815.06.11065, 815.06.11005, 815.06.11095, 900.06.00040 and 900.06.00060 - including riser tube - high-quality chrome-plated Article no.: HEWI 63910 |
| 66988 Pump head 66988 in product catalogue |  | Pump head - for soap dispenser 900.06.000XA (glass) - including riser tube - stainless steel look Article no.: HEWI 66988 |
| 66989 Pump head 66989 in product catalogue | Similar picture | Pump head - for soap dispenser 900.06.001XA (polyamide) - including riser tube - stainless steel look Article no.: HEWI 66989 |
| 800.01.10060 Adjustable mirror white 800.01.10060 in product catalogue |  | System 800 Adjustable mirror - rectangular plate glass mirror with rounded edges, integrated shatter protection film and tilting frame - ergonomically shaped handle made of metal, high-quality chrome-plated - lean and functional with a harmonious, timeless design - tilting frame with high-quality white coating - easy adjustment of the tilt between 0 to 10° from a seated position - for wall mounting - 580 mm wide, 680 mm high and 115 mm deep - including non-corrosive HEWI fixing material Article no.: HEWI 800.01.10060 |
| 800.04.010 Tumbler with flat bottom 800.04.010 in product catalogue |  | System 800 K and Modular washbasins system 950.13.. Tumbler - cylindrical tumbler with flat bottom - made of high-quality polyamide according to HEWI colour chart - suitable for shelves with hole 950.13.00.. and holder 800.00.10090 and the product set 800.04.11041, 815.04.110, 815.04.11090, 162.04.110... - 69 mm diameter, 110 mm high Article no.: HEWI 800.04.010 |
| 800.06.E02 Container for soap dispenser 800.06.010 and product sets 800.06.11041, 162.06.110..., 162.06.119..., 900.06.001... 800.06.E02 in product catalogue | Similar picture | Container - for soap dispenser 800.06.010 and product-Sets 800.06.11041, 815.06.110, 815.06.11090, 162.06.110XA, 162.06.11040, 162.06.119XA, 162.06.11940, 900.06.00140, 900.06.00160 and 900.06.001XA - Diameter 69 mm, 113 mm high - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 800.06.E02 |
| 800.90.01090 Hook 800.90.01090 in product catalogue |  | System 800 K Hook - single hook - for hanging up towels and other pieces of clothing and other utensils - for wall mounting, concealed fixing - 50 mm deep - made of high-quality polyamide according to HEWI colour chart - including non-corrosive HEWI fixing material Article no.: HEWI 800.90.01090 |
| 800.90.02090 Hook 800.90.02090 in product catalogue |  | System 800 K Hook - single hook - for hanging up towels and other pieces of clothing and other utensils - for wall mounting, concealed fixing - 80 mm deep - made of high-quality polyamide according to HEWI colour chart - including non-corrosive HEWI fixing material Article no.: HEWI 800.90.02090 |
| 800.90.03091 Hook ø 50 mm 800.90.03091 in product catalogue |  | System 800 K Single hook - right-angled bend, cylindrical hook with fixing rose - for hanging up towels and other pieces of clothing and other utensils - anti-theft protection through concealed screw fixing - 45 mm deep, rose diameter 50 mm - made of high-quality polyamide - rose cap in HEWI colour 98 (signal white) - hook according to HEWI colour chart Article no.: HEWI 800.90.03091 |
| 800.90.03099 Hook ø 50 mm 800.90.03099 in product catalogue |  | System 800 K Single hook - right-angled bend, cylindrical hook with fixing rose - for hanging up towels and other pieces of clothing and other utensils - anti-theft protection through concealed screw fixing - 45 mm deep, rose diameter 50 mm - made of high-quality polyamide - rose cap in the HEWI colour 99 (pure white) - hook according to HEWI colour chart Article no.: HEWI 800.90.03099 |
| 800.90.04091 Double hook ø 50 mm 800.90.04091 in product catalogue |  | System 800 K Double hook - double hook with fixing rose - for hanging up towels and other pieces of clothing and other utensils - diameter rose 50 mm - 43.5 mm deep - made of high-quality polyamide - rose cap in HEWI colour 98 (signal white) - hook according to HEWI colour chart Article no.: HEWI 800.90.04091 |
| 800.90.04099 Double hook ø 50 mm 800.90.04099 in product catalogue |  | System 800 K Double hook - double hook with fixing rose - for hanging up towels and other pieces of clothing and other utensils - diameter rose 50 mm - 43.5 mm deep - made of high-quality polyamide - rose cap in the HEWI colour 99 (pure white) - hook according to HEWI colour chart Article no.: HEWI 800.90.04099 |
| 800.90.05091 Triple hook ø 50 mm 800.90.05091 in product catalogue |  | Triple hook System 800 K - rotatable hook for screwing on from underneath - for hanging up towels and other pieces of clothing and other utensils - rose diameter 50 mm, 70 mm high - anti-theft protection through concealed screw fixing - made of high-quality polyamide - rose cap in HEWI colour 98 (signal white) - hook in selected colours Article no.: HEWI 800.90.05091 |
| 800.90.05099 Triple hook ø 50 mm 800.90.05099 in product catalogue |  | Triple hook System 800 K - rotatable hook for screwing on from underneath - for hanging up towels and other pieces of clothing and other utensils - rose diameter 50 mm, 70 mm high - anti-theft protection through concealed screw fixing - made of high-quality polyamide - rose cap in HEWI colour 99 (pure white) - hook in selected colours Article no.: HEWI 800.90.05099 |
| 801.90.010 Coat hook ø 40 mm 801.90.010 in product catalogue |  | Range 801 Single hook - right-angled bend, cylindrical hook with fixing rose - for hanging up towels and other pieces of clothing and other utensils - anti-theft protection through concealed screw fixing - 30 mm deep, rose diameter 40 mm - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 801.90.010 |
| 801.90.020 Double hook ø 40 mm 801.90.020 in product catalogue |  | Range 801 Double hook - double hook with fixing rose - for hanging up towels and other pieces of clothing and other utensils - anti-theft protection through concealed screw fixing - 31 mm deep, rose diameter 40 mm - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 801.90.020 |
| 801.90.030 Double coat hook 801.90.030 in product catalogue |  | Range 801 Double hook - two polyamide support pins with a stirrup-shaped hook - for hanging up clothing and utensils - reduced risk of injury as hooks face backwards towards wall - 90 mm wide, 165 mm high and 95 mm deep, centre-to-centre 75 mm - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 801.90.030 |
| 801.90.040 Double coat hook 801.90.040 in product catalogue |  | Range 801 Double hook for cloakroom rails - double hook with forward-facing hooks - for hanging up clothing and utensils - 90 mm wide, 165 mm high and 87 mm deep, centre-to-centre 75 mm - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 801.90.040 |
| 801.90B030 Double coat hook Matt edition 801.90B030 in product catalogue |  | Range 801 Double hook - two polyamide support pins with a stirrup-shaped hook - for hanging up clothing and utensils - reduced risk of injury as hooks face backwards towards wall - 90 mm wide, 165 mm high and 95 mm deep, centre-to-centre 75 mm - made of high-quality matt polyamide according to HEWI colour chart Article no.: HEWI 801.90B030 |
| 801.90B040 Double coat hook Matt edition 801.90B040 in product catalogue |  | Range 801 Double hook for cloakroom rails - double hook with forward-facing hooks - for hanging up clothing and utensils - 90 mm wide, 165 mm high and 87 mm deep, centre-to-centre 75 mm - made of high-quality matt polyamide according to HEWI colour chart Article no.: HEWI 801.90B040 |
| 801.91B010 Symbol, male Matt edition 801.91B010 in product catalogue |  | Symbol male - self-adhesive - 150 mm high, 71.5 mm wide and 3 mm thick - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey), 92 (anthracite grey) and 90 (jet black) Article no.: HEWI 801.91B010 |
| 801.91B020 Symbol, female Matt edition 801.91B020 in product catalogue |  | Symbol female - self-adhesive - 150 mm high, 87 mm wide and 3 mm thick - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey), 92 (anthracite grey) and 90 (jet black) Article no.: HEWI 801.91B020 |
| 802.01.100L Adjustable mirror Grip bar left 802.01.100L in product catalogue |  | Range 802 LifeSystem Adjustable mirror (lever on left) - rectangular crystal glass mirror with rounded edges and integrated anti-splinter film - easy adjustment of the tilt between 0 to 28° from a seated position - lever on left - 725 mm wide, 741 mm high, mirror 73 mm deep, clear distance between wall and handle 129 mm - gripping rail made of highly polished chrome-plated aluminium, axle made of powder coated, matt aluminium - with ergonomic handle made of polyamide according to HEWI colour chart - including non-corrosive HEWI fixing material. Article no.: HEWI 802.01.100L |
| 802.01.100R Adjustable mirror Grip bar right 802.01.100R in product catalogue |  | Range 802 LifeSystem Adjustable mirror (lever on right) - rectangular crystal glass mirror with rounded edges and integrated anti-splinter film - easy adjustment of the tilt between 0 to 28° from a seated position - lever on right - 725 mm wide, 741 mm high, mirror 73 mm deep, clear distance between wall and handle 129 mm - gripping rail made of highly polished chrome-plated aluminium, axle made of powder coated, matt aluminium - with ergonomic handle made of polyamide according to HEWI colour chart - including non-corrosive HEWI fixing material. Article no.: HEWI 802.01.100R |
| 900.00.00040 Hair dryer holder chrome 900.00.00040 in product catalogue |  | Hair dryer holder System 900 - holder made of high-quality stainless steel chrome-plated - With insert to protect against damage to the hairdryer - Insert made of high-quality synthetic material in HEWI colours 92 (anthracite grey) and 98 (signal-white) - inside 78 mm high, 37 mm wide and 104 mm deep - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.00.00040 |
| 900.00.00060 Hair dryer holder powder-coated 900.00.00060 in product catalogue |  | Hair dryer holder System 900 - holder made of high-quality stainless steel powder-coated in the HEWI colour CV (matt black) and ER (matt dark grey pearl mica) with insert in matt black polyamide or powder-coated in the HEWI colours AS (matt white) and AX (matt light grey pearl mica) with insert in matt white polyamide - With insert to protect against damage to the hairdryer - inside 78 mm high, 37 mm wide and 104 mm deep - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.00.00060 |
| 900.00.000XA Hair dryer holder satin 900.00.000XA in product catalogue |  | Hair dryer holder System 900 - holder made of high-quality satin stainless steel - with insert to protect against damage to the hairdryer - insert made of high-quality synthetic material in HEWI colours 92 (anthracite grey) and 98 (signal-white) - inside 78 mm high, 37 mm wide and 104 mm deep - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.00.000XA |
| 900.03.00040 Shelf with grab bar chrome 900.03.00040 in product catalogue |  | Shelf with Support rail - shelf with integrated support rail at the front - continuous mirror solution possible - holder made of high-quality stainless steel chrome-plated - Glass plate with anti-splinter film, white back-painted - 720 mm wide, 70 mm high, 188 mm deep - diameter support rail 25 mm - including non-corrosive HEWI fixing material Article no.: HEWI 900.03.00040 |
| 900.03.00060 Shelf with grab bar powder-coated 900.03.00060 in product catalogue |  | Shelf with Support rail - shelf with integrated support rail at the front - continuous mirror solution possible - holder made of high-quality stainless steel powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - Glass plate with anti-splinter film, white back-painted - 720 mm wide, 70 mm high, 188 mm deep - diameter support rail 25 mm - including non-corrosive HEWI fixing material Article no.: HEWI 900.03.00060 |
| 900.03.000XA Shelf with grab bar satin 900.03.000XA in product catalogue |  | Shelf with support rail - shelf with integrated support rail at the front - continuous mirror solution possible - holder made of high-quality satin stainless steel - glass plate with anti-splinter film, white back-painted - 720 mm wide, 70 mm high, 188 mm deep - diameter support rail 25 mm - including non-corrosive HEWI fixing material Article no.: HEWI 900.03.000XA |
| 900.04.00040 Glass tumbler with holder chrome 900.04.00040 in product catalogue |  | System 900 Tumbler with holder - cylindrical wall holder - holder made of metal, high-quality chrome-plated - holder with protective film on the inside for a secure, defined position and to protect the glass insert from damage - 124 mm high, 85 mm deep, diameter 70 mm - cylindrical tumbler made of high-quality, satin-finish crystal glass - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.04.00040 |
| 900.04.00060 Glass tumbler with holder powder-coated 900.04.00060 in product catalogue |  | System 900 Tumbler with holder - cylindrical wall holder - holder made of high-quality satin stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - holder with protective film on the inside for a secure, defined position and to protect the glass insert from damage - 124 mm high, 85 mm deep, diameter 70 mm - cylindrical tumbler made of high-quality, satin-finish crystal glass - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.04.00060 |
| 900.04.000XA Glass tumbler with holder satin 900.04.000XA in product catalogue |  | System 900 Tumbler with holder - cylindrical wall holder - holder made of high-quality satin stainless steel - holder with protective film on the inside for a secure, defined position and to protect the glass insert from damage - 70 mm diameter, 124 mm high and 85 mm deep - cylindrical tumbler made of high-quality, satin-finish crystal glass - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.04.000XA |
| 900.05.00060 Waste bin powder-coated 900.05.00060 in product catalogue |  | Hygiene waste bin System 900 - hygiene waste bin with hinged lid - the waste bin is detachable - the lid can be opened by lightly pressing the knee against the bin - with sticker to explain operation by knee - invisible, integrated bag holder - lid with soft-close - only suitable for wall mounting - capacity approx. 6 l - 310 mm high, 145 mm deep and 205 mm wide - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including corrosion-free HEWI fixing material Article no.: HEWI 900.05.00060 |
| 900.05.000XA Waste bin satin 900.05.000XA in product catalogue |  | Hygiene waste bin System 900 - hygiene waste bin with hinged lid - the waste bin is detachable - the lid can be opened by lightly pressing the knee against the bin - with sticker to explain operation by knee - invisible, integrated bag holder - lid with soft-close - only suitable for wall mounting - capacity approx. 6 l - 310 mm high, 145 mm deep and 205 mm wide - made of high-quality stainless steel, satin surface - including corrosion-free HEWI fixing material Article no.: HEWI 900.05.000XA |
| 900.05.00160 Wastepaper bin powder-coated 900.05.00160 in product catalogue |  | Wastepaper bin from the System 900 range - angular base body - serves as a wastebin for used paper towels - capacity approx. 25 l - invisible, integrated bag holder with pull-out function for easy insertion and removal of the bag - 260 mm wide, 500 mm high and 203 mm deep - for wall mounting - made of high-quality stainless steel, powder coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including corrosion-free HEWI fixing material Article no.: HEWI 900.05.00160 |
| 900.05.001XA Wastepaper bin satin 900.05.001XA in product catalogue |  | Wastepaper bin from the System 900 range - angular base body - serves as a wastebin for used paper towels - capacity approx. 25 l - invisible, integrated bag holder with pull-out function for easy insertion and removal of the bag - 260 mm wide, 500 mm high and 203 mm deep - for wall mounting - made of high-quality stainless steel, matt satin finish surface - including corrosion-free HEWI fixing material Article no.: HEWI 900.05.001XA |
| 900.05.00260 Wastepaper bin powder-coated 900.05.00260 in product catalogue |  | Wastepaper bin from the System 900 range - angular base body - serves as a wastebin for used paper towels - capacity approx. 25 l - invisible, integrated bag holder with pull-out function for easy insertion and removal of the bag - lid with lifting tab - 260 mm wide, 500 mm high and 203 mm deep - for wall mounting - made of high-quality stainless steel, powder coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including corrosion-free HEWI fixing material Article no.: HEWI 900.05.00260 |
| 900.05.002XA Wastepaper bin satin 900.05.002XA in product catalogue |  | Wastepaper bin from the System 900 range - angular base body - serves as a wastebin for used paper towels - capacity approx. 25 l - invisible, integrated bag holder with pull-out function for easy insertion and removal of the bag - lid with lifting tab - 260 mm wide, 500 mm high and 203 mm deep - for wall mounting - made of high-quality stainless steel, matt satin finish surface - including corrosion-free HEWI fixing material Article no.: HEWI 900.05.002XA |
| 900.05.00360 Wastepaper bin powder-coated 900.05.00360 in product catalogue |  | Wastepaper bin from the System 900 range - angular base body - serves as a wastebin for used paper towels - capacity approx. 60 l - invisible, integrated bag holder with pull-out function for easy insertion and removal of the bag - 340 mm wide, 620 mm high and 263 mm deep - for wall mounting - made of high-quality stainless steel, powder coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including corrosion-free HEWI fixing material Article no.: HEWI 900.05.00360 |
| 900.05.003XA Wastepaper bin satin 900.05.003XA in product catalogue |  | Wastepaper from the System 900 range - angular base body - serves as a wastebin for used paper towels - capacity approx. 60 l - invisible, integrated bag holder with pull-out function for easy insertion and removal of the bag - 340 mm wide, 620 mm high and 263 mm deep - for wall mounting - made of high-quality stainless steel, matt satin finish surface - including corrosion-free HEWI fixing material Article no.: HEWI 900.05.003XA |
| 900.05.00460 Wastepaper bin powder-coated 900.05.00460 in product catalogue |  | Wastepaper bin from the System 900 range - angular base body - serves as a wastebin for used paper towels - capacity approx. 60 l - invisible, integrated bag holder with pull-out function for easy insertion and removal of the bag - lid with lifting tab - 340 mm wide, 620 mm high and 263 mm deep - for wall mounting - made of high-quality stainless steel, powder coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including corrosion-free HEWI fixing material Article no.: HEWI 900.05.00460 |
| 900.05.004XA Wastepaper bin satin 900.05.004XA in product catalogue |  | Wastepaper bin from the System 900 range - angular base body - serves as a wastebin for used paper towels - capacity approx. 60 l - invisible, integrated bag holder with pull-out function for easy insertion and removal of the bag - lid with lifting tab - 340 mm wide, 620 mm high and 263 mm deep - for wall mounting - made of high-quality stainless steel, matt satin finish surface - including corrosion-free HEWI fixing material Article no.: HEWI 900.05.004XA |
| 900.06.00040 Soap dispenser with holder chrome 900.06.00040 in product catalogue |  | System 900 Soap dispenser with holder - suitable for use at the washbasin area - cylindrical wall holder - holder made of metal, high-quality chrome-plated - holder with protective film on the inside for a secure, defined position and to protect the glass insert from damage - 180 mm high, 84 mm deep, diameter 71 mm - soap dispenser made of high-quality, satin-finish crystal glass - high-quality chrome-plated pump head - capacity approx. 200 ml - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.06.00040 |
| 900.06.00060 Soap or disinfectant dispenser with holder powder-coated 900.06.00060 in product catalogue |  | System 900 Soap or disinfectant dispenser with holder - suitable for use at the washbasin area - cylindrical wall holder - holder made of high-quality satin stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - holder with protective film on the inside for a secure, defined position and to protect the glass insert from damage - 180 mm high, 85 mm deep, diameter 70 mm - soap dispenser made of high-quality, satin-finish crystal glass - high-quality chrome-plated pump head - capacity approx. 200 ml - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.06.00060 |
| 900.06.000XA Soap dispenser with holder satin 900.06.000XA in product catalogue |  | System 900 Soap dispenser with holder - suitable for use at the washbasin area - cylindrical wall holder - holder made of high-quality stainless steel, satin surface finish - holder with protective film on the inside for a secure, defined position and to protect the glass insert from damage - 70 mm diameter, 180 mm high and 85 mm deep - soap dispenser made of high-quality, satin-finish crystal glass - pump head in stainless steel finish - capacity approx. 200 ml - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.06.000XA |
| 900.06.00140 Soap dispenser with holder chrome 900.06.00140 in product catalogue |  | System 900 Soap dispenser with holder - suitable for use at the washbasin area - cylindrical wall holder - holder made of metal, high-quality chrome-plated - holder with protective film on the inside for a secure, defined position and to protect the insert from damage - 180 mm high, 84 mm deep, diameter 71 mm - soap dispenser made of high-quality polyamide in HEWI colours 92 (anthracite grey) and 98 (signal white) - high-quality chrome-plated pump head - capacity approx. 200 ml - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.06.00140 |
| 900.06.00160 Soap or disinfectant dispenser with holder powder-coated 900.06.00160 in product catalogue |  | System 900 Soap or disinfectant dispenser with holder - suitable for use at the washbasin area - cylindrical wall holder - holder made of metal, powder-coated in the HEWI colour CV (matt black) and ER (matt dark grey pearl mica) with soap dispenser in matt black polyamide or powder-coated in the HEWI colours AS (matt white) and AX (matt light grey pearl mica) with soap dispenser in matt white polyamide - holder with protective film on the inside for a secure, defined position and to protect the insert from damage - 180 mm high, 85 mm deep, diameter 70 mm - high-quality chrome-plated pump head - capacity approx. 200 ml - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.06.00160 |
| 900.06.001XA Soap dispenser with holder satin 900.06.001XA in product catalogue |  | System 900 Soap dispenser with holder - suitable for use at the washbasin area - cylindrical wall holder - holder made of stainless steel, satin surface finish - holder with protective film on the inside for a secure, defined position and to protect the insert from damage - 180 mm high, 85 mm deep, diameter 70 mm - soap dispenser made of high-quality polyamide in HEWI colours 92 (anthracite grey) and 98 (signal white) - pump head in stainless steel finish - capacity approx. 200 ml - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.06.001XA |
| 900.06.00260 Disinfectant or soap dispenser powder-coated 900.06.00260 in product catalogue |  | Disinfection or soap dispenser 1.000 ml - basic body made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - Front cover made of high-quality stainless steel, powder coated, with viewing window - for dispensing controlled quantities of alcohol-based hand disinfectants or liquid soaps - for 1.000 ml Euro standard bottles - the disposable bottle are easy to replace from the front - dispenser with long lever, lockable - dosing quantity adjustable in several steps: 0.7 ml, 1.0 ml, 1.2 ml, 1.5 ml (depending on the viscosity of the product) - dosing pump made of stainless steel - compatible with hygienic packaging (collapsing bottle with disposable pump) - dispenser and pump dishwasher safe and autoclavable up to 134 °C, 3 bar - 92.5 mm wide, 388 mm high, 207 mm deep - incl. 1.000 ml empty container for free refilling - including non-corrosive HEWI fixing material Article no.: HEWI 900.06.00260 |
| 900.06.00261 Disinfectant or soap dispenser 900.06.00261 in product catalogue |  | Disinfection and soap dispensers 1.000 ml - body made of high-quality stainless steel, powder coated white - front cover with viewing window - for dispensing controlled quantities of alcohol-based hand disinfectants or liquid soaps - for 1.000 ml Euro standard bottles - the disposable bottle are easy to replace from the front - dispenser with long lever, lockable - dosing quantity adjustable in several steps: 0.7 ml, 1.0 ml, 1.2 ml, 1.5 ml (depending on the viscosity of the product) - dosing pump made of stainless steel - compatible with hygienic packaging (collapsing bottle with disposable pump) - dispenser and pump dishwasher safe and autoclavable up to 134 °C, 3 bar - 92.5 mm wide, 388 mm high, 207 mm deep - incl. 1.000 ml empty container for free refilling - including non-corrosive HEWI fixing material Article no.: HEWI 900.06.00261 |
| 900.06.002XA Disinfectant or soap dispenser satin 900.06.002XA in product catalogue |  | Disinfection and soap dispensers 1.000 ml - basic body made of high-quality stainless steel, matt - front cover made of high-quality stainless steel, powder coated white, with viewing window - for dispensing controlled quantities of alcohol-based hand disinfectants or liquid soaps - for 1.000 ml Euro standard bottles - the disposable bottle are easy to replace from the front - dispenser with long lever, lockable - dosing quantity adjustable in several steps: 0.7 ml, 1.0 ml, 1.2 ml, 1.5 ml (depending on the viscosity of the product) - dosing pump made of stainless steel - compatible with hygienic packaging (collapsing bottle with disposable pump) - dispenser and pump dishwasher safe and autoclavable up to 134 °C, 3 bar - 92.5 mm wide, 388 mm high, 207 mm deep - incl. 1.000 ml empty container for free refilling - including non-corrosive HEWI fixing material Article no.: HEWI 900.06.002XA |
| 900.06.00360 Disinfectant or soap dispenser powder-coated 900.06.00360 in product catalogue |  | Disinfection and soap dispensers 500 ml - basic body made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - front cover made of high-quality stainless steel, powder coated, with viewing window - for dispensing controlled quantities of alcohol-based hand disinfectants or liquid soaps - for 500 ml Euro standard bottles - the disposable bottle are easy to replace from the front - dispenser with long lever, lockable - dosing quantity adjustable in several steps: 0.7 ml, 1.0 ml, 1.2 ml, 1.5 ml (depending on the viscosity of the product) - dosing pump made of stainless steel - compatible with hygienic packaging (collapsing bottle with disposable pump) - dispenser and pump dishwasher safe and autoclavable up to 134 °C, 3 bar - 80.5 mm wide, 342 mm high, 203 mm deep - incl. 500 ml empty container for free refilling - including non-corrosive HEWI fixing material Article no.: HEWI 900.06.00360 |
| 900.06.00361 Disinfectant or soap dispenser 900.06.00361 in product catalogue |  | Disinfection and soap dispensers 500 ml - basic body made of high-quality stainless steel, matt - front cover made of high-quality stainless steel, powder coated white, with viewing window - for dispensing controlled quantities of alcohol-based hand disinfectants or liquid soaps - for 500 ml Euro standard bottles - the disposable bottle are easy to replace from the front - dispenser with long lever, lockable - dosing quantity adjustable in several steps: 0.7 ml, 1.0 ml, 1.2 ml, 1.5 ml (depending on the viscosity of the product) - dosing pump made of stainless steel - compatible with hygienic packaging (collapsing bottle with disposable pump) - dispenser and pump dishwasher safe and autoclavable up to 134 °C, 3 bar - 80.5 mm wide, 342 mm high, 203 mm deep - incl. 500 ml empty container for free refilling - including non-corrosive HEWI fixing material Article no.: HEWI 900.06.00361 |
| 900.06.003XA Disinfectant or soap dispenser satin 900.06.003XA in product catalogue |  | Disinfection and soap dispensers 500 ml - basic body made of high-quality stainless steel, matt - front cover made of high-quality stainless steel, powder coated white, with viewing window - for dispensing controlled quantities of alcohol-based hand disinfectants or liquid soaps - for 500 ml Euro standard bottles - the disposable bottle are easy to replace from the front - dispenser with long lever, lockable - dosing quantity adjustable in several steps: 0.7 ml, 1.0 ml, 1.2 ml, 1.5 ml (depending on the viscosity of the product) - dosing pump made of stainless steel - compatible with hygienic packaging (collapsing bottle with disposable pump) - dispenser and pump dishwasher safe and autoclavable up to 134 °C, 3 bar - 80.5 mm wide, 342 mm high, 203 mm deep - incl. 500 ml empty container for free refilling - including non-corrosive HEWI fixing material Article no.: HEWI 900.06.003XA |
| 900.06.00460 Soap dispenser powder-coated 900.06.00460 in product catalogue |  | Soap dispenser from the System 900 range - angular base body - serves to hold standard liquid soap - with inner container for free filling, filling capacity 600 ml - inner container can be removed for cleaning - level indicator at the side - protection against misuse by locking device - 102 mm wide, 270 mm high and 98 mm deep - for wall mounting - made of high-quality stainless steel, powder coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including corrosion-free HEWI fixing material Article no.: HEWI 900.06.00460 |
| 900.06.004XA Soap dispenser satin 900.06.004XA in product catalogue |  | Soap dispenser from the System 900 range - angular base body - serves to hold standard liquid soap - with inner container for free filling, filling capacity 600 ml - inner container can be removed for cleaning - level indicator at the side - protection against misuse by locking device - 102 mm wide, 270 mm high and 98 mm deep - for wall mounting - made of high-quality stainless steel, matt satin finish surface - including corrosion-free HEWI fixing material Article no.: HEWI 900.06.004XA |
| 900.06.00560 HEWI SENSORIC Electronic soap dispenser powder-coated 900.06.00560 in product catalogue |  | Electronic liquid soap dispenser from the System 900 range - angular base body - serves to hold standard liquid soap - with inner container for free filling, filling capacity 500 ml - inner container can be removed for cleaning - level indicator at the side - contact-free soap dispensing - enclosed sticker to identify the dispenser (if required to distinguish between soap, foam soap and disinfectant dispensers) - illumination of the output area when the sensor is activated (blue light) - with battery change indicator - protection against misuse by locking device - maintenance-free - 102 mm wide, 270 mm high and 98 mm deep - for wall mounting - made of high-quality stainless steel, powder coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including corrosion-free HEWI fixing material - batteries (AA 1.5 V / 4 pcs.) not included in the scope of supply - CE marking in accordance with EMC Directive 2014/30/EU Article no.: HEWI 900.06.00560 |
| 900.06.005XA HEWI SENSORIC Electronic soap dispenser satin 900.06.005XA in product catalogue |  | Electronic liquid soap dispenser from the System 900 range - angular base body - serves to hold standard liquid soap - with inner container for free filling, filling capacity 500 ml - inner container can be removed for cleaning - level indicator at the side - contact-free soap dispensing - enclosed sticker to identify the dispenser (if required to distinguish between soap, foam soap and disinfectant dispensers) - illumination of the output area when the sensor is activated (blue light) - with battery change indicator - protection against misuse by locking device - maintenance-free - 102 mm wide, 270 mm high and 98 mm deep - for wall mounting - made of high-quality stainless steel, matt satin finish surface - including corrosion-free HEWI fixing material - batteries (AA 1.5 V / 4 pcs.) not included in the scope of supply - CE marking in accordance with EMC Directive 2014/30/EU Article no.: HEWI 900.06.005XA |
| 900.06.00760 HEWI SENSORIC Electronic foamed soap dispenser powder-coated 900.06.00760 in product catalogue |  | Electronic foamed soap dispenser from the System 900 range - angular base body - serves to hold standard foamed soap - with inner container for free filling, filling capacity 500 ml - inner container can be removed for cleaning - level indicator at the side - contact-free foamed soap dispensing - enclosed sticker to identify the dispenser (if required to distinguish between soap, foam soap and disinfectant dispensers) - illumination of the output area when the sensor is activated (blue light) - with battery change indicator - protection against misuse by locking device - maintenance-free - 102 mm wide, 270 mm high and 98 mm deep - for wall mounting - made of high-quality stainless steel, powder coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including corrosion-free HEWI fixing material - batteries (AA 1.5 V / 4 pcs.) not included in the scope of supply - CE marking in accordance with EMC Directive 2014/30/EU Article no.: HEWI 900.06.00760 |
| 900.06.007XA HEWI SENSORIC Electronic foamed soap dispenser satin 900.06.007XA in product catalogue |  | Electronic foamed soap dispenser from the System 900 range - angular base body - serves to hold standard foamed soap - with inner container for free filling, filling capacity 500 ml - inner container can be removed for cleaning - level indicator at the side - contact-free foamed soap dispensing - enclosed sticker to identify the dispenser (if required to distinguish between soap, foam soap and disinfectant dispensers) - illumination of the output area when the sensor is activated (blue light) - with battery change indicator - protection against misuse by locking device - maintenance-free - 102 mm wide, 270 mm high and 98 mm deep - for wall mounting - made of high-quality stainless steel, matt satin finish surface - including corrosion-free HEWI fixing material - batteries (AA 1.5 V / 4 pcs.) not included in the scope of supply - CE marking in accordance with EMC Directive 2014/30/EU Article no.: HEWI 900.06.007XA |
| 900.06.00860 HEWI SENSORIC Disinfectant dispenser powder-coated 900.06.00860 in product catalogue |  | Electronic disinfectant dispenser from the System 900 range - angular base body - serves to hold standard disinfectant - with inner container for free filling, filling quantity 500 ml - inner container can be removed for cleaning - level indicator at the side - contact-free disinfectant dispensing - disinfectant is dispensed as a spray - enclosed sticker to identify the dispenser (if required to distinguish between soap, foam soap and disinfectant dispensers) - illumination of the disinfection area when the sensor is activated (blue light) - with battery change indicator - protection against misuse by locking device - maintenance-free - 102 mm wide, 270 mm high and 98 mm deep - for wall mounting - made of high-quality stainless steel, powder coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including corrosion-free HEWI fixing material - batteries (AA 1.5 V / 4 pcs.) not included in the scope of supply - CE marking in accordance with EMC Directive 2014/30/EU Article no.: HEWI 900.06.00860 |
| 900.06.008XA HEWI SENSORIC Disinfectant dispenser satin 900.06.008XA in product catalogue |  | Electronic disinfectant dispenser from the System 900 range - angular base body - serves to hold standard disinfectant - with inner container for free filling, filling capacity 500 ml - inner container can be removed for cleaning - level indicator at the side - non-contact disinfectant dispensing - disinfectant is dispensed as a spray - enclosed sticker to identify the dispenser (if required to distinguish between soap, foam soap and disinfectant dispensers) - illumination of the disinfection area when the sensor is activated (blue light) - with battery change indicator - protection against misuse by locking device - maintenance-free - 102 mm wide, 270 mm high and 98 mm deep - for wall mounting - made of high-quality stainless steel, matt satin finish surface - including corrosion-free HEWI fixing material - batteries (AA 1.5 V / 4 pcs.) not included in the scope of supply - CE marking in accordance with EMC Directive 2014/30/EU Article no.: HEWI 900.06.008XA |
| 900.06.00960 Paper towel dispenser powder-coated 900.06.00960 in product catalogue |  | Paper towel dispenser from the System 900 range - angular base body - serves to hold standard paper towels with different folds in the width 235 - 250 mm and folded length / depth of 80 - 110 mm (HEWI recommends paper towels with Z-fold) - filling quantity approx. 300 - 450 paper towels - level indicator at the side - protection against misuse by locking device - 260 mm wide, 350 mm high and 140 mm deep - for wall mounting - made of high-quality stainless steel, powder coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including corrosion-free HEWI fixing material Article no.: HEWI 900.06.00960 |
| 900.06.009XA Paper towel dispenser satin 900.06.009XA in product catalogue |  | Paper towel dispenser from the System 900 range - angular basic body - serves to hold standard paper towels of various folds in the 235 - 250 mm and folded length / depth of 80 - 110 mm (HEWI recommends paper towels with Z-fold) - filling capacity approx. 300 - 450 paper towels - level indicator at the side - protection against misuse by locking device - 260 mm wide, 350 mm high and 140 mm deep - for wall mounting - made of high-quality stainless steel, matt satin finish surface - including corrosion-free HEWI fixing material Article no.: HEWI 900.06.009XA |
| 900.06.01060 Electronic paper roll dispenser powder-coated 900.06.01060 in product catalogue |  | Electronic paper roll dispenser from the System 900 range - angular base body - serves to hold standard paper rolls with external unwinding, max. roll width 210 mm, max. roll diameter 200 mm, sleeve diameter 38 mm - level indicator at the side - easy paper roll replacement - with battery change indicator - automatic paper output at the washbasin after the paper has been torn off - integrated tear-off edge - protection against misuse through locking device - maintenance-free - 284 mm wide, 405 mm high and 210 mm deep - for wall mounting - made of high-quality stainless steel, powder coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including corrosion-free HEWI fixing material - batteries (D - Mono - LR20 - 1.5 V / 4 pcs.) not included in the scope of supply - can be converted to mains connection 950.06.E01 - CE marking in accordance with EU Directive 2015/863/EC Article no.: HEWI 900.06.01060 |
| 900.06.010XA Electronic paper roll dispenser satin 900.06.010XA in product catalogue |  | Electronic paper roll dispenser from the System 900 range - angular base body - serves to hold standard paper rolls with external unwinding, max. roll width 210 mm, max. roll diameter 200 mm, sleeve diameter 38 mm - level indicator at the side - easy paper roll replacement - with battery change indicator - automatic paper output at the washbasin after the paper has been torn off - integrated tear-off edge - protection against misuse by locking device - maintenance-free - 284 mm wide, 405 mm high and 210 mm deep - for wall mounting - made of high-quality stainless steel, matt satin finish surface - including corrosion-free HEWI fixing material - batteries (D - Mono - LR20 - 1.5 V / 4 pcs.) not included in the scope of supply - can be converted to mains connection 950.06.E01 - CE marking in accordance with EU Directive 2015/863/EC Article no.: HEWI 900.06.010XA |
| 900.06.01260 Disinfectant dispenser column powder-coated 900.06.01260 in product catalogue |  | Disinfectant dispenser column - comprising disinfectant dispenser and column - column with angular base body - made of aluminium with steel base, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - contains the disinfectant dispenser 900.06.00260 - base body of the dispenser made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - front trim made of high-quality stainless steel, powder-coated, with viewing window - for dispensing hand disinfectants containing alcohol or liquid soaps - for 1.000 ml Euro standard bottles - simple replacement of the disposable bottom from the front - dispenser with long operating lever, lockable - dispensing quantity can be set in several steps: 0.7 ml, 1.0 ml, 1.2 ml, 1.5 ml (depending on the viscosity of the contents) - dispensing pump made of stainless steel - compatible with hygiene packaging requirements (collapsible bottle with disposable pump) - dispenser and pump dishwater-safe and autoclavable up to 134 °C, 3 bar - incl. 1.000 ml empty container for free refilling - set: 1453 mm high, 454 mm deep, 400 mm wide, weight: approx. 15 kg - including corrosion-free HEWI fixing material Article no.: HEWI 900.06.01260 |
| 900.06.012XA Disinfectant dispenser column satin 900.06.012XA in product catalogue |  | Disinfectant dispenser column - comprising disinfectant dispenser and column - column with angular base body - made of high-quality stainless steel satin finished - contains the disinfectant dispenser 900.06.002XA - base body of the dispenser made of high-quality stainless steel satin finished - front trim made of high-quality stainless steel, white powder-coated, with viewing window - for dispensing hand disinfectants containing alcohol or liquid soaps - for 1.000 ml Euro standard bottles - simple replacement of the disposable bottom from the front - dispenser with long operating lever, lockable - dispensing quantity can be set in several steps: 0.7 ml, 1.0 ml, 1.2 ml, 1.5 ml (depending on the viscosity of the contents) - dispensing pump made of stainless steel - compatible with hygiene packaging requirements (collapsible bottle with disposable pump) - dispenser and pump dishwater-safe and autoclavable up to 134 °C, 3 bar - incl. 1.000 ml empty container for free refilling - set: 1453 mm high, 454 mm deep, 400 mm wide, weight: approx. 21 kg - including corrosion-free HEWI fixing material Article no.: HEWI 900.06.012XA |
| 900.06.01360 HEWI SENSORIC Disinfectant dispenser column powder-coated 900.06.01360 in product catalogue |  | Disinfectant dispenser column - comprising disinfectant dispenser, drip tray and column - column with angular base body - made of aluminium with steel base, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - contains the disinfectant dispenser 900.06.00860 - base body of the dispenser made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pear mica) and SC (matt dark grey pearl mica) - dispenser serves to hold commercially available disinfectant - with inner container for free filling, filling quantity 500 ml - inner container can be removed for cleaning - lateral filling level indicator - non-contact disinfectant dispensing - disinfectant is dispensed as a spray - enclosed sticker to identify the dispenser (if required to distinguish between soap, foam soap and disinfectant dispensers) - illumination of the disinfection area when the sensor is activated (blue light) - with battery change indicator - protection against misuse by locking mechanism - maintenance-free - drip tray made of high-quality stainless steel, powder coated in the respective dispenser colour - serves to collect dripping disinfectant - set: 1330 mm high, 400 mm deep, 400 mm wide, weight: approx. 15 kg - including corrosion-free HEWI fixing material - batteries (AA 1.5 V / 4 pieces) not included in the scope of delivery - CE mark according to EMC Directive 2014/30/EU Article no.: HEWI 900.06.01360 |
| 900.06.013XA HEWI SENSORIC Disinfectant dispenser column satin 900.06.013XA in product catalogue |  | Disinfectant dispenser column - comprising disinfectant dispenser, drip tray and column - column with angular base body - made of high-quality stainless steel satin finished - contains the disinfectant dispenser 900.06.008XA - base body of the dispenser made of high-quality stainless steel satin finished - dispenser serves to hold commercially available disinfectant - with inner container for free filling, filling quantity 500 ml - inner container can be removed for cleaning - lateral filling level indicator - non-contact disinfectant dispensing - disinfectant is dispensed as a spray - enclosed sticker to identify the dispenser (if required to distinguish between soap, foam soap and disinfectant dispensers) - illumination of the disinfection area when the sensor is activated (blue light) - with battery change indicator - protection against misuse by locking mechanism - maintenance-free - drip tray made of high-quality stainless steel, powder-coated in the HEWI colour AY (matt light grey pearl mica) - serves to collect dripping disinfectant - set: 1330 mm high, 400 mm deep, 400 mm wide, weight: approx. 21 kg - including corrosion-free HEWI fixing material - batteries (AA 1.5 V / 4 pieces) not included in the scope of delivery - CE mark according to EMC Directive 2014/30/EU Article no.: HEWI 900.06.013XA |
| 900.09.00140 Towel rail chrome 900.09.00140 in product catalogue |  | Towel rail System 900 - towel rail with a cylindrical holding arm, fixed - for hanging up towels and other items of clothing - made of stainless steel, chrome-plated - 445 mm long 51 mm high, diameter holding arm 15 mm - for wall mounting, concealed fixing - incl. corrosion-free HEWI fixing material Article no.: HEWI 900.09.00140 |
| 900.09.00160 Towel rail powder-coated 900.09.00160 in product catalogue |  | Towel rail System 900 - towel rail with a cylindrical holding arm, fixed - for hanging up towels and other items of clothing - made of high-quality stainless steel, powder-coated in the HEWI colours DX (white deep matt), DC (black deep matt), AY (light grey pearl mica deep matt) and SC (dark grey pearl mica deep matt) - 445 mm long 51 mm high, diameter holding arm 15 mm - for wall mounting, concealed fixing - incl. corrosion-free HEWI fixing material Article no.: HEWI 900.09.00160 |
| 900.09.001XA Towel rail satin 900.09.001XA in product catalogue |  | Towel rail System 900 - towel rail with a cylindrical holding arm, fixed - for hanging up towels and other items of clothing - made of stainless steel, satin finished - 445 mm long 51 mm high, diameter holding arm 15 mm - for wall mounting, concealed fixing - incl. corrosion-free HEWI fixing material Article no.: HEWI 900.09.001XA |
| 900.09.00240 Towel ring chrome 900.09.00240 in product catalogue |  | Towel ring System 900 - closed towel ring, rectangular - serves to hang up towels and other items of clothing - the ring can be swivelled - made of high-quality stainless steel, chrome-plated - 230 mm wide, 130 mm high, ø 15 mm - for wall mounting, concealed fixing - including corrosion-free HEWI fixing material Article no.: HEWI 900.09.00240 |
| 900.09.00260 Towel ring powder-coated 900.09.00260 in product catalogue |  | Towel ring System 900 - closed towel ring, rectangular - for hanging up towels and other items of clothing - the ring can be swivelled - made of high-quality stainless steel, powder-coated in the HEWI colours DX (white deep matt), DC (black deep matt), AY (light grey pearl mica deep matt) and SC (dark grey pearl mica deep matt) - 230 mm wide, 130 mm high, ø 15 mm - for wall mounting, concealed fixing - including corrosion-free HEWI fixing material Article no.: HEWI 900.09.00260 |
| 900.09.002XA Towel ring satin 900.09.002XA in product catalogue |  | Towel ring System 900 - closed towel ring, rectangular - serves to hang up towels and other items of clothing - the ring can be swivelled - made of high-quality stainless steel, satin finished - 230 mm wide, 130 mm high, ø 15 mm - for wall mounting, concealed fixing - including corrosion-free HEWI fixing material Article no.: HEWI 900.09.002XA |
| 900.90.00140 Walking aid holder chrome 900.90.00140 in product catalogue |  | Walking aid holder system 900 - made of high-quality stainless steel, chrome - 120 mm wide, 35 mm high, 135 mm deep - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.90.00140 |
| 900.90.00160 Walking aid holder powder-coated 900.90.00160 in product catalogue |  | Walking aid holder system 900 - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - 120 mm wide, 35 mm high, 135 mm deep - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.90.00160 |
| 900.90.001XA Walking aid holder satin 900.90.001XA in product catalogue |  | Walking aid holder system 900 - made of high-quality stainless steel, satin finish - 120 mm wide, 35 mm high, 135 mm deep - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.90.001XA |
| 950.01.110 Mirror 570 mm wide, 1000 mm high, 6 mm deep 950.01.110 in product catalogue |  | Mirror - rectangular mirror with large radii and coloured backpainted edge - asymmetrical rear painting in HEWI colour 98 (signal white) - concealed fixing - 570 mm wide, 1000 mm high, mirror 6 mm thick - mirror surface 500 mm wide, 930 mm high - mirror made of high-quality float glass - fulfils the DIN 18040, ÖNORM B1600/1601 and SIA 500 requirements Article no.: HEWI 950.01.110 |
| 950.01.225 Cosmetic mirror 950.01.225 in product catalogue |  | Make-up mirror - round, unilluminated - double-sided mirror, both sides mirrored, rotatable - set in a chrome-plated metal frame - one side concave, one side flat - 3-fold magnification - mirror diameter 200 mm, mirror surface diameter 190 mm - on double-hinged arm, mounting allows movement, length 365 mm - for wall mounting, concealed 3-point fixing - wall connection diameter 58 mm, 10 mm deep - including non-corrosive HEWI fixing material - high-quality chrome-plated Article no.: HEWI 950.01.225 |
| 950.01.300 Mirror 700 mm backpainting in HEWI colour 98 (signal white) 950.01.300 in product catalogue |  | Mirror - round mirror with coloured backpainted edge - back-painting in HEWI colour 98 (signal white) - concealed fixing - diameter 700 mm, mirror 6 mm thick - mirror surface, diameter 610 mm - mirror made of high-quality float glass Article no.: HEWI 950.01.300 |
| 950.90.60090 Walking aid holder 950.90.60090 in product catalogue |  | Walking aid holder - used for holding a walking aid - concealed screw connection - can be used in left-hand or right-hand position - outside diameter 150 mm, 146 mm deep and rose diameter 56 mm - walking aid holder and rose cap made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 950.90.60090 |
| 950.90B60090 Walking aid holder Matt edition 950.90B60090 in product catalogue |  | Walking aid holder - used for holding a walking aid - concealed screw connection - can be used in left-hand or right-hand position - outside diameter 150 mm, 146 mm deep and rose diameter 56 mm - walking aid holder and rose cap made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 950.90B60090 |
| Accessories => Toilet area |  |  |
| 162.21.30040 Spare roll holder finish - chrome 162.21.30040 in product catalogue |  | System 162 Spare roll holder - cylindrical spare roll holder - made of metal, high-quality chrome-plated - 122 mm long, diameter 15 mm - for wall mounting, concealed fixing - incl. non-corrosive HEWI fixing materials Article no.: HEWI 162.21.30040 |
| 162.21.30060 Spare roll holder powder-coated 162.21.30060 in product catalogue |  | System 162 Spare roll holder - cylindrical spare roll holder - made of metal, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - 122 mm long, diameter 15 mm - for wall mounting, concealed fixing - incl. non-corrosive HEWI fixing materials Article no.: HEWI 162.21.30060 |
| 162.21.300XA Spare roll holder satin 162.21.300XA in product catalogue |  | System 162 Spare roll holder - cylindrical spare roll holder - made of stainless steel, satin finish - 122 mm long, diameter 15 mm - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 162.21.300XA |
| 477.06.750 Hygiene bag dispenser 477.06.750 in product catalogue |  | Range 477 Hygiene bag dispenser - cube-shaped body with soft radii - linear and functional in a clear and harmonious design - holds and dispenses proprietary hygiene bags made of synthetic material - for wall mounting - 143 mm wide, 103 mm high and 27 mm deep - made of high-quality polyamide in the HEWI colours - including non-corrosive HEWI fixing material Article no.: HEWI 477.06.750 |
| 477.06B750 Hygiene bag dispenser Matt edition 477.06B750 in product catalogue |  | Range 477 Hygiene bag dispenser - cube-shaped body with soft radii - linear and functional in a clear and harmonious design - holds and dispenses proprietary hygiene bags made of synthetic material - for wall mounting - 143 mm wide, 103 mm high and 27 mm deep - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey), 92 (anthracite grey) and 90 (jet black) - including non-corrosive HEWI fixing material Article no.: HEWI 477.06B750 |
| 477.20.010 Toilet brush 477.20.010 in product catalogue |  | Range 477 Toilet brush - brush handle with replaceable black brush head made of polyamide - brush head can be replaced due to bayonet fixing - 460 mm long - brush handle made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.20.010 |
| 477.20.100 Toilet brush unit 477.20.100 in product catalogue |  | Range 477 Toilet brush unit - brush handle with replaceable black brush head made of polyamide - brush holder with curved bottom and spherical, semi-open top in ring-shaped bracket - top part can be rotated - container is removable, but can also be fitted to prevent theft - brush head can be replaced due to bayonet fixing - anti-theft protection through concealed screw fixing - 141 mm wide, 470 mm high and 140 mm deep - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.20.100 |
| 477.20B010 Toilet brush Matt edition 477.20B010 in product catalogue |  | Range 477 Toilet brush - brush handle with replaceable brush head made of black polyamide - brush head can be replaced due to bayonet fixing - 460 mm long - brush handle made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.20B010 |
| 477.20B100 Toilet brush unit Matt edition 477.20B100 in product catalogue |  | Range 477 Toilet brush unit - brush handle with replaceable black brush head made of polyamide - brush holder with curved bottom and spherical, semi-open top in ring-shaped bracket - top part can be rotated - container is removable, but can also be fitted to prevent theft - brush head can be replaced due to bayonet fixing - anti-theft protection through concealed screw fixing - 141 mm wide, 470 mm high and 140 mm deep - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.20B100 |
| 477.21.100 Toilet roll holder 477.21.100 in product catalogue |  | Range 477 Toilet roll holder - right-angled U-shaped holder - anti-theft protection through concealed screw fixing - 160 mm wide, 120 mm high, rail 20 mm in diameter - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.21.100 |
| 477.21.150 Toilet roll holder with retaining element to prevent unauthorised removal 477.21.150 in product catalogue |  | Range 477 Toilet roll holder - right-angled U-shaped holder - with anti-theft feature for the toilet roll - anti-theft protection through concealed screw fixing - 160 mm wide, 120 mm high, rail 20 mm in diameter - including key for unlocking the anti-theft feature - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.21.150 |
| 477.21B100 Toilet roll holder Matt edition 477.21B100 in product catalogue |  | Range 477 Toilet roll holder - right-angled U-shaped holder - anti-theft protection through concealed screw fixing - 160 mm wide, 120 mm high, rail 20 mm in diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.21B100 |
| 477.21B150 Toilet roll holder Matt edition 477.21B150 in product catalogue |  | Range 477 Toilet roll holder - right-angled U-shaped holder - with anti-theft feature for the toilet roll - anti-theft protection through concealed screw fixing - 160 mm wide, 120 mm high, rail 20 mm in diameter - including key for unlocking the anti-theft feature - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.21B150 |
| 66823 Insert for toilet brush unit System 900 66823 in product catalogue |  | Synthetic material inner container - inner liner made of synthetic material, for toilet brush unit 900.20.000XA, 900.20.00040 and 900.20.00060 Article no.: HEWI 66823 |
| 801.20.010 Toilet brush 801.20.010 in product catalogue |  | Range 801 Toilet brush - long brush handle with ergonomic handle grip and black brush head made of polyamide - brush head can be replaced due to bayonet fixing - 540 mm long, handle grip 140 mm long - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 801.20.010 |
| 801.20.100 Toilet brush unit 801.20.100 in product catalogue |  | Range 801 Toilet brush unit - long brush handle with ergonomic handle grip and black brush head made of polyamide - cone-shaped container with large opening - brush head can be replaced due to bayonet fixing - brush is self-centring when put into container and stands upright automatically - with internal reservoir for disinfectant - container is removable - for wall mounting - 575 mm high, 130 mm wide and 140 mm deep - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 801.20.100 |
| 801.20B010 Toilet brush Matt edition 801.20B010 in product catalogue |  | Range 801 Toilet brush - long brush handle with ergonomic handle grip and black brush head made of polyamide - brush head can be replaced due to bayonet fixing - 540 mm long, handle grip 140 mm long - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 801.20B010 |
| 801.20B100 Toilet brush unit Matt edition 801.20B100 in product catalogue |  | Range 801 Toilet brush unit - long brush handle with ergonomic handle grip and black brush head made of polyamide - cone-shaped container with large opening - brush head can be replaced due to bayonet fixing - brush is self-centring when put into container and stands upright automatically - with internal reservoir for disinfectant - container is removable - for wall mounting - 575 mm high, 130 mm wide and 140 mm deep - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 801.20B100 |
| 801.21.401 Spare roll holder for one toilet roll, 162 mm high 801.21.401 in product catalogue |  | Range 801 Spare roll holder - right-angled holder pointing upwards with rose fixing - can hold one spare toilet roll - anti-theft protection through concealed screw fixing - 162 mm high and 105 mm deep, 70 mm rose diameter - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 801.21.401 |
| 801.21.402 Spare roll holder for two toilet rolls, 262 mm high 801.21.402 in product catalogue |  | Range 801 Spare roll holder - right-angled holder pointing upwards with rose fixing - can hold two spare toilet rolls - anti-theft protection through concealed screw fixing - 262 mm high and 105 mm deep, 70 mm rose diameter - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 801.21.402 |
| 801.21.403 Spare roll holder for three toilet rolls, 362 mm high 801.21.403 in product catalogue |  | Range 801 Spare roll holder - right-angled holder pointing upwards with rose fixing - can hold three spare toilet rolls - anti-theft protection through concealed screw fixing - 362 mm high and 105 mm deep, 70 mm rose diameter - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 801.21.403 |
| 801.21B401 Spare roll holder Matt edition 801.21B401 in product catalogue |  | Range 801 Spare roll holder - right-angled holder pointing upwards with rose fixing - can hold one spare toilet roll - anti-theft protection through concealed screw fixing - 162 mm high and 105 mm deep, 70 mm rose diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 801.21B401 |
| 801.21B402 Spare roll holder Matt edition 801.21B402 in product catalogue |  | Range 801 Spare roll holder - right-angled holder pointing upwards with rose fixing - can hold two spare toilet rolls - anti-theft protection through concealed screw fixing - 262 mm high and 105 mm deep, 70 mm rose diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 801.21B402 |
| 801.21B403 Spare roll holder Matt edition 801.21B403 in product catalogue |  | Range 801 Spare roll holder - right-angled holder pointing upwards with rose fixing - can hold three spare toilet rolls - anti-theft protection through concealed screw fixing - 362 mm high and 105 mm deep, 70 mm rose diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 801.21B403 |
| 801.91.010 Symbol, male 801.91.010 in product catalogue |  | Symbol male - self-adhesive - 150 mm high, 71.5 mm wide and 3 mm thick - made of high-quality polyamide - according to HEWI colour chart Article no.: HEWI 801.91.010 |
| 801.91.020 Symbol, female 801.91.020 in product catalogue |  | Symbol female - self-adhesive - 150 mm high, 87 mm wide and 3 mm thick - made of high-quality polyamide - according to HEWI colour chart Article no.: HEWI 801.91.020 |
| 801.91.030 Symbol, accessibility 801.91.030 in product catalogue |  | Symbol, accessibility - self-adhesive - 150 mm high, 135 mm wide and 3 mm thick - made of high-quality polyamide - according to HEWI colour chart Article no.: HEWI 801.91.030 |
| 801.91B010 Symbol, male Matt edition 801.91B010 in product catalogue |  | Symbol male - self-adhesive - 150 mm high, 71.5 mm wide and 3 mm thick - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey), 92 (anthracite grey) and 90 (jet black) Article no.: HEWI 801.91B010 |
| 801.91B020 Symbol, female Matt edition 801.91B020 in product catalogue |  | Symbol female - self-adhesive - 150 mm high, 87 mm wide and 3 mm thick - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey), 92 (anthracite grey) and 90 (jet black) Article no.: HEWI 801.91B020 |
| 801.91B030 Symbol, accessibility Matt edition 801.91B030 in product catalogue |  | Symbol, accessibility - self-adhesive - 150 mm high, 135 mm wide and 3 mm thick - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey), 92 (anthracite grey) and 90 (jet black) Article no.: HEWI 801.91B030 |
| 900.05.00060 Waste bin powder-coated 900.05.00060 in product catalogue |  | Hygiene waste bin System 900 - hygiene waste bin with hinged lid - the waste bin is detachable - the lid can be opened by lightly pressing the knee against the bin - with sticker to explain operation by knee - invisible, integrated bag holder - lid with soft-close - only suitable for wall mounting - capacity approx. 6 l - 310 mm high, 145 mm deep and 205 mm wide - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including corrosion-free HEWI fixing material Article no.: HEWI 900.05.00060 |
| 900.05.000XA Waste bin satin 900.05.000XA in product catalogue |  | Hygiene waste bin System 900 - hygiene waste bin with hinged lid - the waste bin is detachable - the lid can be opened by lightly pressing the knee against the bin - with sticker to explain operation by knee - invisible, integrated bag holder - lid with soft-close - only suitable for wall mounting - capacity approx. 6 l - 310 mm high, 145 mm deep and 205 mm wide - made of high-quality stainless steel, satin surface - including corrosion-free HEWI fixing material Article no.: HEWI 900.05.000XA |
| 900.05.00560 Hygiene waste bin powder-coated 900.05.00560 in product catalogue |  | Hygiene wastebin from the System 900 range - angular base body - capacity approx. 6 l - invisible, integrated bag holder with pull-out function for easy insertion and removal of the bag - lid with lifting tab - 160 mm wide, 290 mm high and 140 mm deep - for wall mounting - made of high-quality stainless steel, powder coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including corrosion-free HEWI fixing material Article no.: HEWI 900.05.00560 |
| 900.05.005XA Hygiene waste bin satin 900.05.005XA in product catalogue |  | Hygienic wastebin from the System 900 range - angular body - capacity approx. 6 l - invisible, integrated bag holder with pull-out function for easy insertion and removal of the bag - lid with lifting tab - 160 mm wide, 290 mm high and 140 mm deep - for wall mounting - made of high-quality stainless steel, matt satin finish surface - including corrosion-free HEWI fixing material Article no.: HEWI 900.05.005XA |
| 900.05.00660 Hygiene combination powder-coated 900.05.00660 in product catalogue |  | Hygiene combination from the System 900 range - angular base body - capacity approx. 6 l - invisible, integrated bag holder with pull-out function for easy insertion and removal of the bag - with integrated holder for standard hygiene bags made of synthetic material (max. pack size L 130 x W 90 x H 20 mm) - lid with lifting tab - 160 mm wide, 290 mm high and 140 mm deep - for wall mounting - made of high-quality stainless steel, powder coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including corrosion-free HEWI fixing material Article no.: HEWI 900.05.00660 |
| 900.05.006XA Hygiene combination satin 900.05.006XA in product catalogue |  | Hygiene combination from the System 900 range - angular basic body - capacity approx. 6 l - invisible, integrated bag holder with pull-out function for easy insertion and removal of the bag - with integrated holder for standard hygiene bags made of synthetic material (max. pack size L 130 x W 90 x H 20 mm) - lid with lifting tab - 160 mm wide, 290 mm high and 140 mm deep - for wall mounting - made of high-quality stainless steel, matt satin finish surface - including corrosion-free HEWI fixing material Article no.: HEWI 900.05.006XA |
| 900.06.01060 Electronic paper roll dispenser powder-coated 900.06.01060 in product catalogue |  | Electronic paper roll dispenser from the System 900 range - angular base body - serves to hold standard paper rolls with external unwinding, max. roll width 210 mm, max. roll diameter 200 mm, sleeve diameter 38 mm - level indicator at the side - easy paper roll replacement - with battery change indicator - automatic paper output at the washbasin after the paper has been torn off - integrated tear-off edge - protection against misuse through locking device - maintenance-free - 284 mm wide, 405 mm high and 210 mm deep - for wall mounting - made of high-quality stainless steel, powder coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - including corrosion-free HEWI fixing material - batteries (D - Mono - LR20 - 1.5 V / 4 pcs.) not included in the scope of supply - can be converted to mains connection 950.06.E01 - CE marking in accordance with EU Directive 2015/863/EC Article no.: HEWI 900.06.01060 |
| 900.06.010XA Electronic paper roll dispenser satin 900.06.010XA in product catalogue |  | Electronic paper roll dispenser from the System 900 range - angular base body - serves to hold standard paper rolls with external unwinding, max. roll width 210 mm, max. roll diameter 200 mm, sleeve diameter 38 mm - level indicator at the side - easy paper roll replacement - with battery change indicator - automatic paper output at the washbasin after the paper has been torn off - integrated tear-off edge - protection against misuse by locking device - maintenance-free - 284 mm wide, 405 mm high and 210 mm deep - for wall mounting - made of high-quality stainless steel, matt satin finish surface - including corrosion-free HEWI fixing material - batteries (D - Mono - LR20 - 1.5 V / 4 pcs.) not included in the scope of supply - can be converted to mains connection 950.06.E01 - CE marking in accordance with EU Directive 2015/863/EC Article no.: HEWI 900.06.010XA |
| 900.20.00040 Toilet brush unit chrome 900.20.00040 in product catalogue |  | System 900 Toilet brush unit - toilet brush unit consisting of bracket, toilet brush holder and brush - brush head can be replaced due to bayonet fixing - inner container can be removed for cleaning - made of metal, high-quality chrome-plated - 89 mm wide, 440 mm high, 106 mm deep - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.20.00040 |
| 900.20.00060 Toilet brush unit powder-coated 900.20.00060 in product catalogue |  | System 900 Toilet brush unit - toilet brush unit consisting of bracket, toilet brush holder and brush - brush head can be replaced due to bayonet fixing - inner container can be removed for cleaning - made of metal, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - 89 mm wide, 440 mm high, 108 mm deep - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.20.00060 |
| 900.20.000XA Toilet brush unit satin 900.20.000XA in product catalogue |  | System 900 Toilet brush unit - toilet brush unit consisting of bracket, toilet brush holder and brush - brush head can be replaced due to bayonet fixing - inner container can be removed for cleaning - made of high-quality stainless steel, satin finish - 89 mm wide, 440 mm high, 108 mm deep - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.20.000XA |
| 900.21.00040 Toilet roll holder chrome 900.21.00040 in product catalogue |  | System 900 Toilet roll holder - U-shaped holder, open on the right-hand side - hinged - made of high-quality stainless steel, chrome - 135 mm wide, 88 mm high, diameter 15 mm - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.21.00040 |
| 900.21.00060 Toilet roll holder powder-coated 900.21.00060 in product catalogue |  | System 900 Toilet roll holder - U-shaped holder, open on the right-hand side - hinged - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - 135 mm wide, 88 mm high, diameter 15 mm - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.21.00060 |
| 900.21.000XA Toilet roll holder satin 900.21.000XA in product catalogue |  | System 900 Toilet roll holder - U-shaped holder, open on the right-hand side - hinged - made of high-quality stainless steel, satin finish - 135 mm wide, 88 mm high, ø 15 mm - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.21.000XA |
| 900.90.00140 Walking aid holder chrome 900.90.00140 in product catalogue |  | Walking aid holder system 900 - made of high-quality stainless steel, chrome - 120 mm wide, 35 mm high, 135 mm deep - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.90.00140 |
| 900.90.00160 Walking aid holder powder-coated 900.90.00160 in product catalogue |  | Walking aid holder system 900 - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - 120 mm wide, 35 mm high, 135 mm deep - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.90.00160 |
| 900.90.001XA Walking aid holder satin 900.90.001XA in product catalogue |  | Walking aid holder system 900 - made of high-quality stainless steel, satin finish - 120 mm wide, 35 mm high, 135 mm deep - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.90.001XA |
| 921037 Brush head for toilet brush units and toilet brushes ranges LifeSystem, 805, 801 and 477, packing unit 1 pcs 921037 in product catalogue |  | Spare brush head - replaceable brush head for HEWI toilet brushes - for toilet brush unit 477.20..., 801.20..., 802.20.100 and 805.20.100 - for toilet brush 477.20..., 801.20..., 802.20.010 and 805.20.010 - for easy replacement on toilet brush due to bayonet fixing - with black bristles Article no.: HEWI 921037 |
| 921038 Brush head for toilet brush units and toilet brushes ranges LifeSystem, 805, 801 and 477, packing unit 25 pcs 921038 in product catalogue |  | Spare brush heads - replaceable brush head for HEWI toilet brushes - for toilet brush unit 477.20..., 801.20..., 802.20.100 and 805.20.100 - for toilet brush 477.20..., 801.20..., 802.20.010 and 805.20.010 - for easy replacement on toilet brush due to bayonet fixing - with black bristles - 25 in packing unit. Article no.: HEWI 921038 |
| 921044 Brush head for toilet brush units and toilet brushes ranges LifeSystem, 805, 801 and 477, packing unit 5 pcs 921044 in product catalogue |  | Spare brush heads - replaceable brush head for HEWI toilet brushes - for toilet brush unit 477.20..., 801.20..., 802.20.100 and 805.20.100 - for toilet brush 477.20..., 801.20..., 802.20.010 and 805.20.010 - for easy replacement on toilet brush due to bayonet fixing - with black bristles - 5 in packing unit Article no.: HEWI 921044 |
| 921050 Spare brush head for toilet brush unit 900.20.000..., 805.20.200, 162.20.100..., 100.20.100..., 800.20.100..., 815.20... and 900.20.000... for toilet brush 805.20.020, 100.20.010... and 800.20.010... 921050 in product catalogue |  | Spare brush head - replaceable brush head for HEWI toilet brushes - for complete toilet brush units 100.20.100..., 162.20.100..., 800.20.100..., 805.20.200, 815.20... and 900.20... - for individual toilet brushes 100.20.010..., 800.20.010..., 805.20.020 and 900.20.01060 - for easy replacement on toilet brush due to bayonet fixing - brush head made of black polyamide with anthracite grey bristles - diameter 76 mm Article no.: HEWI 921050 |
| 921051 Spare brush head for toilet brush unit 900.20.000..., 805.20.200, 162.20.100..., 100.20.100..., 800.20.100..., 815.20... and 900.20.000... for toilet brush 805.20.020, 100.20.010... and 800.20.010... 921051 in product catalogue |  | Spare brush heads - replaceable brush head for HEWI toilet brushes - for complete toilet brush units 100.20.100..., 162.20.100..., 800.20.100..., 805.20.200, 815.20... and 900.20... - for individual toilet brushes 100.20.010..., 800.20.010..., 805.20.020 and 900.20.01060 - for easy replacement on toilet brush due to bayonet fixing - brush head made of black polyamide with anthracite grey bristles - diameter 76 mm - 5 in packing unit Article no.: HEWI 921051 |
| 921052 Spare brush head for toilet brush unit 900.20.000..., 805.20.200, 162.20.100..., 100.20.100..., 800.20.100..., 815.20... and 900.20.000... for toilet brush 805.20.020, 100.20.010... and 800.20.010... 921052 in product catalogue |  | Spare brush heads - replaceable brush head for HEWI toilet brushes - for complete toilet brush units 100.20.100..., 162.20.100..., 800.20.100..., 805.20.200, 815.20... and 900.20... - for individual toilet brushes 100.20.010..., 800.20.010..., 805.20.020 and 900.20.01060 - for easy replacement on toilet brush due to bayonet fixing - brush head made of black polyamide with anthracite grey bristles - diameter 76 mm - 25 in packing unit Article no.: HEWI 921052 |
| Accessories => Shower and bath area |  |  |
| 162.03.20040 Shower shelf 300 mm chrome 162.03.20040 in product catalogue |  | Shower tray System 162 - shower tray mitred joined - made of stainless steel tube, diameter 18 mm, high-quality chrome-plated - for wall mounting with concealed fixing - with drain openings for water drainage - 300 mm wide, 86 mm high and 100 mm deep - internal dimensions of shower tray 264 mm wide and 82 mm deep - Including non-corrosive HEWI fixing material Article no.: HEWI 162.03.20040 |
|  |  | Shower tray System 162 - shower tray mitred joined - made of stainless steel tube, diameter 18 mm, matt black powder-coated - for wall mounting with concealed fixing - with drain openings for water drainage - 300 mm wide, 86 mm high and 100 mm deep - internal dimensions of shower tray 264 mm wide and 82 mm deep - Including non-corrosive HEWI fixing material Article no.: HEWI 162.03.20060 |
| 162.30.11040 Bath towel shelf 600 mm chrome 162.30.11040 in product catalogue |  | Bath towel shelf System 162 - bath towel shelf with mitred towel rail - made of brass tube, high quality chrome plated, diameter 18 mm - 618 mm wide, 120 mm high and 262 mm deep - Towel rail on lower level, projection 77 mm - for wall mounting, concealed mounting - Including non-corrosive HEWI fixing material Article no.: HEWI 162.30.11040 |
| 162.30.20040 Towel rail 600 mm finish - chrome 162.30.20040 in product catalogue |  | System 162 Bath towel rail - bath towel rail made of mitred tube - made of metal, high-quality chrome-plated - centre-to-centre 600 mm, 75 mm deep, diameter 18 mm - for wall mounting, concealed fixing - incl. non-corrosive HEWI fixing materials Article no.: HEWI 162.30.20040 |
| 162.30.20060 Towel rail 600 mm powder-coated 162.30.20060 in product catalogue |  | System 162 Bath towel rail - bath towel rail made of mitred tube - made of metal, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - centre-to-centre 600 mm, 75 mm deep, diameter 18 mm - for wall mounting, concealed fixing - incl. non-corrosive HEWI fixing materials Article no.: HEWI 162.30.20060 |
| 162.30.200XA Bath towel rail 600 mm satin 162.30.200XA in product catalogue |  | System 162 Bath towel rail - bath towel rail made of stainless steel, mitred tube - made of stainless steel, satin finish - centre-to-centre 600 mm, 75 mm deep, diameter 18 mm - for wall mounting, concealed fixing - incl. non-corrosive HEWI fixing materials Article no.: HEWI 162.30.200XA |
| 477.30.100 Bath towel rail C to C 570 mm 477.30.100 in product catalogue |  | Range 477 Bath towel rail - rail with right-angled ends and fixing roses - for hanging up bath towels and pieces of clothing - for wall mounting with roses - with corrosion-resistant steel core - fixing with roses - 570 mm wide (centre-to-centre), 85 mm deep and 28 mm in diameter, 60 mm rose diameter - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.30.100 |
| 477.30.200 Bath towel rail C to C 457 mm 477.30.200 in product catalogue |  | Range 477 Bath towel rail - rail with right-angled ends and fixing roses - for hanging up bath towels and pieces of clothing - for wall mounting with roses - with corrosion-resistant steel core - fixing with roses - 457 mm wide (centre-to-centre), 85 mm deep, 28 mm in diameter, 60 mm rose diameter - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.30.200 |
| 477.30.300 Bath towel rail c to c 600 mm 477.30.300 in product catalogue |  | Range 477 Bath towel rail - rail with right-angled ends and fixing roses - for hanging up bath towels and pieces of clothing - for wall mounting with roses - with corrosion-resistant steel insert - 600 mm wide (centre-to-centre), 90 mm deep and 33 mm in diameter, 70 mm rose diameter - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.30.300 |
| 477.30.310 Bath towel rail C to C 800 mm 477.30.310 in product catalogue |  | Range 477 Bath towel rail - rail with right-angled ends and fixing roses - for hanging up bath towels and pieces of clothing - for wall mounting with roses - with corrosion-resistant steel insert - 800 mm wide (centre-to-centre), 90 mm deep and 33 mm in diameter, 70 mm rose diameter - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.30.310 |
| 477.30.320 Bath towel rail C to C 1000 mm 477.30.320 in product catalogue |  | Range 477 Bath towel rail - rail with right-angled ends and fixing roses - for hanging up bath towels and pieces of clothing - for wall mounting with roses - with corrosion-resistant steel insert - 1000 mm wide (centre-to-centre), 90 mm deep and 33 mm in diameter, 70 mm rose diameter - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.30.320 |
| 477.30.330 Bath towel rail c to c 1100 mm 477.30.330 in product catalogue |  | Range 477 Bath towel rail - rail with right-angled ends and fixing roses - for hanging up bath towels and pieces of clothing - for wall mounting with roses - with corrosion-resistant steel insert - 1100 mm wide (centre-to-centre), 90 mm deep and 33 mm in diameter, 70 mm rose diameter - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.30.330 |
| 477.30B100 Bath towel rail 570 mm Matt edition 477.30B100 in product catalogue |  | Range 477 Bath towel rail - rail with right-angled ends and fixing roses - for hanging up bath towels and pieces of clothing - for wall mounting with roses - with corrosion-resistant steel core - fixing with roses - 570 mm wide (centre-to-centre), 85 mm deep and 28 mm in diameter, 60 mm rose diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.30B100 |
| 477.30B110 Bath towel rail 570 mm Matt edition 477.30B110 in product catalogue |  | Range 477 Bath towel rail - rail with right-angled ends without fixing roses - for hanging up bath towels and pieces of clothing - for wall mounting - with corrosion-resistant steel core - including non-corrosive HEWI fixing material - 570 mm wide (centre-to-centre), 80 mm deep and 28 mm in diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.30B110 |
| 477.30B200 Bath towel rail Matt edition 477.30B200 in product catalogue |  | Range 477 Bath towel rail - rail with right-angled ends and fixing roses - for hanging up bath towels and pieces of clothing - for wall mounting with roses - with corrosion-resistant steel core - fixing with roses - 457 mm wide (centre-to-centre), 85 mm deep, 28 mm in diameter, 60 mm rose diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.30B200 |
| 477.30B210 Bath towel rail Matt edition 477.30B210 in product catalogue |  | Range 477 Bath towel rail - rail with right-angled ends without fixing roses - for hanging up bath towels and pieces of clothing - for wall mounting - with corrosion-resistant steel core - 457 mm wide (centre-to-centre), 80 mm deep, 28 mm in diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.30B210 |
| 477.30B300 Bath towel rail 600 mm Matt edition 477.30B300 in product catalogue |  | Range 477 Bath towel rail - rail with right-angled ends and fixing roses - for hanging up bath towels and pieces of clothing - for wall mounting with roses - with corrosion-resistant steel insert - 600 mm wide (centre-to-centre), 90 mm deep and 33 mm in diameter, 70 mm rose diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.30B300 |
| 477.30B310 Bath towel rail 800 mm Matt edition 477.30B310 in product catalogue |  | Range 477 Bath towel rail - rail with right-angled ends and fixing roses - for hanging up bath towels and pieces of clothing - for wall mounting with roses - with corrosion-resistant steel insert - 800 mm wide (centre-to-centre), 90 mm deep and 33 mm in diameter, 70 mm rose diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.30B310 |
| 477.30B320 Bath towel rail 1000 mm Matt edition 477.30B320 in product catalogue |  | Range 477 Bath towel rail - rail with right-angled ends and fixing roses - for hanging up bath towels and pieces of clothing - for wall mounting with roses - with corrosion-resistant steel insert - 1000 mm wide (centre-to-centre), 90 mm deep and 33 mm in diameter, 70 mm rose diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.30B320 |
| 477.30B330 Bath towel rail 1100 mm Matt edition 477.30B330 in product catalogue |  | Range 477 Bath towel rail - rail with right-angled ends and fixing roses - for hanging up bath towels and pieces of clothing - for wall mounting with roses - with corrosion-resistant steel insert - 1100 mm wide (centre-to-centre), 90 mm deep and 33 mm in diameter, 70 mm rose diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.30B330 |
| 477.30US110 Bath towel rail (USA) 22 7/16" (570 mm) 477.30US110 in product catalogue |  | Towel holder (USA) from Range 477 - rod curved at right-angles at the ends without fixing roses - for hanging bath towels and other laundry items - for wall-mounting - with corrosion-protected steel core - includes non-corroding HEWI fixing material - 22 7/16 inch (570 mm) wide (c to c), 3 1/8 inch (80 mm) deep, 1 1/8 inch (28 mm) in diameter - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) Article no.: HEWI 477.30US110 |
| 477.30US210 Bath towel rail (USA) 18" (457 mm) 477.30US210 in product catalogue |  | Towel holder (USA) from Range 477 - rod curved at right-angles at the ends without fixing roses - for hanging bath towels and other laundry items - for wall-mounting - with corrosion-protected steel core - 18 inch (457 mm) wide (c to c), 3 1/8 inch (80 mm) deep, 1 1/8 inch (28 mm) in diameter - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) Article no.: HEWI 477.30US210 |
| 477.31.100 Grab bar 250 mm 477.31.100 in product catalogue |  | Range 477 Grab bar - rail with right-angled ends and fixing roses - provides support and extra safety in the bathtub - for wall mounting with roses - fixing with roses - 250 mm wide (centre-to-centre), 85 mm deep and 28 mm in diameter, 60 mm rose diameter - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.31.100 |
| 477.31B100 Grab bar 250 mm Matt edition 477.31B100 in product catalogue |  | Range 477 Grab bar - rail with right-angled ends and fixing roses - provides support and extra safety in the bathtub - for wall mounting with roses - fixing with roses - 250 mm wide (centre-to-centre), 85 mm deep and 28 mm in diameter, 60 mm rose diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.31B100 |
| 477.31BUS110 Grab bar (USA) 250 mm Matt edition 477.31BUS110 in product catalogue |  | Grab bar (USA) from Range 477 - rod curved at right-angles at the ends without fixing roses - serves as a support and for holding onto in the bathtub - for wall-mounting - includes non-corroding HEWI fixing material - 9 13/16 inch (250 mm) wide (c to c), 3 1/8 inch (80 mm) deep and 1 1/8 inch (28 mm) in diameter - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.31BUS110 |
| 477.31US110 Grab bar (USA) 9 13/16" (250 mm) 477.31US110 in product catalogue |  | Grab bar (USA) from Range 477 - rod curved at right-angles at the ends without fixing roses - serves as a support and for holding onto in the bathtub - for wall-mounting - includes non-corroding HEWI fixing material - 9 13/16 inch (250 mm) wide (c to c), 3 1/8 inch (80 mm) deep and 1 1/8 inch (28 mm) in diameter - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) Article no.: HEWI 477.31US110 |
| 477.32.100 Corner shelf 477.32.100 in product catalogue |  | Range 477 Corner shelf - round dish with drain hole, inserted between two holders either at right-angles or parallel to one another - for storage utensils in shower area, a push-button facilitated release - lockable as anti-theft protection - for mounting on walls or in corners (80° - 135°) - 205 mm wide, 205 mm deep and 37 mm high, shelf area 150 mm in diameter, holders 90 mm wide and deep - holders and dish made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 477.32.100 |
| 477.32B100 Corner shelf Matt edition 477.32B100 in product catalogue |  | Range 477 Corner shelf - round dish with drain hole, inserted between two holders either at right-angles or parallel to one another - for storage utensils in shower area, a push-button facilitated release - lockable as anti-theft protection - for mounting on walls or in corners (80° - 135°) - 205 mm wide, 205 mm deep and 37 mm high, shelf area 150 mm in diameter, holders 90 mm wide and deep - holders and dish made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 477.32B100 |
| 801.22BUS100 L-shaped grab bar (ADA) Matt edition 801.22BUS100 in product catalogue |  | L-shaped support rail (USA) from Range 801 - can be mounted on the right and left side - positioned horizontally and vertically, rails connected at a right angle with steel fixing roses - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - horizontal length 12 inch (305 mm), vertical length 24 inch (610 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - toilet paper holder upgrade kit 801.50US010 can be retrofitted - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.22BUS100 |
| 801.22BUS110 L-shaped grab bar (ADA) Matt edition 801.22BUS110 in product catalogue |  | L-shaped support rail (USA) from Range 801 - can be mounted on the right and left side - positioned horizontally and vertically, rails connected at a right angle with steel fixing roses - for holding onto and as a support - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - horizontal length 18 inch (457 mm), vertical length 36 inch (915 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - toilet paper holder upgrade kit 801.50US010 can be retrofitted - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.22BUS110 |
| 801.33BUS100 Grab bar with shower head holder (ADA) 24" (610 mm) 801.33BUS100 in product catalogue |  | Rail with shower head holder (USA) from Range 801 - vertical rail curved at right-angles at the ends with fixing roses and shower head holder - suitable for handheld shower heads from various manufacturers - shower head holder can be tilted continuously and its height can be adjusted by pulling or pushing a broad lever - functional area of the shower head holder in high-quality, glossy polyamide - cone-shaped insert on the shower head holder makes it easier to mount the handheld shower head - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - c to c 24 inch (610 mm) - 2 3/4 inch (70 mm) deep, rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.33BUS100 |
| 801.33BUS200 L-shaped grab bar with shower head holder (ADA) 36" x 18" (915 x 457 mm) 801.33BUS200 in product catalogue |  | L-shaped support rail with shower head holder (USA) from Range 801 - can be mounted on the right and left side - positioned horizontally and vertically, rails connected at a right angle with steel fixing roses and shower head holder - for holding onto and as a support in the shower and bathtub - suitable for handheld shower heads from various manufacturers - shower head holder can be tilted continuously and its height can be adjusted by pulling or pushing a broad lever - cone-shaped insert on the shower head holder makes it easier to mount the handheld shower head - with corrosion-protected steel core throughout - wall mounting with wall-specific fixing material and roses from HEWI - horizontal length 18 inch (457 mm), vertical length 36 inch (915 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - maximum loading capacity 330 lbs (150 kg) - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) - meets the requirements of 2010 ADA standards and ICC/ANSI A117.1-2017 Article no.: HEWI 801.33BUS200 |
|  |  | Rail with shower head holder (USA) from Range 801 - vertical rail with right-angled ends with steel fixing roses and shower head holder - centre-to-centre made-to-measure from 23 5/8 inch (600 mm) to 49 3/16 inch (1250 mm) - 2 3/4 inch (70 mm) deep, rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - suitable for shower heads of various manufacturers - shower head holder can be continuously tilted and, after pulling or pushing a large lever, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the handheld shower head - with continuous, corrosion resistant steel core - for wall mounting with special HEWI fixing materials and roses - maximum loading capacity 330 lbs (150 kg) - made of high-quality polyamide in HEWI colours 99 (pure white), 95 (stone grey), 92 (anthracite grey), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 Article no.: HEWI 801.33US1S |
|  |  | L-shaped support rail with shower head holder (USA) from Range 801 - vertical and horizontal rails joined to form an L shape and fitted with rose fixings and a shower head holder - for holding on to and providing support in shower and bath area - vertical length made-to-measure from 23 5/8 inch (600 mm) to 49 3/16 inch (1250 mm), horizontal length made-to-measure from 11 13/16 inch (300 mm) to 33 7/8 inch (850 mm), 2 3/4 inch (70 mm) deep - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - suitable for shower heads of various manufacturers - shower head holder can be continuously tilted and, after pulling or pushing a large lever, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the handheld shower head - with continuous, corrosion resistant steel core - for wall mounting with special HEWI fixing materials and roses - can be mounted in left hand or right hand position - maximum loading capacity 330 lbs (150 kg) - made of high-quality polyamide in HEWI colours 99 (pure white), 95 (stone grey), 92 (anthracite grey), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) - meets requirements according to 2010 ADA standards and ICC / ANSI A117.1-2017 Article no.: HEWI 801.33US2S |
| 801.34B4S Shower curtain rail, special length L: min. 400 mm, max. 1700 mm with aluminum core 801.34B4S in product catalogue |  | Range 801 Shower curtain rail - rail with curtain rings - for installing a shower curtain - with aluminium core - fixed onto wall with roses - can be shortened to 200 mm on site - with a corresponding number of curtain rings - made-to-measure length from min. 400 mm to max. 1700 mm, rail diameter 33 mm and rose diameter 70 mm - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 801.34B4S |
| 801.34BUS020 Ceiling support (USA) 21 1/4" (540 mm) 801.34BUS020 in product catalogue |  | Ceiling support (USA) from Range 801 - rod with fixing rose - for mounting curtain rods - with aluminum core - mounted to ceiling with rose - axis curtain rod - ceiling: 21 1/4 inch (540 mm) - length can be adjusted by 3/8 inch (10 mm) and can be shortened at the rose by max. 3 15/16 inch (100 mm) - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - Fixing material included in delivery - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 801.34BUS020 |
| 801.34BUS110 Shower curtain rail (USA) 60" x 36" (1524 x 915 mm) 801.34BUS110 in product catalogue |  | Curtain rod (USA) from Range 801 - arranged horizontally, rods connected at a right angle with curtain rings - for mounting a shower curtain - with aluminum core - mounting to wall and ceiling with roses, ceiling support required - with 24 curtain rings - c to c 60 inch x 36 inch (1524 x 915 mm), rod diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - the rose ends can be shortened up to 8 11/16 inch (220 mm) on site - made of high-quality matt polyamide in HEWI colours 99 (pure white), 95 (stone grey) and 92 (anthracite grey) Article no.: HEWI 801.34BUS110 |
| 801.34US020 Ceiling support (USA) 21 1/4" (540 mm) 801.34US020 in product catalogue |  | Ceiling support (USA) from Range 801 - rod with fixing rose - for mounting curtain rods - with aluminum core - mounted to ceiling with rose - axis curtain rod - ceiling: 21 1/4 inch (540 mm) - length can be adjusted by 3/8 inch (10 mm) and can be shortened at the rose by max. 3 15/16 inch (100 mm) - rail diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - Fixing material included in delivery - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) Article no.: HEWI 801.34US020 |
| 801.34US100 Shower curtain rail (USA) 36" x 36" (915 x 915 mm) 801.34US100 in product catalogue |  | Curtain rod (USA) from Range 801 - arranged horizontally, rods connected at a right angle with curtain rings - for mounting a shower curtain - with aluminum core - mounting to wall and ceiling with roses, ceiling support required - with 20 curtain rings - c to c 36 inch x 36 inch (915 x 915 mm), rod diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - the rose ends can be shortened up to 8 11/16 inch (220 mm) on site - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) Article no.: HEWI 801.34US100 |
| 801.34US110 Shower curtain rail (USA) 60" x 36" (1524 x 915 mm) 801.34US110 in product catalogue |  | Curtain rod (USA) from Range 801 - arranged horizontally, rods connected at a right angle with curtain rings - for mounting a shower curtain - with aluminum core - mounting to wall and ceiling with roses, ceiling support required - with 24 curtain rings - c to c 60 inch x 36 inch (1524 x 915 mm), rod diameter 1 5/16 inch (33 mm), rose diameter 2 3/4 inch (70 mm) - the rose ends can be shortened up to 8 11/16 inch (220 mm) on site - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) Article no.: HEWI 801.34US110 |
| 801.34US400 Shower curtain rail (USA) 39 3/8 " (1000 mm) 801.34US400 in product catalogue |  | Curtain rod (USA) from Range 801 - rail with curtain rings - for mounting a shower curtain - with aluminum core - fixed onto wall with roses - can be shortened to 7 9/10 inch (200 mm) mm on site - with 10 curtain rings - 39 3/8 inch (1000 mm) long, rail diameter 1 5/16 inch (33 mm) and rose diameter 2 3/4 inch (70 mm) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) Article no.: HEWI 801.34US400 |
| 801.34US410 Shower curtain rail (USA) 19 11/16" (500 mm) 801.34US410 in product catalogue | Similar picture | Curtain rod (USA) from Range 801 - rail with curtain rings - for mounting a shower curtain - with aluminum core - fixed onto wall with roses - can be shortened to 7 9/10 inch (200 mm) mm on site - with 6 curtain rings - 19 11/16 inch (500 mm) long, rail diameter 1 5/16 inch (33 mm) and rose diameter 2 3/4 inch (70 mm) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) Article no.: HEWI 801.34US410 |
| 801.34US420 Shower curtain rail (USA) 59 1/16" (1500 mm) 801.34US420 in product catalogue | Similar picture | Curtain rod (USA) from Range 801 - rail with curtain rings - for mounting a shower curtain - with aluminum core - fixed onto wall with roses - can be shortened to 7 9/10 inch (200 mm) mm on site - with 15 curtain rings - 59 1/16 inch (1500 mm) long, rail diameter 1 5/16 inch (33 mm) and rose diameter 2 3/4 inch (70 mm) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) Article no.: HEWI 801.34US420 |
| 801.34US430 Shower curtain rail (USA) 78 3/4" (2000 mm) 801.34US430 in product catalogue | Similar picture | Curtain rod (USA) from Range 801 - rail with curtain rings - for mounting a shower curtain - with aluminum core - fixed onto wall with roses - can be shortened to 7 9/10 inch (200 mm) mm on site - with 20 curtain rings - 78 3/4 inch (2000 mm) long, rail diameter 1 5/16 inch (33 mm) and rose diameter 2 3/4 inch (70 mm) - made from high-grade synthetic material in the HEWI colors 99 (pure white), 95 (stone gray), 92 (anthracite gray), 90 (jet black), 86 (sand), 84 (umber) and 33 (ruby red) Article no.: HEWI 801.34US430 |
| 801.90.010 Coat hook ø 40 mm 801.90.010 in product catalogue |  | Range 801 Single hook - right-angled bend, cylindrical hook with fixing rose - for hanging up towels and other pieces of clothing and other utensils - anti-theft protection through concealed screw fixing - 30 mm deep, rose diameter 40 mm - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 801.90.010 |
| 801.90.020 Double hook ø 40 mm 801.90.020 in product catalogue |  | Range 801 Double hook - double hook with fixing rose - for hanging up towels and other pieces of clothing and other utensils - anti-theft protection through concealed screw fixing - 31 mm deep, rose diameter 40 mm - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 801.90.020 |
| 801.90.030 Double coat hook 801.90.030 in product catalogue |  | Range 801 Double hook - two polyamide support pins with a stirrup-shaped hook - for hanging up clothing and utensils - reduced risk of injury as hooks face backwards towards wall - 90 mm wide, 165 mm high and 95 mm deep, centre-to-centre 75 mm - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 801.90.030 |
| 801.90.040 Double coat hook 801.90.040 in product catalogue |  | Range 801 Double hook for cloakroom rails - double hook with forward-facing hooks - for hanging up clothing and utensils - 90 mm wide, 165 mm high and 87 mm deep, centre-to-centre 75 mm - made of high-quality polyamide according to HEWI colour chart Article no.: HEWI 801.90.040 |
| 801.90B030 Double coat hook Matt edition 801.90B030 in product catalogue |  | Range 801 Double hook - two polyamide support pins with a stirrup-shaped hook - for hanging up clothing and utensils - reduced risk of injury as hooks face backwards towards wall - 90 mm wide, 165 mm high and 95 mm deep, centre-to-centre 75 mm - made of high-quality matt polyamide according to HEWI colour chart Article no.: HEWI 801.90B030 |
| 801.90B040 Double coat hook Matt edition 801.90B040 in product catalogue |  | Range 801 Double hook for cloakroom rails - double hook with forward-facing hooks - for hanging up clothing and utensils - 90 mm wide, 165 mm high and 87 mm deep, centre-to-centre 75 mm - made of high-quality matt polyamide according to HEWI colour chart Article no.: HEWI 801.90B040 |
| 802.03.220 Corner shelf with holder 802.03.220 in product catalogue |  | Corner shelf with holder Range 802 LifeSystem - right-angled corner shelf with rounded front, high sides and drainage feature - used to store accessories - opening at the front - loading capacity 3 kg - base with inclined draining surfaces and non-slip profile - for wall mounting - side lengths 210 mm, 125 mm high - made of high-quality polyamide in selected colours - holder highly polished chrome Article no.: HEWI 802.03.220 |
| 802.03B220 Corner shelf with holder Matt edition 802.03B220 in product catalogue |  | Corner shelf with holder Range 802 LifeSystem - right-angled corner shelf with rounded front, high sides and drainage feature - used to store accessories - opening at the front - loading capacity 3 kg - base with inclined draining surfaces and non-slip profile - for wall mounting - side lengths 210 mm, 125 mm high - made of high-quality matt polyamide in HEWI colours 99 (pure white), 98 (signal white), 97 (light grey), 95 (stone grey) and 92 (anthracite grey) - holder highly polished chrome Article no.: HEWI 802.03B220 |
| 900.03.00140 Shower basket chrome 900.03.00140 in product catalogue |  | System 900 Shower basket - for wall mounting - holder made of high-quality stainless steel chrome-plated - Insert made of high-quality synthetic material in HEWI colours 92 (anthracite grey) and 98 (signal-white) - 220 mm wide, 67 mm high, 115 mm deep - including non-corrosive HEWI fixing material. Article no.: HEWI 900.03.00140 |
| 900.03.00160 Shower basket powder-coated 900.03.00160 in product catalogue |  | System 900 Shower basket - for wall mounting - holder made of high-quality stainless steel powder-coated in the HEWI colour CV (matt black) and ER (matt dark grey pearl mica) with insert made of high-quality synthetic material in HEWI colour 90 (jet black) or powder-coated in the HEWI colours AS (matt white) and AX (matt light grey pearl mica) with insert made of high-quality synthetic material in HEWI colour 98 (signal-white) - 220 mm wide, 67 mm high, 115 mm deep - including non-corrosive HEWI fixing material. Article no.: HEWI 900.03.00160 |
| 900.03.001XA Shower basket satin 900.03.001XA in product catalogue |  | System 900 Shower basket - for wall mounting - holder made of high-quality satin stainless steel - Insert made of high-quality synthetic material in HEWI colours 92 (anthracite grey) and 98 (signal-white) - 220 mm wide, 67 mm high, 115 mm deep - including non-corrosive HEWI fixing material. Article no.: HEWI 900.03.001XA |
| 900.03.00240 Shower basket chrome 900.03.00240 in product catalogue |  | System 900 Shower basket corner solution - for wall mounting - holder made of high-quality stainless steel chrome-plated - Insert made of high-quality synthetic material in HEWI colours 92 (anthracite grey) and 98 (signal-white) - 67 mm high, side length 170 mm - including non-corrosive HEWI fixing material Article no.: HEWI 900.03.00240 |
| 900.03.00260 Shower basket powder-coated 900.03.00260 in product catalogue |  | System 900 Shower basket corner solution - for wall mounting - holder made of high-quality stainless steel powder-coated in the HEWI colour CV (matt black) and ER (matt dark grey pearl mica) with insert made of high-quality synthetic material in HEWI colour 90 (jet black) or powder-coated in the HEWI colours AS (matt white) and AX (matt light grey pearl mica) with insert made of high-quality synthetic material in HEWI colour 98 (signal-white) - 67 mm high, side length 170 mm - including non-corrosive HEWI fixing material Article no.: HEWI 900.03.00260 |
| 900.03.002XA Shower basket satin 900.03.002XA in product catalogue |  | System 900 Shower basket corner solution - for wall mounting - holder made of high-quality satin stainless steel - Insert made of high-quality synthetic material in HEWI colours 92 (anthracite grey) and 98 (signal-white) - 67 mm high, side length 170 mm - including non-corrosive HEWI fixing material. Article no.: HEWI 900.03.002XA |
| 900.33.00040 Rail with shower head holder 600 mm chrome 900.33.00040 in product catalogue |  | Shower head rail, System 900 - vertical rail with supports perpendicular to the wall, mounting and shower head holder - fixing spacing 600 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of stainless steel, chrome - shower head holder made of high-quality matt polyamide in the HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.33.00040 |
| 900.33.00060 Rail with shower head holder 600 mm powder-coated 900.33.00060 in product catalogue |  | Shower head rail, System 900 - vertical rail with supports perpendicular to the wall, mounting and shower head holder - fixing spacing 600 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, powder-coated in the HEWI colour CV (matt black) and ER (matt dark grey pearl mica) with a showerhead holder in matt black polyamide or powder-coated in the HEWI colours AS (matt white) and AX (matt light grey pearl mica) with a showerhead holder in matt white polyamide - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.33.00060 |
| 900.33.000XA Rail with shower head holder 600 mm satin 900.33.000XA in product catalogue |  | Shower head rail, System 900 - vertical rail with supports perpendicular to the wall, mounting and shower head holder - fixing spacing 600 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of stainless steel, satin - shower head holder made of high-quality matt polyamide in the HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.33.000XA |
| 900.33.00140 Rail with shower head holder 900 mm chrome 900.33.00140 in product catalogue |  | Shower head rail, System 900 - vertical rail with supports perpendicular to the wall, mounting and shower head holder - fixing spacing 900 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of stainless steel, chrome - shower head holder made of high-quality matt polyamide in the HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.33.00140 |
| 900.33.00160 Rail with shower head holder 900 mm powder-coated 900.33.00160 in product catalogue |  | Shower head rail, System 900 - vertical rail with supports perpendicular to the wall, mounting and shower head holder - fixing spacing 900 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, powder-coated in the HEWI colour CV (matt black) and ER (matt dark grey pearl mica) with a showerhead holder in matt black polyamide or powder-coated in the HEWI colours AS (matt white) and AX (matt light grey pearl mica) with a showerhead holder in matt white polyamide - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.33.00160 |
| 900.33.001XA Rail with shower head holder 900 mm satin 900.33.001XA in product catalogue |  | Shower head rail, System 900 - vertical rail with supports perpendicular to the wall, mounting and shower head holder - fixing spacing 900 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of stainless steel, satin - shower head holder made of high-quality matt polyamide in the HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.33.001XA |
| 900.33.00240 Rail with shower head holder 1100 mm chrome 900.33.00240 in product catalogue |  | Shower head rail, System 900 - vertical rail with supports perpendicular to the wall, mounting and shower head holder - fixing spacing 1100 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of stainless steel, chrome - shower head holder made of high-quality matt polyamide in the HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.33.00240 |
| 900.33.00260 Rail with shower head holder 1100 mm powder-coated 900.33.00260 in product catalogue |  | Shower head rail, System 900 - vertical rail with supports perpendicular to the wall, mounting and shower head holder - fixing spacing 1100 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, powder-coated in the HEWI colour CV (matt black) and ER (matt dark grey pearl mica) with a showerhead holder in matt black polyamide or powder-coated in the HEWI colours AS (matt white) and AX (matt light grey pearl mica) with a showerhead holder in matt white polyamide - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.33.00260 |
| 900.33.002XA Rail with shower head holder 1100 mm satin 900.33.002XA in product catalogue |  | Shower head rail, System 900 - vertical rail with supports perpendicular to the wall, mounting and shower head holder - fixing spacing 1100 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of stainless steel, satin - shower head holder made of high-quality matt polyamide in the HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.33.002XA |
| 900.33.03040 Rail with shower head holder 600 mm chrome 900.33.03040 in product catalogue |  | Shower head rail, System 900 - vertical rail with supports perpendicular to the wall, mounting and shower head holder - fixing spacing 600 mm, 84.5 mm deep - rail diameter 25 mm, tube thickness 2 mm, rose diameter 70 mm - made of stainless steel, chrome - shower head holder made of synthetic material, chrome - suitable for shower heads of various manufacturers - shower head holder can be tilted with detent and height-adjusted by turning with spring-assisted return - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.33.03040 |
| 900.33.03060 Rail with shower head holder 600 mm powder-coated 900.33.03060 in product catalogue |  | Shower head rail, System 900 - vertical rail with supports perpendicular to the wall, mounting and shower head holder - fixing spacing 600 mm, 84.5 mm deep - rail diameter 25 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, powder-coated in the HEWI colours WD (matt white), JK (matt black), CT (matt light grey pearl mica) and DD (matt dark grey pearl mica) - shower head holder made of synthetic material, chrome - suitable for shower heads of various manufacturers - shower head holder can be tilted with detent and height-adjusted by turning with spring-assisted return - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.33.03060 |
| 900.33.03140 Rail with shower head holder 900 mm chrome 900.33.03140 in product catalogue |  | Shower head rail, System 900 - vertical rail with supports perpendicular to the wall, mounting and shower head holder - fixing spacing 900 mm, 84.5 mm deep - rail diameter 25 mm, tube thickness 2 mm, rose diameter 70 mm - made of stainless steel, chrome - shower head holder made of synthetic material, chrome - suitable for shower heads of various manufacturers - shower head holder can be tilted with detent and height-adjusted by turning with spring-assisted return - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.33.03140 |
| 900.33.03160 Rail with shower head holder 900 mm powder-coated 900.33.03160 in product catalogue |  | Shower head rail, System 900 - vertical rail with supports perpendicular to the wall, mounting and shower head holder - fixing spacing 900 mm, 84.5 mm deep - rail diameter 25 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, powder-coated in the HEWI colours WD (matt white), JK (matt black), CT (matt light grey pearl mica) and DD (matt dark grey pearl mica) - shower head holder made of synthetic material, chrome - suitable for shower heads of various manufacturers - shower head holder can be tilted with detent and height-adjusted by turning with spring-assisted return - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.33.03160 |
| 900.33.03240 Rail with shower head holder 1100 mm chrome 900.33.03240 in product catalogue |  | Shower head rail, System 900 - vertical rail with supports perpendicular to the wall, mounting and shower head holder - fixing spacing 1100 mm, 84.5 mm deep - rail diameter 25 mm, tube thickness 2 mm, rose diameter 70 mm - made of stainless steel, chrome - shower head holder made of synthetic material, chrome - suitable for shower heads of various manufacturers - shower head holder can be tilted with detent and height-adjusted by turning with spring-assisted return - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.33.03240 |
| 900.33.03260 Rail with shower head holder 1100 mm powder-coated 900.33.03260 in product catalogue |  | Shower head rail, System 900 - vertical rail with supports perpendicular to the wall, mounting and shower head holder - fixing spacing 1100 mm, 84.5 mm deep - rail diameter 25 mm, tube thickness 2 mm, rose diameter 70 mm - made of high-quality stainless steel, powder-coated in the HEWI colours WD (matt white), JK (matt black), CT (matt light grey pearl mica) and DD (matt dark grey pearl mica) - shower head holder made of synthetic material, chrome - suitable for shower heads of various manufacturers - shower head holder can be tilted with detent and height-adjusted by turning with spring-assisted return - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.33.03260 |
| 900.33.0S0XA Rail with shower head holder, special length satin 900.33.0S0XA in product catalogue |  | Shower head rail, System 900 - vertical rail with supports perpendicular to the wall, mounting and shower head holder - fixing spacing made-to-measure from 600 to 1250 mm, 88 mm deep - rail diameter 32 mm, tube thickness 2 mm, rose diameter 70 mm - made of stainless steel, satin - shower head holder made of high-quality matt polyamide in the HEWI colours 98 (signal white) and 92 (anthracite grey) - suitable for shower heads of various manufacturers - shower head holder can be tilted continuously and, after pressing a large pushbutton, its height can be adjusted - conical adapter on shower head holder makes it easier to hook in the shower head - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.33.0S0XA |
| 900.34.00040 Ceiling support 300 mm chrome 900.34.00040 in product catalogue |  | Ceiling hanger, System 900 - straight rail with mounting rose - for mounting curtain rails - made of high-quality stainless steel, chrome - length 300 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the rose to 200 mm on site - fixing on the ceiling with rose - including non-corrosive fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.00040 |
| 900.34.00060 Ceiling support 300 mm powder-coated 900.34.00060 in product catalogue |  | Ceiling hanger, System 900 - straight rail with mounting rose - for mounting curtain rails - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - length 300 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the rose to 200 mm on site - fixing on the ceiling with rose - including non-corrosive fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.00060 |
| 900.34.000XA Ceiling support 300 mm satin 900.34.000XA in product catalogue |  | Ceiling hanger, System 900 - straight rail with mounting rose - for mounting curtain rails - made of high-quality stainless steel, satin surface - length 300 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the rose to 200 mm on site - fixing on the ceiling with rose - including non-corrosive fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.000XA |
| 900.34.00140 Ceiling support 400 mm chrome 900.34.00140 in product catalogue |  | Ceiling hanger, System 900 - straight rail with mounting rose - for mounting curtain rails - made of high-quality stainless steel, chrome - length 400 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the rose to 200 mm on site - fixing on the ceiling with rose - including non-corrosive fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.00140 |
| 900.34.00160 Ceiling support 400 mm powder-coated 900.34.00160 in product catalogue |  | Ceiling hanger, System 900 - straight rail with mounting rose - for mounting curtain rails - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - length 400 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the rose to 200 mm on site - fixing on the ceiling with rose - including non-corrosive fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.00160 |
| 900.34.001XA Ceiling support 400 mm satin 900.34.001XA in product catalogue |  | Ceiling hanger, System 900 - straight rail with mounting rose - for mounting curtain rails - made of high-quality stainless steel, satin surface - length 400 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the rose to 200 mm on site - fixing on the ceiling with rose - including non-corrosive fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.001XA |
| 900.34.00240 Ceiling support 500 mm chrome 900.34.00240 in product catalogue |  | Ceiling hanger, System 900 - straight rail with mounting rose - for mounting curtain rails - made of high-quality stainless steel, chrome - length 500 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the rose to 200 mm on site - fixing on the ceiling with rose - including non-corrosive fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.00240 |
| 900.34.00260 Ceiling support 500 mm powder-coated 900.34.00260 in product catalogue |  | Ceiling hanger, System 900 - straight rail with mounting rose - for mounting curtain rails - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - length 500 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the rose to 200 mm on site - fixing on the ceiling with rose - including non-corrosive fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.00260 |
| 900.34.002XA Ceiling support 500 mm satin 900.34.002XA in product catalogue |  | Ceiling hanger, System 900 - straight rail with mounting rose - for mounting curtain rails - made of high-quality stainless steel, satin surface - length 500 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the rose to 200 mm on site - fixing on the ceiling with rose - including non-corrosive fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.002XA |
| 900.34.00340 Ceiling support 600 mm chrome 900.34.00340 in product catalogue |  | Ceiling hanger, System 900 - straight rail with mounting rose - for mounting curtain rails - made of high-quality stainless steel, chrome - length 600 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the rose to 200 mm on site - fixing on the ceiling with rose - including non-corrosive fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.00340 |
| 900.34.00360 Ceiling support 600 mm powder-coated 900.34.00360 in product catalogue |  | Ceiling hanger, System 900 - straight rail with mounting rose - for mounting curtain rails - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - length 600 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the rose to 200 mm on site - fixing on the ceiling with rose - including non-corrosive fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.00360 |
| 900.34.003XA Ceiling support 600 mm satin 900.34.003XA in product catalogue |  | Ceiling hanger, System 900 - straight rail with mounting rose - for mounting curtain rails - made of high-quality stainless steel, satin surface - length 600 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the rose to 200 mm on site - fixing on the ceiling with rose - including non-corrosive fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.003XA |
| 900.34.00440 Ceiling support 1500 mm chrome 900.34.00440 in product catalogue |  | Ceiling hanger, System 900 - straight rail with mounting rose - for mounting curtain rails - made of high-quality stainless steel, chrome - length 1500 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the rose to 200 mm on site - fixing on the ceiling with rose - including non-corrosive fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.00440 |
| 900.34.00460 Ceiling support 1500 mm powder-coated 900.34.00460 in product catalogue |  | Ceiling hanger, System 900 - straight rail with mounting rose - for mounting curtain rails - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - length 1500 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the rose to 200 mm on site - fixing on the ceiling with rose - including non-corrosive fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.00460 |
| 900.34.00SXA Ceiling support, special length satin 900.34.00SXA in product catalogue |  | Ceiling hanger, System 900 - straight rail with mounting rose - for mounting curtain rails - made of high-quality stainless steel, satin surface - length made-to-measure from 150 mm to 299 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the rose to 200 mm on site - fixing on the ceiling with rose - including non-corrosive fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.00SXA |
| 900.34.01SXA Ceiling support, special length satin 900.34.01SXA in product catalogue |  | Ceiling hanger, System 900 - straight rail with mounting rose - for mounting curtain rails - made of high-quality stainless steel, satin surface - length made-to-measure from 301 mm to 1500 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the rose to 200 mm on site - fixing on the ceiling with rose - including non-corrosive fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.01SXA |
| 900.34.02SXA Ceiling support for suspended ceilings satin 900.34.02SXA in product catalogue |  | Ceiling hanger, System 900 - straight rod with cover rose - serves to fix curtain rails to suspended ceilings - made of high-quality stainless steel, satin surface finish - length made to measure from 150 mm to 1500 mm - rail diameter 25 mm, rose diameter 70 mm - fixing directly to threaded rod M8 provided by customer - including corrosion-free fixing material for the rose Article no.: HEWI 900.34.02SXA |
| 900.34.10040 Shower curtain rail for shower 800 x 800 mm chrome 900.34.10040 in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles, with curtain rings - made of high-quality stainless steel, chrome - for shower tray 800 x 800 mm - fixing spacing 787 x 787 mm - with 16 curtain rings made of white POM - recommended curtain width 2300 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing with roses to wall and ceiling, ceiling support required - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.10040 |
| 900.34.10060 Shower curtain rail for shower 800 x 800 mm powder-coated 900.34.10060 in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles, with curtain rings - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for shower tray 800 x 800 mm - fixing spacing 787 x 787 mm - with 16 curtain rings made of white POM - recommended curtain width 2300 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing with roses to wall and ceiling, ceiling support required - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.10060 |
| 900.34.100XA Shower curtain rail for shower 800 x 800 mm satin 900.34.100XA in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles, with curtain rings - made of high-quality stainless steel, satin - for shower tray 800 x 800 mm - fixing spacing 787 x 787 mm - with 16 curtain rings made of white POM - recommended curtain width 2300 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing with roses to wall and ceiling, ceiling support required - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.100XA |
| 900.34.10140 Shower curtain rail for shower 900 x 900 mm chrome 900.34.10140 in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles, with curtain rings - made of high-quality stainless steel, chrome - for shower tray 900 x 900 mm - fixing spacing 887 x 887 mm - with 18 curtain rings made of white POM - recommended curtain width 2500 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing with roses to wall and ceiling, ceiling support required - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.10140 |
| 900.34.10160 Shower curtain rail for shower 900 x 900 mm powder-coated 900.34.10160 in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles, with curtain rings - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for shower tray 900 x 900 mm - fixing spacing 887 x 887 mm - with 18 curtain rings made of white POM - recommended curtain width 2500 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing with roses to wall and ceiling, ceiling support required - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.10160 |
| 900.34.101XA Shower curtain rail for shower 900 x 900 mm satin 900.34.101XA in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles, with curtain rings - made of high-quality stainless steel, satin - for shower tray 900 x 900 mm - fixing spacing 887 x 887 mm - with 18 curtain rings made of white POM - recommended curtain width 2500 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing with roses to wall and ceiling, ceiling support required - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.101XA |
| 900.34.10240 Shower curtain rail for shower 1000 x 1000 mm chrome 900.34.10240 in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles, with curtain rings - made of high-quality stainless steel, chrome - for shower tray 1000 x 1000 mm - fixing spacing 987 x 987 mm - with 20 curtain rings made of white POM - recommended curtain width 3000 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing with roses to wall and ceiling, ceiling support required - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.10240 |
| 900.34.10260 Shower curtain rail for shower 1000 x 1000 mm powder-coated 900.34.10260 in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles, with curtain rings - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for shower tray 1000 x 1000 mm - fixing spacing 987 x 987 mm - with 20 curtain rings made of white POM - recommended curtain width 3000 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing with roses to wall and ceiling, ceiling support required - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.10260 |
| 900.34.102XA Shower curtain rail for shower 1000 x 1000 mm satin 900.34.102XA in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles, with curtain rings - made of high-quality stainless steel, satin - for shower tray 1000 x 1000 mm - fixing spacing 987 x 987 mm - with 20 curtain rings made of white POM - recommended curtain width 3000 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing with roses to wall and ceiling, ceiling support required - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.102XA |
| 900.34.10340 Shower curtain rail for shower 1200 x 1200 mm chrome 900.34.10340 in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles, with curtain rings - made of high-quality stainless steel, chrome - for shower tray 1200 x 1200 mm - fixing spacing 1187 x 1187 mm - with 24 curtain rings made of white POM - recommended curtain width 3500 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing with roses to wall and ceiling, ceiling support required - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.10340 |
| 900.34.10360 Shower curtain rail for shower 1200 x 1200 mm powder-coated 900.34.10360 in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles, with curtain rings - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for shower tray 1200 x 1200 mm - fixing spacing 1187 x 1187 mm - with 24 curtain rings made of white POM - recommended curtain width 3500 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing with roses to wall and ceiling, ceiling support required - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.10360 |
| 900.34.103XA Shower curtain rail for shower 1200 x 1200 mm satin 900.34.103XA in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles, with curtain rings - made of high-quality stainless steel, satin - for shower tray 1200 x 1200 mm - fixing spacing 1187 x 1187 mm - with 24 curtain rings made of white POM - recommended curtain width 3500 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing with roses to wall and ceiling, ceiling support required - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.103XA |
| 900.34.10440 Shower curtain rail for shower 1500 x 1500 mm chrome 900.34.10440 in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles, with curtain rings - made of high-quality stainless steel, chrome - for shower tray 1500 x 1500 mm - fixing spacing 1487 x 1487 mm - with 28 curtain rings made of white POM - recommended curtain width 4000 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing with roses to wall and ceiling, ceiling support required - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.10440 |
| 900.34.10460 Shower curtain rail for shower 1500 x 1500 mm powder-coated 900.34.10460 in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles, with curtain rings - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for shower tray 1500 x 1500 mm - fixing spacing 1487 x 1487 mm - with 28 curtain rings made of white POM - recommended curtain width 4000 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing with roses to wall and ceiling, ceiling support required - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.10460 |
| 900.34.104XA Shower curtain rail for shower 1500 x 1500 mm satin 900.34.104XA in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles, with curtain rings - made of high-quality stainless steel, satin - for shower tray 1500 x 1500 mm - fixing spacing 1487 x 1487 mm - with 28 curtain rings made of white POM - recommended curtain width 4000 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing with roses to wall and ceiling, ceiling support required - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.104XA |
| 900.34.10SXA Shower curtain rail, special length satin 900.34.10SXA in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles, with curtain rings - made of high-quality stainless steel, satin - length made-to-measure from 320 mm to 1987 mm - with a corresponding number of curtain rings made of white POM - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing with roses to wall and ceiling, ceiling support required - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.10SXA |
| 900.34.30040 Shower curtain rail for shower 1000 x 1000 mm chrome 900.34.30040 in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles in U-shape, with curtain rings - made of high-quality stainless steel, chrome - for shower tray 1000 x 1000 mm - fixing spacing 987 x 975 x 987 mm - with 28 curtain rings made of white POM - recommended curtain width 4000 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing onto wall and ceiling with roses, ceiling hangers required (max. 600 mm long) - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.30040 |
| 900.34.30060 Shower curtain rail for shower 1000 x 1000 mm powder-coated 900.34.30060 in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles in U-shape, with curtain rings - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for shower tray 1000 x 1000 mm - fixing spacing 987 x 975 x 987 mm - with 28 curtain rings made of white POM - recommended curtain width 4000 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing onto wall and ceiling with roses, ceiling hangers required (max. 600 mm long) - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.30060 |
| 900.34.300XA Shower curtain rail for shower 1000 x 1000 mm satin 900.34.300XA in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles in U-shape, with curtain rings - made of high-quality stainless steel, satin - for shower tray 1000 x 1000 mm - fixing spacing 987 x 975 x 987 mm - with 28 curtain rings made of white POM - recommended curtain width 4000 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing onto wall and ceiling with roses, ceiling hangers required (max. 600 mm long) - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.300XA |
| 900.34.30140 Shower curtain rail for shower 1200 x 1200 mm chrome 900.34.30140 in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles in U-shape, with curtain rings - made of high-quality stainless steel, chrome - for shower tray 1200 x 1200 mm - fixing spacing 1187 x 1175 x 1187 mm - with 34 curtain rings made of white POM - recommended curtain width 5000 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing onto wall and ceiling with roses, ceiling hangers required (max. 600 mm long) - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.30140 |
| 900.34.30160 Shower curtain rail for shower 1200 x 1200 mm powder-coated 900.34.30160 in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles in U-shape, with curtain rings - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for shower tray 1200 x 1200 mm - fixing spacing 1187 x 1175 x 1187 mm - with 34 curtain rings made of white POM - recommended curtain width 5000 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing onto wall and ceiling with roses, ceiling hangers required (max. 600 mm long) - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.30160 |
| 900.34.301XA Shower curtain rail for shower 1200 x 1200 mm satin 900.34.301XA in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles in U-shape, with curtain rings - made of high-quality stainless steel, satin - for shower tray 1200 x 1200 mm - fixing spacing 1187 x 1175 x 1187 mm - with 34 curtain rings made of white POM - recommended curtain width 5000 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing onto wall and ceiling with roses, ceiling hangers required (max. 600 mm long) - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.301XA |
| 900.34.30240 Shower curtain rail for shower 1500 x 1500 mm chrome 900.34.30240 in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles in U-shape, with curtain rings - made of high-quality stainless steel, chrome - for shower tray 1500 x 1500 mm - fixing spacing 1487 x 1475 x 1487 mm - with 46 curtain rings made of white POM - recommended curtain width 2 x 3000 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing onto wall and ceiling with roses, ceiling hangers required (max. 600 mm long) - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.30240 |
| 900.34.30260 Shower curtain rail for shower 1500 x 1500 mm powder-coated 900.34.30260 in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles in U-shape, with curtain rings - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - for shower tray 1500 x 1500 mm - fixing spacing 1487 x 1475 x 1487 mm - with 46 curtain rings made of white POM - recommended curtain width 2 x 3000 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing onto wall and ceiling with roses, ceiling hangers required (max. 600 mm long) - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.30260 |
| 900.34.302XA Shower curtain rail for shower 1500 x 1500 mm satin 900.34.302XA in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles in U-shape, with curtain rings - made of high-quality stainless steel, satin - for shower tray 1500 x 1500 mm - fixing spacing 1487 x 1475 x 1487 mm - with 46 curtain rings made of white POM - recommended curtain width 2 x 3000 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing onto wall and ceiling with roses, ceiling hangers required (max. 600 mm long) - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.302XA |
| 900.34.30SXA Shower curtain rail, special length A1: min. 320 mm, max. 1700 mm A2: min. 525 mm, max. 1675 mm A3: min. 320 mm, max. 1700 mm satin 900.34.30SXA in product catalogue |  | Curtain rail, System 900 - horizontally arranged rails connected at right-angles in U-shape, with curtain rings - made of high-quality stainless steel, satin - Lengths made-to-measure with rails for wall mounting from 320 mm to 1700 mm and connecting rail from 525 mm to 1675 mm - with a corresponding number of curtain rings made of white POM - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 300 mm - fixing onto wall and ceiling with roses, ceiling hangers required (max. 600 mm long) - including non-corrosive and tested HEWI fixing material and sealing tape for sealing the drilling points Article no.: HEWI 900.34.30SXA |
| 900.34.40040 Shower curtain rail 1000 mm chrome 900.34.40040 in product catalogue |  | Curtain rail, System 900 - straight rail with curtain rings - made of high-quality stainless steel, chrome - length 1000 mm - with 10 curtain rings made of white POM - recommended curtain width 1200 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 200 mm - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.34.40040 |
| 900.34.40060 Shower curtain rail 1000 mm powder-coated 900.34.40060 in product catalogue |  | Curtain rail, System 900 - straight rail with curtain rings - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - length 1000 mm - with 10 curtain rings made of white POM - recommended curtain width 1200 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 200 mm - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.34.40060 |
| 900.34.400XA Shower curtain rail 1000 mm satin 900.34.400XA in product catalogue |  | Curtain rail, System 900 - straight rail with curtain rings - made of high-quality stainless steel, satin - length 1000 mm - with 10 curtain rings made of white POM - recommended curtain width 1200 mm - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 200 mm - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.34.400XA |
|  |  | Curtain rail, System 900 - straight rail with curtain rings - made of high-quality stainless steel, chrome - length 48 inch (1219 mm) - with 10 curtain rings made of white POM - rail diameter 1 inch (25 mm), rose diameter 2 3/4 inch (70 mm) - can be shortened at the ends to 7 7/8 inch (200 mm) on site - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.34.40140 |
|  |  | Curtain rail, System 900 - straight rail with curtain rings - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light gray pearl mica) and SC (matt dark gray pearl mica) - length 48 inch (1219 mm) - with 10 curtain rings made of white POM - rail diameter 1 inch (25 mm), rose diameter 2 3/4 inch (70 mm) - can be shortened at the ends to 7 7/8 inch (200 mm) on site - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.34.40160 |
|  |  | Curtain rail, System 900 - straight rail with curtain rings - made of high-quality stainless steel, matt sanded - length 48 inch (1219 mm) - with 10 curtain rings made of white POM - rail diameter 1 inch (25 mm), rose diameter 2 3/4 inch (70 mm) - can be shortened at the ends to 7 7/8 inch (200 mm) on site - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.34.401XA |
| 900.34.40240 Shower curtain rail chrome 900.34.40240 in product catalogue |  | Curtain rail, System 900 - straight rail with curtain rings - made of high-quality stainless steel, chrome - length 60 inch (1524 mm) - with 12 curtain rings made of white POM - rail diameter 1 inch (25 mm), rose diameter 2 3/4 inch (70 mm) - can be shortened at the ends to 7 7/8 inch (200 mm) on site - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.34.40240 |
| 900.34.40260 Shower curtain rail powder-coated 900.34.40260 in product catalogue |  | Curtain rail, System 900 - straight rail with curtain rings - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light gray pearl mica) and SC (matt dark gray pearl mica) - length 60 inch (1524 mm) - with 12 curtain rings made of white POM - rail diameter 1 inch (25 mm), rose diameter 2 3/4 inch (70 mm) - can be shortened at the ends to 7 7/8 inch (200 mm) on site - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.34.40260 |
| 900.34.402XA Shower curtain rail satin 900.34.402XA in product catalogue |  | Curtain rail, System 900 - straight rail with curtain rings - made of high-quality stainless steel, matt sanded - length 60 inch (1524 mm) - with 12 curtain rings made of white POM - rail diameter 1 inch (25 mm), rose diameter 2 3/4 inch (70 mm) - can be shortened at the ends to 7 7/8 inch (200 mm) on site - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.34.402XA |
| 900.34.40SXA Shower curtain rail, special length L: min. 400 mm, max. 2000 mm satin 900.34.40SXA in product catalogue |  | Curtain rail, System 900 - straight rail with curtain rings - made of high-quality stainless steel, satin - length made-to-measure from 400 mm to 2000 mm - with a corresponding number of curtain rings made of white POM - rail diameter 25 mm, rose diameter 70 mm - can be shortened at the ends on site to 200 mm - including non-corrosive and tested HEWI fixing material for wall mounting and sealing tape for sealing the drilling points Article no.: HEWI 900.34.40SXA |
| 900.90.00140 Walking aid holder chrome 900.90.00140 in product catalogue |  | Walking aid holder system 900 - made of high-quality stainless steel, chrome - 120 mm wide, 35 mm high, 135 mm deep - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.90.00140 |
| 900.90.00160 Walking aid holder powder-coated 900.90.00160 in product catalogue |  | Walking aid holder system 900 - made of high-quality stainless steel, powder-coated in the HEWI colours DX (matt white), DC (matt black), AY (matt light grey pearl mica) and SC (matt dark grey pearl mica) - 120 mm wide, 35 mm high, 135 mm deep - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.90.00160 |
| 900.90.001XA Walking aid holder satin 900.90.001XA in product catalogue |  | Walking aid holder system 900 - made of high-quality stainless steel, satin finish - 120 mm wide, 35 mm high, 135 mm deep - for wall mounting, concealed fixing - including non-corrosive HEWI fixing material Article no.: HEWI 900.90.001XA |
| 921056 Curtain ring for curtain rails 802.34.... and 900.34.... 921056 in product catalogue |  | Curtain rings for curtain rails 900.34.... - open rings with small hook to receive a curtain - for hanging curtains on curtain rails with 24 and 25 mm diameter - outer diameter 50 mm, inner diameter 30 mm, 10 mm thick - made of high-quality POM in white - 5 in packing unit Article no.: HEWI 921056 |
| 950.30.11001 Bath towel shelf 950.30.11001 in product catalogue |  | Bath towel rack - mitred bath towel rack - made of brass tube, matt black coated, diameter 18 mm - 644 mm wide, 260 mm deep - for wall mounting, concealed fixing - including corrosion-free HEWI fixing material Article no.: HEWI 950.30.11001 |
| 950.30.11040 Bath towel shelf 600 mm 950.30.11040 in product catalogue |  | Bath towel shelf - bath towel shelf mitred joined - made of brass tube, high quality chrome plated, diameter 18 mm - 644 mm wide, 260 mm deep - for wall mounting, concealed mounting - Including non-corrosive HEWI fixing material Article no.: HEWI 950.30.11040 |